DMC-4103 Firmware Command Reference

Revision: 1840

05/17/22

©2022 Galil Motion Control

# Table of Content

Table of Content	2

Legend	8

'   Comment	9

-   Subtraction Operator	10

#   Label Designator	11

#AMPERR   Amplifier error automatic subroutine	12

#AUTO   Subroutine to run automatically upon power up	15

#AUTOERR   Bootup Error Automatic Subroutine	16

#CMDERR   Command error automatic subroutine	17

#COMINT   Communication interrupt automatic subroutine	18

#FWERR   Firmware Error Automatic Subroutine	19

#ININT   Input interrupt automatic subroutine	20

#LIMSWI   Limit switch automatic subroutine	21

#MCTIME   MC command timeout automatic subroutine	22

#POSERR   Position error automatic subroutine	23

#SERERR   Serial Encoder Error Automatic Subroutine	24

#TCPERR   Ethernet communication error automatic subroutine	25

$   Hexadecimal	26

%   Modulo Operator	27

&   JS subroutine pass variable by reference	28

&   Bitwise AND Operator	29

( , )   Parentheses (order of operations)	30

*   Multiplication Operator	31

/   Division Operator	32

;   Semicolon (Command Delimiter)	33

@ABS   Absolute value	34

@ACOS   Inverse cosine	35

@AN   Analog Input Query	36

@ASIN   Inverse sine	37

@ATAN   Inverse tangent	38

@COM   Bitwise complement	39

@COS   Cosine	40

@FLOT   Convert Galil 4.2 to Floating Point	41

@FRAC   Fractional part	42

@IN   Read digital input	43

@INT   Integer part	44

@OUT   Read digital output	45

@REAL   Convert Floating Point to Galil 4.2	46

@RND   Round	47

@SIN   Sine	48

@SQR   Square Root	49

@TAN   Tangent	50

[,]   Square Brackets (Array Index Operator)	51

^   JS subroutine stack variable	52

^L^K   Lock program	53

^R^S   Master Reset	54

^R^V   Revision Information	55

_   Operand Overview	56

_GP   Gearing Phase Differential Operand	57

_LF   Forward Limit Switch Operand	58

_LR   Reverse Limit Switch Operand	59

|   Bitwise OR Operator	60

~   Variable Axis Designator	61

+   Addition Operator	62

<   Less than comparator	63

<=   Less than or Equal to comparator	65

<>   Not Equal to comparator	67

=   Equal to comparator	68

=   Assignment Operator	69

>   Greater than comparator	70

>=   Greater than or Equal to comparator	72

AB   Abort	74

AC   Acceleration	75

AD   After Distance	76

AF   Analog Feedback Select	77

AG   Amplifier Gain	78

AI   After Input	79

AL   Arm Latch	80

AM   After Move	82

AO   Analog Output	83

AP   After Absolute Position	84

AQ   Analog Input Configuration	85

AR   After Relative Distance	86

AS   At Speed	87

AT   At Time	88

AU   Set amplifier current loop	89

AV   After Vector Distance	91

AZ   Clear Latched Amplifier Errors	92

BA   Brushless Axis	94

BC   Brushless Calibration	95

BD   Brushless Degrees	96

BG   Begin	97

BI   Brushless Inputs	98

BK   Breakpoint	99

BL   Reverse Software Limit	100

BM   Brushless Modulo	101

BN   Burn	102

BP   Burn Program	103

BR   Brush Axis	104

BT   Begin PVT Motion	105

BV   Burn Variables and Array	106

BW   Brake Wait	107

BX   Sine Amp Initialization	108

BZ   Brushless Zero	109

CA   Coordinate Axes	111

CB   Clear Bit	112

CC   Configure Communications Port 2	113

CD   Contour Distance	114

CE   Configure Encoder	115

CF   Configure Unsolicited Messages Handle	116

CI   Configure Communication Interrupt	117

CM   Contour Mode	118

CN   Configure	119

CP   Dead band within which the motor is shut off (MO)	120

CR   Circle	121

CS   Clear Sequence	123

CW   Copyright information and Data Adjustment bit on/off	124

DA   Deallocate Variables and Arrays	125

DB   Range in which PID and antifriction bias are turned on (on band)	126

DC    Deceleration	127

DE   Dual (Auxiliary) Encoder Position	128

DF   Dual Feedback (DV feedback swap)	129

DH   DHCP Client Enable	130

DL   Download	131

DM   Dimension Array	132

DP   Define Position	133

DR   Configures I O Data Record Update Rate	134

DS   Range in which PID and antifriction bias are turned off (off band)	135

DT   Delta Time	136

DV   Dual Velocity (Dual Loop)	137

EA   Choose ECAM master	138

EB   Enable ECAM	139

EC   ECAM Counter	140

ED   Edit	141

EG   ECAM go (engage)	142

EI   Event Interrupts	143

ELSE   Else function for use with IF conditional statement	145

EM   Ecam modulus	146

EN   End	147

ENDIF   End of IF conditional statement	148

EO   Echo	149

EP   Cam table master interval and phase shift	150

EQ   ECAM quit (disengage)	151

ER   Error Limit	152

ES   Ellipse Scale	153

ET   Electronic cam table	154

EW   ECAM Widen Segment	155

EY   ECAM Cycle Count	156

FA   Acceleration Feedforward	157

FC   Distance-selectable feedforward gain	158

FE   Find Edge	159

FI   Find Index	160

FL   Forward Software Limit	161

FN   Distance from end of move when FC is engaged	162

FV   Velocity Feedforward	163

GA   Master Axis for Gearing	164

GD   Gear Distance	165

GM   Gantry mode	166

GR   Gear Ratio	167

HM   Home	168

HS   Handle Assignment Switch	169

HV   Homing Velocity	170

HX   Halt Execution	171

IA   IP Address	172

ID   Identify	174

IF   IF conditional statement	175

IH   Open IP Handle	177

II   Input Interrupt	179

IK   Block Ethernet ports	180

IL   Integrator Limit	181

IP   Increment Position	182

IT   Independent Time Constant - Smoothing Function	183

JG   Jog	184

JP   Jump to Program Location	185

JS   Jump to Subroutine	187

K1   Proportional gain during motion	190

K2   Integrator gain during motion	191

K3   Derivative gain during motion	192

KD   Derivative Constant	193

KI   Integrator	194

KP   Proportional Constant	195

KS   Step Motor Smoothing	196

LA   List Arrays	197

LC   Low Current Stepper Mode	198

LD   Limit Disable	200

LE   Linear Interpolation End	201

LI   Linear Interpolation Distance	202

LL   List Labels	204

LM   Linear Interpolation Mode	205

LS   List	206

LV   List Variables	207

LZ   Omit leading zeros	208

MB   Modbus	209

MC   Motion Complete	212

ME   Modbus array write enable	213

MF   Forward Motion to Position	215

MG   Message	216

MO   Motor Off	218

MR   Reverse Motion to Position	219

MT   Motor Type	220

MU   Multicast Address	221

MW   Modbus Wait	222

NB   Notch Bandwidth	224

NF   Notch Frequency	225

NO   No Operation	226

NZ   Notch Zero	227

OA   Off on encoder failure	228

OB   Output Bit	229

OC   Output Compare	230

OE   Off-on-Error	232

OF   Offset	234

OP   Output Port	235

OT   Off on encoder failure time	236

OV   Off on encoder failure voltage	237

P2CD   Serial port 2 code	238

P2CH   Serial port 2 character	239

P2NM   Serial port 2 number	240

P2ST   Serial port 2 string	241

PA   Position Absolute	242

PF   Position Format	243

PL   Pole	244

PR   Position Relative	245

PT   Position Tracking	246

PV   PVT Data	247

PW   Password	248

QD   Download Array	249

QH   Query Hall State	250

QQ   Clear Sample Time Overflow	251

QR   I O Data Record	252

QS   Error Magnitude	253

QU   Upload Array	254

QZ   Return Data Record information	255

RA   Record Array	256

RC   Record	257

RD   Record Data	258

RE   Return from Error Routine	259

REM   Remark	260

RI   Return from Interrupt Routine	261

RL   Report Latched Position	262

RP   Reference Position	263

RS   Reset	264

SB   Set Bit	265

SC   Stop Code	266

SD   Limit Switch Deceleration	267

SH   Servo Here	268

SI   Configure the special Galil SSI feature	269

SL   Single Step	271

SM   Subnet Mask	272

SP   Speed	273

SS   Configure the special Galil BiSS feature	274

ST   Stop	276

SY   Serial encoder BiSS active level	277

TA   Tell amplifier error status	278

TB   Tell Status Byte	281

TC   Tell Error Code	282

TD   Tell Dual Encoder	285

TE   Tell Error	286

TH   Tell Ethernet Handle	287

TI   Tell Inputs	288

TIME   Time Operand	289

TK   Peak Torque Limit	290

TL   Torque Limit	291

TM   Update Time	292

TN   Vector Tangent	293

TP   Tell Position	294

TR   Trace	295

TS   Tell Switches	296

TT   Tell Torque	297

TV   Tell Velocity	298

TW   Timeout for MC trippoint	299

TZ   Tell I O Configuration	300

UI   User Interrupt	301

UL   Upload	302

US   USB port configuration	303

VA   Vector Acceleration	304

VD   Vector Deceleration	305

VE   Vector Sequence End	306

VF   Variable Format	307

VM   Vector Mode	308

VP   Vector Position	310

VR   Vector Speed Ratio	312

VS   Vector Speed	313

VV   Vector Speed Variable	314

WH   Which Handle	315

WT   Wait	316

XQ   Execute Program	317

YA   Step Drive Resolution	318

YB   Step Motor Resolution	319

YC   Encoder Resolution	320

YR   Error Correction	321

YS   Stepper Position Maintenance Mode Enable, Status	322

ZA   User Data Record Variables	323

ZN   Negative Antifriction Bias	324

ZP   Positive Antifriction Bias	325

ZS   Zero Subroutine Stack	326

# Legend

Burnable

Not Burnable

Burnable

Not Burnable

## Description

Commands with the "burnable" icon can be saved into memory with the BN command. If a reset is issued, the value of the command with this icon will persist if it has been burned into memory.

Scaled By TM

Scaled By TM

## Description

Any command with the "scaled by TM" icon will be automatically adjusted whenever a change is made to the TM setting. Commands with this icon are dependEnt on the sample rate.

Trippoint

Trippoint

## Description

A command with the "trippoint" icon will halt further program execution until the trippoint's condition is satisfied. Most trippoints cannot be issued as discrete commands, and are only valid in programs.

Valid In Program

Not Valid In Program

Valid In Program

Not Valid In Program

## Description

Commands with the "valid in program" icon can be used inside of a DMC program that is run locally on the controller. Certain commands may not be used in the program space, and can only be issued as discrete command from an external source such as a terminal.

Valid In Terminal

Not Valid In Terminal

Valid In Terminal

Not Valid In Terminal

## Description

When communicating with a controller externally, only commands which are "valid in terminal" may be sent to the controller as discrete commands. Some commands are only valid when executed in a DMC program and cannot be issued independently.

Valid In Motion

Not Valid In Motion

Valid In Motion

Not Valid In Motion

## Description

If a command is "valid in motion" then it may be executed while an axis is in motion. Some commands may not be executed while certain axes are in motion, and can only be executed when the associated axis is stopped.

Variable Axis Supported

Description

Commands with the "variable axis supported" icon support the use of variable axes when using the '~'. See '~' for more details.

Variable Axis Supported

Description

Commands with the "variable axis supported" icon support the use of variable axes when using the '~'. See '~' for more details.

PROGRAMMING '   Comment

' str

Description

The ' allows for a user to insert in a comment on a blank line or after a command following a semicolon ";". See examples for valid uses of '.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| str | String | Comments added into program | Comment strings are restricted to the maximum row size for a program. This will vary per controller. |

Remarks

Comments will be downloaded to controller, thus taking up program space and execution time. See REM for comments that will not download to controller.

Examples

'Galil DMC Code Example

REM This comment is not downloaded to controller and does not take up program

REM   space or execution time

'This comment is downloaded to controller and takes up program space

SH AB;'Comments following a command MUST be preceeded by a semi-colon.

KP 10'This is NOT valid use of the '

### ' applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

'

PROGRAMMING, MATH FUNCTIONS

### -   Subtraction Operator

n

0

- n

1

n

0

- n

1

| Usage | variable = (value1 - value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

The subtraction operator takes any two values and returns a value equal to the difference of the arguments.

Arguments

| Argument | Min | Max | Default | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to subtract from |
| n1 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to subtract |

Remarks

An operator is not a command and is not valid individually.

Evaluation occurs left to right. Use parenthesis for operator precedence.

n0 and n1 may also be variables, array elements, operands, or @ functions (e.g. @SIN[]).

Examples

'Galil DMC Code Example 'Terminal Example :apple = 10-4 :banana = apple - 3 :MG banana - 1 2.0000 :

'Galil DMC Code Example

REM It is recommended that parenthesis be used when more than one mathmatical REM   operation is combined in one command. cherry = ((10*30)-(60/30));' evaluates as 298 date = 10*30-60/30;' evalutes as 8 EN

#### - applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

-

PROGRAMMING, SUBROUTINE

### #   Label Designator

#str

#str

Description

The # denotes the name of a program label, for example, #move. Labels are often used to implement subroutines or loops. Labels are either user-defined or are reserved names, called "automatic subroutines", that automatically execute when a particular event occurs.

Arguments

| Argument | Min | Max | Default | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| str | 1 char | 7 chars | N/A | String | Name of label |

Remarks

Labels can include the characters A-Z, a-z, 1-9. Numbers can not be the first character. All other characters are invalid. A label can only be defined at the beginning of a new line.

The number of labels available can be queried with MG _DL.

LL returns the current label table in the controller.

Galil recommends that at least the first character be lowercase for user labels to differentiate from automatic subroutines. Automatic subroutines are listed in the command reference starting with a # character.

There is a maximum of 510 labels available.

Examples

'Galil DMC Code Example REM A sample FOR loop REM Routine will run 10 times and sum all integers 1 through 10 sum=0;' Variable to hold sum of integers i=1;' Create a counter #for sum=sum+i;' Add counter to sum i=i+1;' Increment counter JP#for,(i<=10) EN

'Galil DMC Code Example REM A sample Do-While loop REM Routine will run while A axis main encoder position is under 100 counts #while WT10;' Wait 10 mseconds JP#while,(_TPA<100);' Loop back if position is under 100 counts MG"Position is equal or greater than 100" EN

#### # applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#

PROGRAMMING, ERROR CONTROL, SUBROUTINE #AMPERR   Amplifier error automatic subroutine

#AMPERR

Description

Automatic subroutine used to run code when a fault occurs on a Galil amplifier. See the TA command and individual amplifier information in the controller user manual.

Arguments

Label must be the first element on a line of code.

Remarks

Use RE to return from the AMPERR subroutine. See the TA command for more information.

See the AZ command for more information on clearing latched amplifier errors.

Thread 0 does not need to be running for #AMPERR to be executed. This was a requirement on earlier products.

When an external servo driver is used on an axes where the AMP-43000 is also installed, the axis should be setup as a brushed motor (BRm=1), otherwise the lack of hall inputs will cause an amplifier error.

Related Commands

AG - Amplifier Gain

AU - Set Amplifier Current Loop

AZ - Clear Amplifier Errors

MT - Motor Type

TA - Tell Amplifier Error

TK - Peak Torque Limit

TL - Torque Limit

Examples

'Galil DMC Code Example 'this code will run in the event of an amplifier error, 'setting a digital output and notifying the operator. #AMPERR 'Set a digital bit to signal an amplifier error to peripheral hardware SB4 'Send a message to the user MG"An amplifier error has occurred" 'Return from the AMPERR subroutine, restoring trippoints that were running RE1*

## Author: Galil Motion Control ## Date: 3.3.2020 ## Controller Firmware: 4103 r1.3e // This is an example of a #AMPERR automatic subroutine. // The best method to demonstrate the behavior of this example is to assert the ELO input. #AUTO // Setup the amplifier for the A axis. MO // disable all motors to configure axes AZ2 // enable enhanced error clearing mode MTA=1 // set motor type for servo motor AGA=1 // set amplifier gain AUA=9 // set current loop gain TLA=3 // set continuous torque limit to 3V TKA=6 // set peak torque limit to 6V // Set motion parameters SPA=4000 // set speed of the A axis to 4000 counts/sec ACA=16000 // set acceleration of the A axis to 16000 counts/sec/sec DCA=16000 // set deceleration of the A axis to 16000 counts/sec/sec // Set tuning parameters KPA=6 // set proportional term for the A axis to 6 KDA=64 // set derivative term for the A axis to 64 KIA=0 // set integrator term for the A axis to 0 // Put the program into debug mode. debug=1 // send message for amplifier errors when debug=1 // Initialize sinusoidal commutation for the A axis. BAA // specify A axis for sine commutation BMA=2000 // define brushless modulus for A axis BIA=-1 // initialize commutation with hall sensors BCA // refine sinusoidal commutation from hall transition SB1;CB2 // set indicator lights tied to outputs 1 and 2 to operating DP0 // define position to 0

### #AMPERR

// Main routine that commands motion forward and backward for the A axis. #main; // label for main application program SHA // enable A axis // Command motion forward and backward. #loop // label to jump back to for loop PAA=4000 // command motion forward BGA // begin motion AMA // wait for motion to finish before continuing program PAA=0 // command motion backward BGA // begin motion AMA // wait for motion to finish before continuing program JP#loop // jump up to line with #loop label EN // #AMPERR automatic routine // This routine stops motion, sets the indicator lights to error. #AMPERR // Amplifier Error Automatic Subroutine ST;AM // stop motion and wait until motion is finished on all axes CB1;SB2 // set indicator lights tied to outputs 1 and 2 to error MO;WT2 // disable all axes JS#report,(debug=1) // print messages if in debug mode // Clear any latched amplifier errors and message to the terminal if in debug mode IF((_TA3&64)¦(_TA3&128)) // if there are any latched amplifier errors AZ1;WT4 // clear latched amplifier errors IF(debug=1) // print message if in debug mode MG"Latched amplifier error cleared" ENDIF ENDIF // Create variables to track amplifier error states ta0=0 // variable to keep track of _TA0 ta3=0 // variable to keep track of _TA3 // Loop to check if there is still an amplifier error reported. // While in debug mode, this will send a message to the terminal when any amplifier error state changes. #check IF((ta0<>_TA0)¦(ta3<>_TA3)) // if the state of the amplifier errors has changed ta0=_TA0;ta3=_TA3 // set variables to new reported amplifier error state JS#report,(debug=1) // print messages if in debug mode ENDIF WT2 JP#check,((_TA0<>0)¦(_TA3<>0)) // jump back to #check if there is still an error IF(debug=1) // print message if in debug mode MG"No amplifier errors" ENDIF // When there are no reported errors, set the indicator lights to operating and return to the main program ZS0 // zero the stack SB1;CB2 // set indicator lights tied to outputs 1 and 2 to operating JP#main // jump back to main program EN // Debug routine for sending messages when an amplifier error is reported #report; IF(_TA0&1) // message for Over-Current error on axis bank A-D MG "Over-Current amplifier error on axis bank A-D" ENDIF IF(_TA0&16) // message for Over-Current error on axis bank E-H MG "Over-Current amplifier error on axis bank E-H" ENDIF IF(_TA0&2) // message for Over-Voltage error on axis bank A-D MG "Over-Voltage amplifier error on axis bank A-D" ENDIF IF(_TA0&32) // message for Over-Voltage error on axis bank E-H MG "Over-Voltage amplifier error on axis bank E-H" ENDIF IF(_TA0&4) // message for Over-Temperature error on axis bank A-D MG "Over-Temperature amplifier error on axis bank A-D" ENDIF IF(_TA0&64) // message for Over-Temperature error on axis bank E-H MG "Over-Temperature amplifier error on axis bank E-H" ENDIF IF(_TA0&8) // message for Under-Voltage error on axis bank A-D MG "Under-Voltage amplifier error on axis bank A-D" ENDIF IF(_TA0&128) // message for Under-Voltage error on axis bank E-H MG "Under-Voltage amplifier error on axis bank E-H" ENDIF IF((_TA3&1)¦((_TA3&2)) // message for ELO MG "ELO input asserted" ENDIF EN

#AMPERR applies to DMC50000,DMC4000,DMC4103,DMC30010,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#AMPERR

Galil Motion Control	#AMPERR	14/326

PROGRAMMING, SUBROUTINE #AUTO   Subroutine to run automatically upon power up

#AUTO

Description

Defines the automatic entry point of embedded DMC code. When power is applied to the controller, or after the controller is reset, the program will automatically begin executing at this label. When no host software is used with the controller, #AUTO is required to run an application program on the controller stand-alone.

Arguments

Label must be the first element on a line of code.

Remarks

Use EN to end the routine

Thread 0 is used to execute #AUTO on startup

The BP command must be used to burn a program into EEPROM for the #AUTO to function.

Examples

'Galil DMC Code Example

'On startup, this code will create a 50% duty cycle square wave on output 1 with a period of 1 second. #AUTO;'     Start on powerup

SB 1;'     Set bit 1

WT 500;'   Wait 500msec

CB 1;'     Clear bit 1

WT 500;'   Wait 500msec

JP #AUTO;'  Jump back to #AUTO

#AUTO applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#AUTO

PROGRAMMING, ERROR CONTROL, SUBROUTINE

#AUTOERR   Bootup Error Automatic Subroutine

#AUTOERR

Description

Automatic subroutine that runs code upon power up if the firmware detects errors. If the EEPROM is corrupted, #AUTOERR will run. The EEPROM is considered corrupt if the checksum calculated on the bytes in the EEPROM do not match the checksum written to the EEPROM.

For SSI and BiSS operation, #AUTOERR will also run if the time to acquire serial position data exceeds 90% of the hardware sample loop. This type of error is very rare and should never occur in normal operation.

Arguments

Label must be the first element on a line of code.

Remarks

Use EN to end the routine

The type of checksum error can be queried with MG_RS

For SSI and BiSS operation

In the event of a serial position acquisition timeout, the following will occur:

The controller will reset

The controller servo loop will not run, TM will be set to zero

TC1 will return "143 TM timed out"

The automatic subroutine #AUTOERR will run, if present The Error output will be set

When using serial encoders (SSI or BiSS), the #AUTOERR should follow these guidlines

IF _TC=143 do not employ any trippoints in following code because the timer interrupt is suspended

Serial encoders can be disabled with the commands SIn=0 or SSn=0 where n is the axis indicator ABCDEFG or H

In order to re-enable the timer interrupt issue "TM n" where n is the servo update period in us (usually n=1000). See TM for more details

Examples

'Galil DMC Code Example

'Code detects a checksum error and notifies the user

#AUTOERR

MG"EEPROM ERROR ",_RS EN

'Galil DMC Code Example

'Use for BiSS and SSI only (-SER firmware)

'Distinguishing between a serial timeout

' condition and an EEProm condition

#AUTOERR

IF _TC=143

REM BiSS or SSI timeout

REM No trippoints in this clause

REM Print message to DMC-4020 LCD

LU0

MG"BiSS"{L1}

MG"Timeout"{L2}

SSA=0

SSB=0

ELSE

REM Checksum error

REM trippoints ok here

REM Print message to DMC-4020 LCD

LU0

MG"EEProm:"{L1}

### MG{Z10.0}_RS{L2}

ENDIF

EN

#AUTOERR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#AUTOERR

PROGRAMMING, ERROR CONTROL, SUBROUTINE

#CMDERR   Command error automatic subroutine

#CMDERR

Description

Automatic subroutine that runs code when a DMC code error occurs. Without #CMDERR defined, if an error (see TC command) occurs in an application program running on the Galil controller, the program (and all threads) will stop.

Arguments

Label must be the first element on a line of code.

Remarks

Use EN to end the routine

#CMDERR will only run from errors generated within embedded DMC code, not from the terminal or host

In a single threaded application (Thread 0 only), the EN command in the #CMDERR routine will restart thread 0 where it left off.

In a multi-threaded application, the thread that has an error will be halted when a command error occurs. Thread 0 will be interrupted to run the #CMDERR routine but other threads will continue to run.

In order to restart the thread that encountered the error, see the example in Chapter 7 of the User Manual and the _ED operand.

Thread 0 does not need to be running in order for the #CMDERR routine to execute.

Examples

'Galil DMC Code Example 'This code will put the motion controller in Position Tracking mode. 'Variable "target" is updated from the terminal or from a host program 'to specify a new target. #CMDERR is used to detect a bad target value. #start DPA=0;' Define current position as zero PTA=1;' Turn on position tracking target=0;' Initialize target variable #track;' Start tracking PAA=target;' Track to current value of target WT500;' Wait 500 ms JP#track;' Continue to track ' ' #CMDERR;' runs if an error occurs JP#done,_TC<>6 ;'check that an out of range occured (See TC) MG"Value ",target," is out of range for Position Tracking" target=_PAA ;' reset target #done EN1 ;'return to tracking logic

#CMDERR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#CMDERR

PROGRAMMING, SUBROUTINE #COMINT   Communication interrupt automatic subroutine

#COMINT

Description

Automatic subroutine to provide interrupt driven communications from the serial port. #COMINT can be configured by the CI command to run either when any character is received, or when a carriage return is received over the com port. The auxiliary port is used if equipped.

Arguments

Label must be the first element on a line of code.

Remarks

Use EN to end the routine

#COMINT runs in thread 0, and an application must be running in thread 0 in order for #COMINT to be enabled.

Code running in thread zero will be interrupted by the #COMINT subroutine.

It is important to handle the interrupt condition and return without delay. The controller will continue to receive data and update the data operands (P1CH,P2CH, etc) while in #COMINT. This can lead to missed characters, numbers, and strings if #COMINT is unnecessarily delayed.

It is the user's responsiblity to ensure the communication buffer is not filled when in this mode, otherwise the controller will report error code 5 "Input Buffer Full". The buffer on the controller is cleared when either of the two following conditions are met: A carriage return is recieved on the communication port. CI -1 is called.

Examples

'Galil DMC Code Example #A; 'Program Label CC9600,0,1,0 CI2; 'interrupt on any character #Loop MG "Loop"; 'print a message every second WT 1000 JP#Loop #COMINT MG "COMINT:", P2CH{S1}; 'print a message and the received character CI -1; 'Clear the buffer of the received character EN1,1; ' End this subroutine, re-arming trip points that ' were running and re-enabling the CI mask

#COMINT applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#COMINT

PROGRAMMING, SUBROUTINE #FWERR   Firmware Error Automatic Subroutine

#FWERR

Description

Automatic subroutine to run when the servo sample overflows. Many features require that the controller perform an action every sample. Running many of these features simultaneously can lead to there not being enough time to complete every action. See remarks for a list of these features. For the majority of applications, this will never occur. This behavior should only occur when running a low sample time (TM) and using absolute encoders with a high number of bits at a low clock frequency.

Arguments

Label must be the first element on a line of code.

Remarks

The following are features, and their associated DMC comands, which add a non-negligible amount of time to the servo sample calculations.

Serial Encoders (SI/SS)

Analog Encoders (AF)

Notch Filter (NF,NZ,NB)

Low Sample Time setting (TM)

Use EN to end the routine.

#FWERR runs on thread 0. Code does not need to be running in thread 0 for #FWERR to be enabled.

_QQ reports a value based on whether the interrupt overflow event has occured. See QQ for more details.

When the servo interrupt overlows, the following will occur:

_QQ is set to 1.

#FWERR, if present, will run.

The notch filter is turned off.

Serial and Analog feedback is disabled.

All axes are aborted (See AB command).

SC will report 41 for all that were profiling.

Examples

'Galil DMC Code Example 'Code detects a checksum error and notifies the user #FWERR MG "Interrupt overflow event occurred:" ,_QQ; 'printing the _QQ operand showing that the interrupt has overflown QQ; 'returns the _QQ operand to 0 MG "Shutdown for diagnostics" EN

#FWERR applies to DMC4000,DMC4103,DMC30010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#FWERR

PROGRAMMING, IO, SUBROUTINE #ININT   Input interrupt automatic subroutine

#ININT

Description

The #ININT subroutine is used to execute specific code when inputs specified by the II command are in the desired state.

Arguments

Label must be the first element on a line of code.

Remarks

Use the II command to enable the routine.

Use RI to exit the routine.

To make an unconditional jump from #ININT, there are two methods for re-enabling the interrupt capability Issue a ZS and then re-issue the command II before the JP

or, use a "null" routine. The "null" routine allows for the execution of the RI command before the unconditional jump. For more information see Application Note #2418, http://www.galil.com/download/application-note/note2418.pdf

Examples

'Galil DMC Code Example

II 1,1,,0;'                 Specify interrupt on input 1 only, and triggers when input 1 = 0. EN;'                        End Program

#ININT;'                   Interrupt subroutine

'                           The code the user wants to run when II triggers goes here.

WT100;'                     Debounce the input.

RI 1;'                      Return to main program, re-enabling trip point.

'                           Specify RI 0 if it is not desired to re-enable trip points.

#ININT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#ININT

PROGRAMMING, ERROR CONTROL, SUBROUTINE

#LIMSWI Limit switch automatic subroutine

#LIMSWI

Description

Automatic sub for running user-defined code on a limit switch event. A limit switch event requires the following conditions.

- Motion profiling in the direction of the given limit. I.E. RPm increasing for forward switch, RPm decreasing for reverse switch.

- Limit switch toggles active, either a hardware or software limit. See CN for inverting the active sense of the limit swithes.

Without #LIMSWI defined, the controller will issue ST on the axis when its limit switch is tripped during motion in the direction of the switch. With #LIMSWI defined, code is executed in addition to the stop.

In lieu of a controlled stop, the motor can turn off and coast stop in the event of a limit switch event. See OE for this feature.

Arguments

Label must be the first element on a line of code.

Remarks

Use RE to terminate the subroutine

See _LF and _LR for switch state operands

#LIMSWI runs on thread 0. Code does not need to be running in thread 0 for #LIMSWI to be enabled.

LD can be used to disable the limit operation

SD can be used to set the deceleration speed on the limit.

Examples

'Galil DMC Code Example

#Main        ;'print a message every second

MG "Main"

WT1000

JP#Main

EN

'

#LIMSWI ;'runs when a limit switch is tripped

MG "Limit switch:"{N}

IF ((_LFA = 0) | (_LRA = 0))

MG "Axis A"

ENDIF

IF ((_LFB = 0) | (_LRB = 0))

MG "Axis B"

ENDIF

RE1;' RE used to exit the #LIMSWI sub

#LIMSWI applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#LIMSWI

PROGRAMMING, SUBROUTINE

#MCTIME MC commandtimeout automatic subroutine

#MCTIME

Description

Automatic sub used to run user-code if a Motion Complete (MC) trippoint times out. If the motor position does not reach or pass the target within the specified timeout (TW), #MCTIME wil run if present.

MC uses position from TP for servos, or TD for steppers.

Arguments

Label must be the first element on a line of code.

Remarks

Use EN to terminate the subroutine

Examples

'Galil DMC Code Example #BEGIN;' Begin main program TWA= 1000;' Set the time out to 1000 ms PRA= 10000;' Position relative BG A;' Begin motion MC A;' Motion Complete trip point EN;' End main program ' #MCTIME;' Motion Complete Subroutine MG "A fell short";' Send out a message EN 1;' End subroutine

#MCTIME applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#MCTIME

#POSERR Position errorautomatic subroutine

#POSERR

Description

Automatic subroutine that runs user code when a position error event occurs. The factory default behavior of the Galil controller upon a position error (_TEn > _ERn) is to drive the error signal low only, turning on the red error LED. If OE is set to 1, the motor whose position error (TE) equals or exceeds its threshold (ER) will be turned off (MO). #POSERR is used to run code upon a position error, for example to notify a host computer.

Arguments

Label must be the first element on a line of code.

Remarks

Use RE to end the routine.

#POSERR runs on thread 0. Code does not need to be running in thread 0 for #POSERR to be enabled.

#POSERR will also run when OE1 is set for an axes and that axis is also setup for encoder failure detection (see OA, OT, OV commands).

Examples

## Author: Galil Motion Control

## Date 6.3.2020

//how to recover from position error on the A axis

#POSERR

ST; // stop commanding motion to all axes

AM; // wait until motion is halted

MO; // disable all axes

MG "Position error occurred"; // send message indicating position error occured SHA;//reenable A axis, position error is cleared

EN

#POSERR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#POSERR

#SERERR Serial EncoderError Automatic Subroutine

#SERERR

Description

During operation, the #SERERR automatic subroutine allows user code to run when there is a serial encoder fault. Encoder faults are reported in the _SSm0 operand, see the SS command for more details.

Arguments

Label must be the first element on a line of code.

Remarks

Use the RE command to end this routine. #SERERR runs on thread 0

The following are the fault conditions which will cause #SERERR to interrupt.

Serial Encoder Faults

BiSS

Encoder timeout (bit 0 of _SSm0)

CRC error (bit 1 of _SSm0)

Warning bit* (bit 2 of _SSm0)

Error bit* (bit 3 of _SSm0)

The active level of the Error and Warning bits for BiSS must be configured with SY.

Examples

'Galil DMC Code Example #SERERR 'Serial Error routine messages out to the host the type of error. sercode=_SSA0 IF (sercode & 1) MG "BiSS Timeout" ENDIF IF (sercode & 2) MG "Invalid CRC" ENDIF IF (sercode & 4) MG "Warning Bit Set" ENDIF IF (sercode & 8) MG "Error Bit Set" ENDIF RE

'Galil DMC Code Example

#SERERR;' display error, shutdown axis

MG"SERERR"

MG_SSA

REM disable axis A

OEA=1;ERA=0

REM disable axis serial encoder

SSA=0

RE

#SERERR applies to SER

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#SERERR

#TCPERR Ethernet communication error automatic subroutine

#TCPERR

Description

Automatic subroutine which allows execution of user code when an TCP error event occurs. #TCPERR allows the application programmer to run code (for example to reestablish the connection) when error code 123 occurs.

Arguments

Label must be the first element on a line of code.

Remarks

Use RE to exit this subroutine.

Error code 123 (TCP lost sync or timeout) occurs when a message is sent out a handle, and no acknowledgement is received. When this occurs, the handle the message was sent out is closed.

#TCPERR can be used to reestablish the handle

Code does not need to be running in thread 0 for #TCPERR to run.

Examples

'Galil DMC Code Example

#loop

### MG {EA} "L"

WT1000

JP#loop

#TCPERR

MG {P1} "TCPERR.  Dropped handle", _IA4

RE

'Galil DMC Code Example 'example of reestablishing connection after TCPERR ' #main IHE=192,168,1,30;' connect to 192,168,1,30 WT100;' wait for handle to be established ipe=_IHE0;' save IP for reconnection use n=0;' connection counter #loop;' endless message loop MG"hello" WT1000 JP#loop EN #TCPERR IHE=>-3;' make sure handle E is clear JP#TCPERR,_IHE2<>0;' wait for clear handle IHE=ihe;' set handle with saved IP var WT100 n=n+1;' increment counter JP#END,n>5;' try at least 5 times JP#TCPERR,_IHE2<>-2;'repeat if handle failed #END IF(n>5) MG"failed connection" HX0;' stop code if connection lost ELSE MG"Reconnected" n = 0;' reset connection counter ENDIF RE

#TCPERR applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,RIO47000,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#TCPERR

PROGRAMMING

### $   Hexadecimal

$n

$n

Description

The $ operator denotes that the following string is in hexadecimal notation.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | $80000000.0000 | $7FFFFFFF.FFFF | N/A | $0.0001 | Value of hexadecimal number | 32 bits of integer and 16 bits of fraction in total |

Remarks

None

Examples

'Galil DMC Code Example

x = $7fffffff.0000              ;'store 2147483647 in x y = x & $0000ffff.0000          ;'store lower 16 bits of x in y z = x & $ffff0000.0000 / $10000 ;'store upper 16 bits of x in z

#### $ applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

$

MATH FUNCTIONS

### %   Modulo Operator

n % n

n % n

| Usage | variable = (value1 % value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

The % symbol is the modulo operator. It takes as arguments any two values, variables, array elements, operands, or At functions (@SIN[]) and returns a value equal to the modulo of the arguments.

Mathmatical operations are calculated left to right rather than multiplication and division calculations performed prior to addition and subraction. Example:

1+2*3 = 9, not 7

It is recommended that parenthesis be used when more than one mathmatical operation is combined in one command. Example:

var = ((10*30)+(60/30));' evaluates as 302 var = 10*30+60/30;' evalutes as 12

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | -2,147,483,647.9999 | N/A | 1/65,536 | Value to use in modulo operation |  |

Remarks

This is a binary operator (takes two arguments and returns one value). The result of this operation is a value, which is not valid on its own. It must be coupled with a command. See examples below.

Mathmatical operations are calculated left to right rather than multiplication and division calculations performed prior to addition and subraction. Example: 1+2*3 = 9, not 7

It is recommended that parenthesis be used when more than one mathmatical operation is combined in one command. Example: var = ((10*30)+(60/30));' evaluates as 302 var = 10*30+60/30;' evalutes as 12

Examples

'Galil DMC Code Example 'Determine the day of week in n days DM name[7];'Strings for day of week name[0]="SUN" name[1]="MON" name[2]="TUE" name[3]="WED" name[4]="THU" name[5]="FRI" name[6]="SAT" today=2;'Tuesday days=123;'Days from now dow=((days + today)%7);'calculate future day of week MG"The day of week in ",days{Z10.0}," days will be ", name[dow]{S3.0} EN REM Code Returns: The day of week in 123 days will be SAT

#### % applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,RIO47000,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

%

SUBROUTINE &   JS subroutine pass variable by reference

## JS#str0(&str1,&str1,&str1,&str1,&str1,&str1,&str1,&str1)

Description

The & symbol is used to pass a variable by reference on the subroutine stack. When passed by reference, a change to the local-scope variable changes the global value.

Arguments

| Argument | Min | Max | Default | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| str0 | 1 char | 7 chars | N/A | String | Name of label to use for subroutine call |
| str1 | 1 char | 8 chars | N/A | String | Name of variable to pass by reference to the subroutine |

Remarks

Variables sent to a subroutine must be global variables that are already dimensioned.

Do not dimension any variables in a subroutine when passing variables by reference. This can break the variable pointer.

Examples

'Galil DMC Code Example REM Pass By Reference Example: #main value=5;' a value to be passed by reference global=8;' a global variable JS#SUM(&value,1,2,3,4,5,6,7);' note first arg passed by reference MG value;' message out value after subroutine. MG _JS;' message out returned value EN ' #SUM;' (* ^a,^b,^c,^d,^e,^f,^g) ^a=^b+^c+^d+^e+^f+^g+^h+global EN,,^a 'notes- 'do not use spaces when working with ^ 'If using global variables, they MUST be created before the subroutine is run 'From Terminal : Executed program from program2.dmc 36.0000 36.0000

### & applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

&

PROGRAMMING

### & Bitwise AND Operator

n & n

n & n

| Usage | variable = (value1 & value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

The & symbol is the bitwise AND operator used with IF, JP, and JS decisions, and also to perform bitwise ANDING of values.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to use with AND operator |  |

Remarks

The result of this operation is a value, which is not valid on its own. It must be coupled with a command. See examples below. For IF, JP, and JS, the values used for n are typically the results of logical expressions such as (x > 2) & (y=8)

Examples

'Galil DMC Code Example 'Bitwise use :var1=$F;'00001111 :var2=$F0;'1111000 :MG (var1 & var2) 0.0000 :MG var1 15.0000 :MG var2 240.0000 :

'Galil DMC Code Example 'Conditional Use var1=$F;'00001111 var2=$F0;'1111000

IF (var1 = $F) & (var2 = $F1)

MG"True"

ELSE

MG"False"

ENDIF

EN

REM Returned: False

#### & applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

&

### ( , )   Parentheses (order of operations)

(

n

)

(

n

)

Description

The parentheses denote the order of math and logical operations.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647.9999 | N/A | 1/65,536 | Math or logical expression for evaluation |  |

Remarks

Note that the controller evaluates expressions from left to right, and does not follow academic algebraic standards (e.g. multiplication and division first, followed by addition or subtraction)

It is required to use parentheticals to ensure intended mathematical precedence

Examples

'Galil DMC Code Example :MG 1+2*3 9.0000 :MG 1+(2*3) 7.0000

'Galil DMC Code Example

:var1=$1F

:var2=$F

:MG var1&var2/$10

0.9375      ($0.F000)

:MG var1&(var2/$10)

0.0000      ($0.0000)

( , ) applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

( , )

### * Multiplication Operator

n * n

n * n

| Usage | variable = (value1 * value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

The * symbol is the multiplication operator. It takes as arguments any two values, variables, array elements, operands, or At functions (@SIN[]) and returns a value equal to the product of the arguments.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | -2,147,483,647 | N/A | 1/65,536 | Value to use in multiplication operation |  |

Remarks

This is a binary operator (takes two arguments and returns one value). The result of this operation is a value, which is not valid on its own. It must be coupled with a command. See examples below.

Mathmatical operations are calculated left to right rather than multiplication and division calculations performed prior to addition and subraction. Example: 1+2*3 = 9;' not 7

It is recommended that parenthesis be used when more than one mathmatical operation is combined in one command. Example: var = ((10*30)+(60/30));' evaluates as 302 var = 10*30+60/30;' evalutes as 12

Examples

'Galil DMC Code Example :var1 = (2 + 3) * 2 :var2 = var1 * 10 :MG var2 * 0.5 50.0000 :

#### * applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

*

### / Division Operator

n

0

/ n

1

n

0

/ n

1

| Usage | variable = (value1 / value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

The / symbol is the division operator. It takes as arguments any two values, variables, array elements, operands, or At functions (@SIN[]) and returns a value equal to the quotient of the arguments.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Numerator of divide operation |  |
| n1 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Denominator of divide operation |  |

Remarks

This is a binary operator (takes two arguments and returns one value). The result of this operation is a value, which is not valid on its own. It must be coupled with a command. See examples below.

Mathmatical operations are calculated left to right rather than multiplication and division calculations performed prior to addition and subraction. Example: 1+2*3 = 9;' not 7

It is recommended that parenthesis be used when more than one mathmatical operation is combined in one command. Example: var = ((10*30)+(60/30));' evaluates as 302 var = 10*30+60/30;' evalutes as 12

Examples

'Galil DMC Code Example :var1 = 100/10 :var2 = var1/2 :MG var2 + 1 6.0000 :

#### / applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

/

PROGRAMMING ; Semicolon (Command Delimiter)

## arg;arg;arg;arg

Description

The semicolon operator allows multiple Galil commands to exist on a single line.

Arguments

arg represents any valid Galil command

Remarks

Examples

'Galil DMC Code Example

SB1;WT500;CB1;' multiple commands separated by semicolons with a comment

'Galil DMC Code Example #High;' #High priority thread executes twice as fast as a = a + 1; b = b + 1 JP#High #Low;' #Low when run in parallel c = c + 1 d = d + 1 JP#Low

### ; applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,RIO47000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

;

MATH FUNCTIONS

### @ABS   Absolute value

@ABS[n]

@ABS[n]

| Usage | variable = @ABS[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @ABS[] operation takes the absolute value of the given number. Returns the value if positive, and returns -1 times the value if negative.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,535 | Number to display as absolute value |  |

Remarks

@ABS[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example

:MG @ABS[-2147483647]

2147483647.0000

@ABS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@ABS

### @ACOS   Inverse cosine

@ACOS[n]

@ACOS[n]

| Usage | variable = @ACOS[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @ACOS operator returns in degrees the arc cosine of the given number.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | -1 |  | 1 |  | N/A | 1/65,536 | Value used for arc cosine operation |  |

Remarks

@ACOS[] is an operand, not a command. It can only be used as an argument to other commands and operators @ACOS[] is also referred to as the inverse cosine function

Examples

'Galil DMC Code Example :MG @ACOS[-1] 180.0000 :MG @ACOS[0] 90.0000 :MG @ACOS[1] 0.0001

@ACOS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@ACOS

INTERROGATION, IO

### @AN   Analog Input Query

@AN[n]

@AN[n]

| Usage | variable = @AN[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @AN[] operator returns the value of the given analog input in volts.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 1 | 8 | N/A | 1 | Analog input to query |  |
| n | 1,000 | 8,999 | N/A | 1 | Read Modbus slave analog input | See Remarks |

Remarks

@AN[] is an operand, not a command. It can only be used as an argument to other commands and operators

#### Using @AN with a Modbus Slave

RIO as Modbus Slave

3rd Party Modbus Slave Device

n is the I/O number calculated using the following equations:

n = (HandleNum*1000) + (Bitnum-1)

HandleNum is the handle specifier from A to H.

Handle must be assigned to port 502 for Modbus comms (See IH) BitNum is the I/O point in the module from 1 to 8

Examples

'Galil DMC Code Example

:MG @AN[1] ;'print analog input 1

1.7883

:x = @AN[1] ;'assign analog input 1 to a variable

@AN applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,RIO57400,DMC1806,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@AN

### @ASIN   Inverse sine

@ASIN[n]

@ASIN[n]

| Usage | variable = @ASIN[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @ASIN operator returns in degrees the arc sine of the given number.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | -1 |  | 1 |  | N/A | 1/65,536 | Value used for arc sine operation |  |

Remarks

@ASIN[] is an operand, not a command. It can only be used as an argument to other commands and operators @ASIN[] is also referred to as the inverse sine function

Examples

'Galil DMC Code Example :MG @ASIN[-1] -90.0000 :MG @ASIN[0] 0.0000 :MG @ASIN[1] 90.0000

@ASIN applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@ASIN

### @ATAN   Inverse tangent

@ATAN[n]

@ATAN[n]

| Usage | variable = @ATAN[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @ATAN operator returns in degrees the arc tangent of the given number.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,638 | 2,147,483,647 | N/A | 1/65,536 | Value used for arc tangent operation |  |

Remarks

@ATAN[] is an operand, not a command. It can only be used as an argument to other commands and operators @ATAN[] is also referred to as the inverse tangent function

Examples

'Galil DMC Code Example

:MG @ATAN[-10]  -84.2894

:MG @ATAN[0]

0.0000

:MG @ATAN[10]  84.2894

@ATAN applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@ATAN

### @COM Bitwise complement

@COM[n]

@COM[n]

| Usage | variable = @COM[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @COM[] operation performs the bitwise complement (NOT) operation to the given number.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | - 2,147,483,648 | 2,147,483,647 | N/A | 1 | Value to perform bitwise complement operation. | Integer interpreted as a 32-bit field |

Remarks

@COM[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

| 'Galil DMC Code Example :MG {$8.0} @COM[0] |  |
| --- | --- |
| $FFFFFFFF :MG {$8.0} $00000000 | @COM[$FFFFFFFF] |
| 'Galil DMC Code Example 'toggle output 1 OB 1,@COM[@OUT[1]] & 1;' read current state of output 1, take the bitwise complement, mask out bits. |  |

@COM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@COM

### @COS Cosine

@COS[n]

@COS[n]

| Usage | variable = @COS[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @COS[] operation returns the cosine of the given angle in degrees

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -32,768 | 32,767 | N/A | 1/65,536 | Value in degrees to use for cosine operation |  |

Remarks

@COS[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example :MG @COS[0] 1.0000 :MG @COS[90] 0.0000 :MG @COS[180] -1.0000 :MG @COS[270] 0.0000 :MG @COS[360] 1.0000

@COS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@COS

### @FLOT Convert Galil 4.2 to Floating Point

@FLOT[n]

@FLOT[n]

| Usage | variable = @FLOT[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @FLOT operation returns the 32bit floating representation of a number

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to use for floating point conversion |  |

Remarks

@FLOT[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example :MG @FLOT[2.5] {$8.0} $40200000 :MG @REAL[$40200000] 2.5000 :

@FLOT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@FLOT

### @FRAC   Fractional part

@FRAC[n]

@FRAC[n]

| Usage | variable = @FRAC[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @FRAC operation returns the fractional part of the given number

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to use in fractional operation |  |

Remarks

The sign of the number input to the operation will be maintained in the fractional output.

@FRAC[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example :MG @FRAC[1.2] 0.2000 :MG @FRAC[-2.4] -0.4000

@FRAC applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@FRAC

INTERROGATION, IO

### @IN   Read digital input

@IN[n]

@IN[n]

| Usage | variable = @IN[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @IN operand returns the value of the given digital input (either 0 or 1).

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 1 | 16 | N/A | 1 | General input to query | Inputs 9-16 only valid for 5-8 axis controller |
|  | 81 | 96 | N/A | 1 | Aux encoder input to query | Used when repurposing aux encoder inputs as digital inputs |
| n | 1,000 | 8,999 | N/A | 1 | Read Modbus slave bit | See Remarks |

Remarks

@IN[] is an operand, not a command. It can only be used as an argument to other commands and operators

Using @IN with a Modbus Slave n = (SlaveAddress*10000) + (HandleNum*1000) + ((Module-1)*4) + (Bitnum-1)

Slave Address is used when the ModBus device has slave devices connected to it and specified as Addresses 0 to 255. Please note that the use of slave devices for modbus are very rare and this number will usually be 0.

HandleNum is the handle specifier where A is 1, B is 2 and so on.

Module is the position of the module in the rack from 1 to 16. BitNum is the I/O point in the module from 1 to 4

Examples

'Galil DMC Code Example

:MG @IN[1]

1.0000

:x = @IN[1]

:x = ?;'  print digital input 1

1.000

@IN applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,RIO57400,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@IN

### @INT   Integer part

@INT[n]

@INT[n]

| Usage | variable = @INT[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @INT operation returns the integer part of the given number. Note that the modulus operator can be implemented with @INT (see example below).

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to use in integer operation |  |

Remarks

@INT[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example

:MG @INT[1.2]

1.0000

:MG @INT[-2.4]

-2.0000

'Galil DMC Code Example #AUTO;' modulus example x = 10;' prepare arguments y = 3 JS#mod;' call modulus MG z;' print return value EN 'subroutine: integer remainder of x/y (10 mod 3 = 1) 'arguments are x and y. Return is in z #mod z = x - (y * @INT[x/y]) EN

@INT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@INT

INTERROGATION, IO

### @OUT   Read digital output

@OUT[n]

@OUT[n]

| Usage | variable = @OUT[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

Returns the value of the given digital output (either 0 or 1)

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 1 | 16 | N/A | 1 | General output to query | Outputs 9-16 only valid for 5-8 axis controller |
| n | 1,000 | 8,999 | N/A | 1 | Query Modbus slave bit | See Remarks |

Remarks

@OUT[] is an operand, not a command. It can only be used as an argument to other commands and operators

Using @OUT with a Modbus Slave n = (SlaveAddress*10000) + (HandleNum*1000) + ((Module-1)*4) + (Bitnum-1)

Slave Address is used when the ModBus device has slave devices connected to it and specified as Addresses 0 to 255. Please note that the use of slave devices for modbus are very rare and this number will usually be 0.

HandleNum is the handle specifier where A is 1, B is 2 and so on.

Module is the position of the module in the rack from 1 to 16. BitNum is the I/O point in the module from 1 to 4

Examples

'Galil DMC Code Example

:MG @OUT[1];'     print state of digital output 1

1.0000

:x = @OUT[1];'    assign state of digital output 1 to a variable

@OUT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,RIO57400,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@OUT

### @REAL   Convert Floating Point to Galil 4.2

@REAL[n]

@REAL[n]

| Usage | variable = @REAL[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @REAL operation returns the Galil 4.2 equivalent of a 32 bit floating point number

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1 | 32 bit floating point number to convert to Galil 4.2 integer |  |

Remarks

@REAL[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example :MG @FLOT[2.5] {$8.0} $40200000 :MG @REAL[$40200000] 2.5000 :

@REAL applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@REAL

### @RND   Round

@RND[n]

@RND[n]

| Usage | variable = @RND[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @RND operation rounds the given number to the nearest integer.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to use in rounding operation |  |

Remarks

@FRAC[] is an operand, not a command. It can only be used as an argument to other commands and operators The sign of the number input to the operation will be maintained in the rounded output.

Examples

'Galil DMC Code Example :MG @RND[1.2] 1.0000 :MG @RND[1.6] 2.0000 :MG @RND[-1.2] -1.0000 :MG @RND[5.7] 6.0000 :MG @RND[-5.7] -6.0000 :MG @RND[5.5] 6.0000 :MG @RND[-5.5] -5.0000

@RND applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@RND

### @SIN   Sine

@SIN[n]

@SIN[n]

| Usage | variable = @SIN[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @SIN[] operation returns the sine of the given angle in degrees

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -32,768 | 32,767 | N/A | 1/65,536 | Value in degrees to use for sine operation |  |

Remarks

@SIN[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example :MG @SIN[0] 0.0000 :MG @SIN[90] 1.0000 :MG @SIN[180] 0.0000 :MG @SIN[270] -1.0000 :MG @SIN[360] 0.0000

@SIN applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@SIN

### @SQR   Square Root

@SQR[n]

@SQR[n]

| Usage | variable = @SQR[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @SQR operation takes the square root of the given number.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | - 2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to use in square root operation | If n < 0, the absolute value is taken first. |

Remarks

@SQR[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example :MG @SQR[2] 1.4142 :MG @SQR[-2] 1.4142

@SQR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@SQR

### @TAN   Tangent

@TAN[n]

@TAN[n]

| Usage | variable = @TAN[value] | Performs a function on a value or evaluated statement and returns a value |
| --- | --- | --- |

Description

The @TAN[] operation returns the tangent of the given angle in degrees.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -32,768 | 32,767 | N/A | 1/65,536 | Value in degrees to use for tangent operation |  |

Remarks

@TAN[] is an operand, not a command. It can only be used as an argument to other commands and operators

Examples

'Galil DMC Code Example :MG @TAN[23] 0.4245 :

@TAN applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

@TAN

PROGRAMMING

[,]   Square Brackets (Array Index Operator)

str[n]

Description

The square brackets are used to denote the array index for an array, or to denote an array name.

They are also used to designate the argument to a function, such as @ABS[n].

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 8 chars | N/A | String | Name of array to access | Must be a valid dimensioned array name. |
| n | -1 | 15,999 | N/A | 1 | Element of array to query | n = -1 returns the array length |
| Argument | Min | Max | Default | Resolution | Description | Notes |
| str | 1 char | 8 chars | N/A | String | Name of array to access | Must be a valid dimensioned array name. |
| n | 0 | 399 | N/A | 1 | Element of array to query | For RIO-47x00 |
|  | 0 | 999 | N/A | 1 | Element of array to query | For RIO-47xx2 and RIO-473xx |

Remarks

If the array will be passed by reference on the subroutine stack (JS), the array name MUST be 6 characters or less.

Examples

'Galil DMC Code Example

DM A[50]        ;'define a 50 element array

A[0] = 3        ;'set first element to 3

MG A[0]         ;'print element 0

'Galil DMC Code Example #array DM A[5];' define a 5 element array A[0] = 3;' set first element to 3 MG "A[0]=",A[0];' print element 0 len= A[-1];' len now contains the length of A[] QU A[],0,len-1,1;MG"";' print entire array MG "A[] length=",len;' display Variable len EN 'Example Output from terminal :XQ#array : A[0]= 3 3, 4320, 216666, 217522, 607950 A[] length= 5 :

[,] applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

[,]

PROGRAMMING

### ^   JS subroutine stack variable

^s

^s

Description

The ^ character provides local subroutine access for variables passed on the subroutine stack. Passing values on the stack is advanced DMC programming, and is recommended for experienced DMC programmers familiar with the concept of passing arguments by value and by reference.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s | a |  | h |  | N/A | N/A | Stack variable name | a,b,c,d,e,f,g,h supported |

Remarks

See the JS command for a full explanation of passing stack variables.

Passing parameters has no type checking, so it is important to exercise good programming style when passing parameters. See examples below for recommended syntax.

Do not use spaces in expressions containing ^.

Global variables MUST be assigned prior to any use in subroutines where variables are passed by reference.

Arrays passed on the stack must have names no longer than 6 chars.

Stack zero has no local-scope variables. Accessing these variables from stack zero writes to stack 1's variable table.

Examples

'Galil DMC Code Example #Add JS#SUM(1,2,3,4,5,6,7,8) ;' call subroutine, pass values MG_JS ;' print return value EN ' #SUM ;NO(^a,^b,^c,^d,^e,^f,^g,^h) Sums the values ^a to ^h and returns the result EN,,(^a+^b+^c+^d+^e+^f+^g+^h) ;' return sum 'Output from the previous program :XQ#Add 36.0000

#### ^ applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

^

SYSTEM CONFIG

### ^L^K   Lock program

^L^K str,n

^L^K str,n

| Usage | ^L^K n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

Locks user access to the application program. When locked, the ED, UL, LS, and TR commands will give privilege error #106. The application program will still run when locked. Once the program is unlocked, it will remain accessible until a lock command or a reset (with the locked condition burned in) occurs.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 0 char | 8 chars | "" | String | Controller password string | Password assigned with the PW command. |
| n | 0 | 1 | 0 | 1 | Set lock/unlock state for controller | n = 1 locks the application program. n = 0 unlocks the application program. |

Remarks

The PW command can only be set while the application program is unlocked. ^L^K ? will return a 0 if the controller is not locked, and a 1 if it is locked. ASCII Values

| Char | Dec | Hex |
| --- | --- | --- |
| ^L | 12 | 0C |
| ^K | 11 | 0B |

Examples

'Galil DMC Code Example :PW test,test;' Set password to "test" :^L^K test,1;' Lock the program :LS;' Attempt to list the program ? :TC 1 106 Privilege violation :

^L^K applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,RIO47000,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

^L^K

SYSTEM CONFIG

#### ^R^S   Master Reset

^R^S

^R^S

| Usage | ^R^S | Command takes no arguments |
| --- | --- | --- |

Description

The Master Reset command resets the controller to factory default settings and erases EEPROM. A master reset can also be performed by installing a jumper at the location labeled MRST and resetting the board (power cycle or pressing the reset button). Remove the jumper after this procedure.

Arguments

^R^S has no parameters

Remarks

Sending a ^R^S over an Ethernet connection will cause the IP address to be cleared from the controller and will result in a timeout. ASCII Values

| Char | Dec | Hex |
| --- | --- | --- |
| ^R | 18 | 12 |
| ^S | 19 | 13 |

Examples

'Galil DMC Code Example REM Example burns-in a non-default value for KP, does a standard reset with REM the RS command, then performs a master reset with ^R^S. :KP? 6.00 :KP10 :KP? 10.00 :BN :RS :KP? 10.00 :^R^S :KP? 6.00 :

^R^S applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

^R^S

INTERROGATION

#### ^R^V   Revision Information

^R^V

^R^V

| Usage | ^R^V | Command takes no arguments |
| --- | --- | --- |

Description

The Revision Information command causes the controller to return the firmware revision information.

Arguments

^R^V has no arguments

Remarks

Do not use ^ symbols to send ^R^V command. ^ symbols denote using the control (Ctrl) key when pressing the characters. ASCII Values

| Char | Dec | Hex |
| --- | --- | --- |
| ^R | 18 | 12 |
| ^V | 22 | 16 |

Examples

'Galil DMC Code Example :^R^V DMC4143 Rev 1.1a1 :

^R^V applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

^R^V

### _   Operand Overview

#### Operand Usage

Operands allow motion or status parameters of the controller to be incorporated into programmable variables and expressions. Most DMC commands have an equivalent operand - which are designated by adding an underscore (_) prior to the DMC command. An operand typically contains the value of the command associated with it, for instance _TPA contains the current position of axis A. Below is an example of proper and improper usage for an operand.

#### Example Usage

'Galil DMC Code Example

'Correct usage

MG _TPA;' Message the A Axis' current position. err = _TC;' Save the current error code to a variable, err.

'Incorrect usage

_TPA;' Sending this to the controller will result in an error, as operands are not valid commands on their own.

#### Special Operands

The majority of DMC operands return information directly related to their command. However, there are a few operands which provide access to internal variables that are not accessible by standard DMC commands. Below is a list of special operands which contain information not stored in a typical DMC command.

For more details on the content of these operands, see their associated command page.

| Special Operand | Description |
| --- | --- |
| _BN | Contains the controller's serial number. |
| _BV | Contains the number of axes on the controller. |
| _DA | Contains the number of array space left in the controller's memory. |
| _DL | Contains the number of label space left in the controller's memory. |
| _DM | Contains the number of array space left in the controller's memory. |
| _ED0 | Contains the line number where an error last occured. |
| _ED1 | Contains the thread where an error last occured. |
| _ED4 | Contains the thread ID of the thread evaluating the operand. |
| _HXn | Contains the running status of thread 'n'. |
| _NO | Contains a bitmask of the running threads. |
| _RS | Contains a bitmask of checksum errors. |
| _UL | Contains the number of variables left in the controller's memory. |
| _XQn | Contains the current line number of thread 'n'. |
| TIME | Contains the current value of the controller's free running clock. |

_

ECAM/GEARING, OPERAND ONLY

### _GP GearingPhase Differential Operand

_GPm

_GPm

| Usage | variable= _GP | Holds a value |
| --- | --- | --- |
| Operands | _GPm | Operand has special meaning, see Remarks |

Description

The _GP operand contains the value of the specified slave's "phase differential" accumulated on the most recent change in the gear ratio between the master and the slave axes. The value does not update if the distance over which the slave will engage is set to 0 with the GD command.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis of interest |  |

Remarks

An operand is not valid individually. Instead, _GP would be used in an expression. See example below.

Phase Differential is a term that is used to describe the lead or lag between a master axis and the specified slave axis due to gradual gear shift. See application note 2440 for more information on _GP and GD.

Examples

'Galil DMC Code Example GA DA;' Sets the A axis aux encoder as the gearing master for the A axis. GD1000;' Set the distance that the master will travel to 1000 ' counts before the gearing is fully engaged for the A ' axis slave. AI-1;' Wait for input 1 to go low. In this example, this ' input is representing a sensor that senses an object ' on a conveyor. This will trigger the controller to ' begin gearing and synchronize the master and slave ' axes together. GR1;' Engage gearing between the master and slave p1=_TDA;' Sets the current A axis position to variable P1. This ' variable is used in the next command #wait ' Wait for the aux encoder to move forward 1000 ' encoder counts so the gearing engagement period is ' complete. Then the phase difference can be adjusted ' for. Note this example assumes forward motion. JP#wait, (_TDA < (p1+1000)) IP _GPA;' Increment the difference to bring the master/slave in ' position sync from the point that the GR1 command was ' issued. EN;' End Program

_GP applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

_GP

IO, OPERAND ONLY

### _LF ForwardLimit Switch Operand

_LFm

_LFm

| Usage | variable= _LF | Holds a value |
| --- | --- | --- |
| Operands | _LFm | Operand has special meaning, see Remarks |

Description

The _LF operand contains the state of the forward limit.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis of forward limit switch |  |

Remarks

_LF is an operand only with the following output:

_LFm = 1 when the limit switch state will allow motion in the positive direction.

_LFm = 0 when the limit switch state will not allow motion in the positive direction.

This operand is not a direct readout of the digital input and is affected by the command CN. See Connecting Hardware in User Manual for active/inactive state

Values of _LF

| Digital Input activation | _LF value for CN-1 | _LF value for CN1 |
| --- | --- | --- |
| On. Grounded for TTL, or sufficient activation current flowing for optos. | 0 (forward motion prohibited) | 1 (forward motion allowed) |
| Off. Pullup for TTL, or insufficient activation current flowing for optos. | 1 (forward motion allowed) | 0 (forward motion prohibited) |

Examples

'Galil DMC Code Example

MG _LFA;' Display the status of the A axis forward limit switch

_LF applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

_LF

IO, OPERAND ONLY

### _LR ReverseLimit Switch Operand

_LRm

_LRm

| Usage | variable= _LR | Holds a value |
| --- | --- | --- |
| Operands | _LRm | Operand has special meaning, see Remarks |

Description

The _LR operand contains the state of the reverse limit.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis of reverse limit switch |  |

Remarks

_LR is an operand with the following output

_LRm= 1 when the limit switch state will allow motion in the reverse direction.

_LRm= 0 when the limit switch state will not allow motion in the reverse direction.

This operand is not a direct readout of the digital input and is affected by the command CN. See Connecting Hardware in User Manual for active/inactive state

Values of _LR

| Digital input activation | _LR value for CN-1 | _LR value for CN1 |
| --- | --- | --- |
| On. Grounded for TTL, or sufficient activation current flowing for optos. | 0 (reverse motion prohibited) | 1 (reverse motion allowed) |
| Off. Pullup for TTL, or insufficient activation current flowing for optos. | 1 (reverse motion allowed) | 0 (reverse motion prohibited) |

Examples

'Galil DMC Code Example

MG _LRA;' Display the status of the A axis reverse limit switch

_LR applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

_LR

PROGRAMMING

### | Bitwise OR Operator

n | n

n | n

| Usage | variable = (value1 ¦ value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

The | symbol is the bitwise OR operator used with IF, JP, and JS decisions, and also to perform bitwise ORING of values.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to use with OR operator |  |

Remarks

For IF, JP, and JS, the values used for m are typically the results of logical expressions such as (x > 2) | (y=8)

The result of this operation is a value, which is not valid on its own. It must be coupled with a command. See examples below.

Examples

'Galil DMC Code Example 'Bitwise use var1=$F;'00001111 var2=$F0;'1111000 MG (var1 ¦ var2) EN REM Returned: 255.0000 (same as 11111111)

'Galil DMC Code Example 'Conditional Use var1=$F;'00001111 var2=$F0;'1111000

IF (var1 = $F) | (var2 = $F1)

MG"True"

ELSE

MG"False"

ENDIF

EN

REM Returned: True

#### | applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

|

PROGRAMMING

### ~ Variable Axis Designator

~s= "str"

~s= n

Description

~s= "str"

~s= n

Description

Variable axis designator. Each variable can be assigned an individual axis, a vector plane, or a virtual axis. Motion commands on the variable will then apply to the assigned axis.

Commands supporting variable axes are denoted in this command reference with the following icon.

Variable axis supported icon

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| s | a |  | h | N/A | N/A | Variable axis name | a,b,c,d,e,f,g,h supported |
| str | "A" |  | "H" | N/A | String | Name of axis | "A", "B", "C", "D", "E", "F", "G", "H" supported |
|  | "M" |  | "N" | N/A | String | Virtual axis | "M", "N" supported |
|  | "S" |  | "T" | N/A | String | Coordinate System | "S","T" supported |
| n | 0 |  | 7 | N/A | 1 | Index of the axis | A= 0, B= 1, C= 2, etc. |
|  | 8 |  | 9 | N/A | 1 | Coordinate System | S=8, T=9 |
|  | 10 |  | 11 | N/A | 1 | Virtual Axis | M= 11, N=10 |

Remarks

~s contains the axis number as defined by n and can be used in expressions (see example)

Examples

'Galil DMC Code Example

~a=2;~b=6;'     Sets ~a to 2 (Z axis).  Sets ~b to 6 (G axis)

MG"~a=",~a;'    Print axis number

MG"~b=",~b;'    Print axis number

PR~a=1000;'     Relative position move 1000 counts on ~a variable (set as Z axis)

JG~b=9000;'     Set jog speed of ~b variable (set as G axis) to 9000 cts/sec

BG~a~b;'        Begin motion on ~a and ~b variables (Z and G)

#### ~ applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

~

MATH FUNCTIONS

### + Addition Operator

n + n

n + n

| Usage | variable = (value1 + value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

The + symbol is the addition operator. It takes as arguments any two values, variables, array elements, operands, or At functions (@SIN[]) and returns a value equal to the sum of the arguments.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to use in addition operation |  |

Remarks

This is a binary operator (takes two arguments and returns one value). The result of this operation is a value, which is not valid on its own. It must be coupled with a command. See examples below.

Mathmatical operations are calculated left to right rather than multiplication and division calculations performed prior to addition and subraction. Example: 1+2*3 = 9;' not 7

It is recommended that parenthesis be used when more than one mathmatical operation is combined in one command. Example: var = ((10*30)+(60/30));' evaluates as 302 var = 10*30+60/30;' evalutes as 12

Examples

'Galil DMC Code Example :var1 = 1+2 :var2 = var1 + 1 :MG var2 + 2 6.0000 :

#### + applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

+

### < Less than comparator

n

0

< n

1

n

0

< n

1

| Usage | variable = (value1 < value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

"Less than" comparator for testing if one value is less than another. Comparators are used in mathematical expressions, IFs, and in conditional jumps. The result is a boolean.

Comparators in DMC Code

| Symbol | Comparator |
| --- | --- |
| < | Less than |
| > | Greater than |
| = | Equal to |
| <= | Less than or equal to |
| >= | Greater than or equal to |
| <> | Not equal to |

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |
| n1 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |

Remarks

A comparator is not a command and is not valid individually. Instead, the above expression would be used as part of a jump (JP,JS), IF expression, or assignment. See examples below.

If n0 < n1, the expression will evaluate to 1.0000. If the comparision is false, it will evaluate to 0.0000. Evaluation occurs left to right. Use parenthesis for operator precedence.

Examples

'Galil DMC Code Example :bool= (1<2) :MG bool 1.0000 :bool= (1<0) :MG bool 0.0000 :

'Galil DMC Code Example REM Example to find the largest REM value in an array REM REM *************************** REM Create an array and fill it len= 5 DM array[len] array[0]= 5 array[1]= 100.0001 array[2]= 42 array[3]= 3.14 array[4]= 100 JS #max;' call max subroutine MG "Max value is ", max EN REM REM **************************** REM Find max element in array #max i= 0 max = -2147483648;' start at min #max_h IF (array[i] > max) max = array[i] ENDIF i= i+1 JP #max_h, (i < len) EN REM REM **************************** REM Program output REM :XQ REM : REM Max value is 100.0001

<

#### < applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

<

### <=   Less thanor Equal to comparator

n

0

<= n

1

n

0

<= n

1

| Usage | variable = (value1 <= value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

"Less than or Equal to" comparator for testing if one value is less than or equal to another. Comparators are used in mathematical expressions, IFs, and in conditional jumps. The result is a boolean. Comparators in DMC Code

| Symbol | Comparator |
| --- | --- |
| < | Less than |
| > | Greater than |
| = | Equal to |
| <= | Less than or equal to |
| >= | Greater than or equal to |
| <> | Not equal to |

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |
| n1 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |

Remarks

A comparator is not a command and is not valid individually. Instead, the above expression would be used as part of a jump (JP,JS), IF expression, or assignment. See examples below.

If n0 <= n1, the expression will evaluate to 1.0000. If the comparision is false, it will evaluate to 0.0000. Evaluation occurs left to right. Use parenthesis for operator precedence.

Examples

'Galil DMC Code Example :bool= (1 <= 2) :MG bool 1.0000 :bool= (2 <= 2) :MG bool 1.0000 :bool= (3 <= 2) :MG bool 0.0000 :

'Galil DMC Code Example max= 2.05 min= 1.47 value = 0.025 JS #check value = 1.471 JS #check EN REM REM ******************************* REM Determine if in range #check inrange=0 IF ((value >= min) & (value <= max)) inrange= 1 ENDIF IF (inrange) MG "Value ",value," in range" ELSE MG "Value ",value," NOT in range" ENDIF EN REM REM ******************************* REM Program output REM :XQ REM : REM Value 0.0250 NOT in range REM Value 1.4710 in range

<= applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

<=

Galil Motion Control	<=	66/326

### <>   Not Equalto comparator

n

0

<> n

1

n

0

<> n

1

| Usage | variable = (value1 <> value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

"Not Equal to" comparator for testing if one value is not equal to another. Comparators are used in mathematical expressions, IFs, and in conditional jumps. The result is a boolean. Comparators in DMC Code

| Symbol | Comparator |
| --- | --- |
| < | Less than |
| > | Greater than |
| = | Equal to |
| <= | Less than or equal to |
| >= | Greater than or equal to |
| <> | Not equal to |

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |
| n1 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |

Remarks

A comparator is not a command and is not valid individually. Instead, the above expression would be used as part of a jump (JP,JS), IF expression, or assignment. See examples below.

If n0 <> n1, the expression will evaluate to 1.0000. If the comparision is false, it will evaluate to 0.0000. Evaluation occurs left to right. Use parenthesis for operator precedence.

Examples

'Galil DMC Code Example

:bool= (1 <> 2)

:MG bool

1.0000

:bool= (2 <> 2)

:MG bool

0.0000

'Galil DMC Code Example

REM Lock out code until

REM a particular digital

REM input pattern is detected

#AUTO

JS#lock;'block until pattern

REM

REM

REM Rest of code here

REM

REM

EN

REM

REM *************************

#lock

JP #lock, (_TI0 <> 170)

EN

<> applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

<>

### = Equal to comparator

n

0

= n

1

n

0

= n

1

| Usage | variable = (value1 = value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

"Equal to" comparator for testing if one value is equal to another. Comparators are used in mathematical expressions, IFs, and in conditional jumps. The result is a boolean.

Comparators in DMC Code

| Symbol | Comparator |
| --- | --- |
| < | Less than |
| > | Greater than |
| = | Equal to |
| <= | Less than or equal to |
| >= | Greater than or equal to |
| <> | Not equal to |

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |
| n1 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |

Remarks

A comparator is not a command and is not valid individually. Instead, the above expression would be used as part of a jump (JP,JS), IF expression, or assignment. See examples below.

If n0 = n1, the expression will evaluate to 1.0000. If the comparision is false, it will evaluate to 0.0000. Evaluation occurs left to right. Use parenthesis for operator precedence.

Examples

'Galil DMC Code Example :bool= (1=0) :MG bool 0.0000 :bool= (3.14=3.14) :MG bool 1.0000 :

'Galil DMC Code Example

REM Checks for a digital

REM input pattern and

REM sets a bit if matched

#loop

IF (_TI0 = 170)

SB 1

ELSE

CB 1

ENDIF

JP#loop

#### = applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

=

PROGRAMMING

= AssignmentOperator

str = n

Description

The = operator is the assignment operator for the controller. The assignment operator is used for two reasons:

- to define and initialize a variable (x = 0) before it is used

- to assign a new value to a variable (x = 5)

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 8 chars | N/A | String | Variable name to access |  |
| n | - 2,147,483,648 | 2,147,483,647 | see Notes | 1/65,536 | Value to assign to specified variable | Default n, or n = null results in a query of the value of variable |

Remarks

None

Examples

'Galil DMC Code Example :x=5 :x=? 5.0000 :MG x 5.0000 'define and initialize x to 5 'print x two different ways

### = applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

=

### > Greater than comparator

n

0

> n

1

n

0

> n

1

| Usage | variable = (value1 > value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

"Greater than" comparator for testing if one value is greater than another. Comparators are used in mathematical expressions, IFs, and in conditional jumps. The result is a boolean. Comparators in DMC Code

| Symbol | Comparator |
| --- | --- |
| < | Less than |
| > | Greater than |
| = | Equal to |
| <= | Less than or equal to |
| >= | Greater than or equal to |
| <> | Not equal to |

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |
| n1 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |

Remarks

A comparator is not a command and is not valid individually. Instead, the above expression would be used as part of a jump (JP,JS), IF expression, or assignment. See examples below.

If n0 > n1, the expression will evaluate to 1.0000. If the comparision is false, it will evaluate to 0.0000. Evaluation occurs left to right. Use parenthesis for operator precedence.

Examples

'Galil DMC Code Example :bool= (1>2) :MG bool 0.0000 :bool= (1>0) :MG bool 1.0000 :

'Galil DMC Code Example REM Example to find the largest REM value in an array REM REM *************************** REM Create an array and fill it len= 5 DM array[len] array[0]= 5 array[1]= 100.0001 array[2]= 42 array[3]= 3.14 array[4]= 100 JS #max;' call max subroutine MG "Max value is ", max EN REM REM **************************** REM Find max element in array #max i= 0 max = -2147483648;' start at min #max_h IF (array[i] > max) max = array[i] ENDIF i= i+1 JP #max_h, (i < len) EN REM REM **************************** REM Program output REM :XQ REM : REM Max value is 100.0001

>

#### > applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

>

### >=   Greater than or Equal to comparator

n

0

>= n

1

n

0

>= n

1

| Usage | variable = (value1 >= value2) | Performs an operation between two values or evaluated statements |
| --- | --- | --- |

Description

"Greater than or Equal to" comparator for testing if one value is greater than or equal to another. Comparators are used in mathematical expressions, IFs, and in conditional jumps. The result is a boolean. Comparators in DMC Code

| Symbol | Comparator |
| --- | --- |
| < | Less than |
| > | Greater than |
| = | Equal to |
| <= | Less than or equal to |
| >= | Greater than or equal to |
| <> | Not equal to |

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |
| n1 | -2,147,483,648 | 2,147,483,647 | N/A | 1/65,536 | Value to test |  |

Remarks

A comparator is not a command and is not valid individually. Instead, the above expression would be used as part of a jump (JP,JS), IF expression, or assignment. See examples below.

If n0 >= n1, the expression will evaluate to 1.0000. If the comparision is false, it will evaluate to 0.0000. Evaluation occurs left to right. Use parenthesis for operator precedence.

Examples

'Galil DMC Code Example :bool= (1 >= 2) :MG bool 0.0000 :bool= (2 >= 2) :MG bool 1.0000 :bool= (3 >= 2) :MG bool 1.0000 :

'Galil DMC Code Example max= 2.05 min= 1.47 value = 0.025 JS #check value = 1.471 JS #check EN REM REM ******************************* REM Determine if in range #check inrange=0 IF ((value >= min) & (value <= max)) inrange= 1 ENDIF IF (inrange) MG "Value ",value," in range" ELSE MG "Value ",value," NOT in range" ENDIF EN REM REM ******************************* REM Program output REM :XQ REM : REM Value 0.0250 NOT in range REM Value 1.4710 in range

>= applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### >=

Galil Motion Control	>=	73/326

PROGRAMMING

### AB Abort

AB

AB n

AB

AB n

| Usage | AB n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _AB | Operand has special meaning, see Remarks |

Description

The AB command is a command to issue an abort to controller operation.

AB (Abort) stops motion instantly without a controlled deceleration. If there is a program operating, AB can also be specified to abort the program and all running threads. The command, AB, will shut off the motors for any axis in which the off on error function is enabled (see command "OE").

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Abort motion and the program operation | Default if omitted |
|  | 1 | Abort motion only |  |

Remarks

_AB gives state of Abort Input, 1 inactive and 0 active.

AB aborts motion on all axes in motion and cannot stop individual axes.

Examples

'Galil DMC Code Example

:AB;'     Stops motion

:OE*= 1;' Enable off on error on axes

:AB;'     Shuts off motor command and stops motion

'Galil DMC Code Example

#A;'         Label - Start of program

JG 20000;'   Specify jog speed on A-axis

BG A;'       Begin jog on A-axis

WT 5000;'    Wait 5000 msec

AB 1;'       Stop motion without aborting program

WT 5000;'    Wait 5000 milliseconds

SH;'         Servo Here

JP #A;'      Jump to Label A

EN;'         End of the routine

'Remember to use the parameter 1 following AB if you only want the motion to be aborted 'Otherwise, your application program will also be aborted.

AB applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,RIO47000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### AB

INDEPENDENT MOTION AC Acceleration

## ACm= n AC n,n,n,n,n,n,n,n

| Usage | ACm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | AC n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _ACm | Operand holds the value last set by the command |

Description

The Acceleration command (AC) sets the linear acceleration of the motors for independent moves, such as PR, PA, and JG moves. The parameters will be rounded down to the nearest factor of 1024 and have units of counts per second squared.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
|  | M | N | N/A | Axis | Virtual axis to assign value |  |
| n | 1,024 | 1,073,740,800 | 256,000 | 1,024 | Acceleration rate | At TM 1000. Resolution and Min depend on TM, see remarks. |

Remarks

The AC command is used to designate acceleration

Specify realistic acceleration rates based on physical system parameters such as:

motor torque rating loads

amplifier current rating

Specifying an excessive acceleration will cause a large following error during acceleration and the motor will not follow the commanded profile The acceleration feedforward command (FA) will help minimize the error for aggressive accelerations

### Resolution

The Min and Resolution depend on the sampling period of the control loop (TM). The equation to calculate these values is: Resolution = Min = 1024*(1000/TM)^2 example:

With TM 500 the minimum AC setting and resolution is 4096 counts/second^2 resolution = 1024*(1000/500)^2 = 4096

Examples

'Galil DMC Code Example SHAB;' Enable axes A and B REM Set a different acceleration for each axis AC 10000,400000; REM Axis A acceleration is 10000 cts/sec REM Axis B acceleration is 400000 cts/sec SP 40000,40000;' Set the speed for each axis to 40000 cts/sec a= _ACB;' Assigns the B acceleration to the variable a PR 100000, 100000;' Set the move distance to 100000 cts BG AB;' Begin motion on both axes t=TIME;' Assign the time started to a variable AMB;' Halt code execution until motion is completed on axis B MG{Z5.0} "B axis finished in",TIME - t," samples" AMA;' Halt code execution until motion is completed on axis A MG{Z5.0} "A axis finished in",TIME - t," samples" MO;' Disable all axes after moves are completed EN

AC applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AC

TRIPPOINT AD After Distance

## ADm= n AD n,n,n,n,n,n,n,n

| Usage | ADm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | AD n ... | Arguments specified with an implicit, comma-separated order |

Description

Trippoint to block command execution until a given distance is traversed. This is a profiled trippoint which means it depends on the motion profiler and not the actual motor encoder. AD can only be used when there is commanded motion on the axis.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 2,147,483,647 | N/A | 1 | Distance of motion | Cannot specify more than 1 argument at a time |

Remarks

AD will hold up the execution of the following command until one of the following conditions have been met The commanded motor position crosses the specified relative distance from the start of the move The motion profiling on the axis is complete

If in jog (JG) mode, the commanded motion is in the direction which moves away from the specified position

Not valid for a slave during ECAM or Gearing, use MF and MR

If the direction of motion is reversed when in PT mode, the starting position for AD is reinitialized to the position at which the motor is reversed The AD command is accurate to the number of counts that occur in 2*TM msec

AD command will be affected when the motion smoothing time constant, IT, is not 1. See IT command for further information AD measures incremental distance from start of move on one axis

Examples

'Galil DMC Code Example

#A

DP 0,0;'                  Zero position

PR 10000,20000;'          Specify position relative moves

BG ;'                     Begin motion

AD 5000;'                 After A reaches 5000

MG "Halfway to A";TP A;'  Send message

AD ,10000;'               After B reaches 10000

MG "Halfway to B";TP B;'  Send message

EN;'                      End Program

AD applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AD

FILTER/CONTROL

### AF Analog Feedback Select

AFm= n

AF n,n,n,n,n,n,n,n

AFm= n

AF n,n,n,n,n,n,n,n

| Usage | AFm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | AF n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _AFm | Operand holds the value last set by the command |

Description

The AF command configures analog feedback mode for the PID filter.

The controller ADC can be used as position feedback for the axis control law. The analog input used for feedback is fixed and uses the input that corresponds with the axis letter. For example, Analog input 1 is used for the A axis.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 1 |  | 0 | 1 | Use the controller ADC as servo feedback | 1= analog feedback, 0= digital feedback |

Remarks

Below is the feedback in counts decoded by the controller hardware when reading in analog feedback for certain analog input ranges.

|  | 12 Bit ADC | 16 Bit ADC |
| --- | --- | --- |
| -5V to +5V, -10V to +10V | -2048 to 2047 counts | -32768 to 32767 counts |
| 0V to +5V, 0V to +10V | 0 to 4095 counts | 0 to 65535 counts |

The analog voltage range is set using the AQ command. AQ must be set prior to setting AF

The analog feedback is decoded by a 12-bit A/D converter. An upgrade option is available for 16-bits.

Examples

'Galil DMC Code Example AF 1;' Analog feedback on A axis v1= _AFA;' Assign feedback type to variable KP 1;' Assigns PID's for motor using analog feedback on A-axis KD 10;' KI 0.5;'

AF applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AF

FILTER/CONTROL

### AG Amplifier Gain

AG n,n,n,n,n,n,n,n

AGm=n

AG n,n,n,n,n,n,n,n

AGm=n

| Usage | AGm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | AG n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _AGm | Operand holds the value last set by the command |

Description

The AG command sets the amplifier current/voltage gain for the internal amplifier. Note: some Galil internal amplifiers have fixed gains. Please reference the manual or data-sheet for more details.

For Servo motors, to convert motor command output (V) to actual motor current (A), use the following equation.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 3 |  | 1 | 1 | Gain setting | See table in Remarks for gain settings |

Remarks

#### Current Gain Settings with Servo Amplifiers

Gain settings by Amplifier (Amps/Volt)

| Gain Setting, n= |  | 0 | 1 | 2 | Notes |
| --- | --- | --- | --- | --- | --- |
| D3040 | 0.4 |  | 0.7 | 1 |  |
| D3140 | N/A |  | N/A | N/A | Fixed at 0.1 A/V |
| D3240 | 0.5 |  | 1 | 2 |  |
| D3540 | 0.4 |  | 0.8 | 1.6 |  |
| D3547 | 0.4 |  | 0.8 | 1.6 |  |
| D3640 | N/A |  | N/A | N/A | Fixed at 0.2 A/V |

#### Current Settings with Stepper Drivers

Current settings by Amplifier (Amps per phase)

| Current Setting, n= | 0 |  | 1 |  | 2 |  | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D3547 | 0.75 | 1.5 |  | 3 |  | 6 |  |
| D4040 | 0.5 | 0.75 |  | 1 |  | 1.4 |  |
| D4140 | 0.5 | 1 |  | 2 |  | 3 |  |

The axis should be in the motor off state (MO) before setting AG.

The MT command must be issued prior to the AG command to set the proper range.

Related Commands

#AMPERR - Amplifier error automatic subroutine

AU - Set amplifier current loop

AZ - Clear Amplifier Errors

MT - Motor Type

TA - Tell Amplifier Error

TK - Peak Torque Limit

TL - Torque Limit

Examples

'Galil DMC Code Example

ST ;'      Stop any motion

AM ;'      Wait for motion to decel and stop

MO ;'      Turn motor off

MT 1;'     Set the A axis as a servo

AG 2;'     Sets the highest amplifier gain for A axis on servo amplifier

BN;'      Save AG setting to EEPROM

AG applies to DMC50000,DMC4000,DMC4103,DMC30010,DMC2103,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AG

TRIPPOINT

### AI After Input

AI n

AI n

| Usage | AI n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The AI command is a trippoint used in motion programs to wait until after a specified input has changed state. This command can be configured such that the controller will wait until the input goes high or the input goes low.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 1 | 16 | N/A | 1 | General input to use for trippoint | +n = High trigger. -n = low trigger. 9-16 only valid for 5-8 axis controller |
|  | 81 | 96 | N/A | 1 | Aux encoder input to use for trippoint |  |

Remarks

The AI command actually halts execution until specified input is at desired logic level. Use the conditional Jump command (JP) or input interrupt (II) if you do not want the program sequence to halt.

AI functions only on local input points. See Example below for network based digital inputs.

Examples

'Galil DMC Code Example

#A;'         Begin Program

AI 8;'       Wait until input 8 is high

SP 10000;'   Speed is 10000 counts/sec

AC 20000;'   Acceleration is 20000 counts/sec2

PR 400;'     Specify position

BGA;'        Begin motion EN;'         End Program

'Galil DMC Code Example

REM When using a remote I/O device (e.g. the RIO), the following provides

REM a similar function as AI. Assume that the remote device is already

REM configured on handle C (see IH)

'code before

JS #remote;' this call blocks and waits for the remote logic to return 'code after

EN

'***** The example subroutine *****

#remote

WT10;' wait a reasonable interval so we don't flood the network

JP#remote,(@IN[3001] = 1);'loop while input 1 on the remote device is high EN;' return to calling code.

AI applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AI

IO

### AL ArmLatch

AL mm

AL mm

| Usage | AL mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _ALm | Operand has special meaning, see Remarks |

Description

The AL command enables the latch function (high speed main or auxiliary position capture) of the controller. When the position latch is armed, the main or auxiliary encoder position will be captured upon a rising or falling edge on the specified digital input. Use the CN command to configure the edge that the latch input will trigger on.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| mm | A | ABCDEFGH | N/A | Multi-Axis Mask | Encoder to latch | Latch main encoder |
| mm | SA | SASBSCSDSESFSGSH | N/A | Multi-Axis Mask | Encoder to latch | Latch aux encoder |
| mm | TA | TATBTCTDTETFTGTH | N/A | Multi-Axis Mask | Index input to trigger latch | Main encoder is latched from the index pulse instead of a digital input |

Remarks

Latch input by Axis

| Axis | Latch Input |
| --- | --- |
| A | Input 1 |
| B | Input 2 |
| C | Input 3 |
| D | Input 4 |
| E | Input 9 |
| F | Input 10 |
| G | Input 11 |
| H | Input 12 |

The command RL returns the latched position

_ALm contains the state of the specified latch. 0 = not armed, 1 = armed

The CN command can be used to change the edge which causes the latch to trigger.

The latch function is available on incremental quadrature encoder inputs only. For other position capture methods contact Galil.

Examples

'Galil DMC Code Example

#start

AL A;'              Arm A-axis latch

JG 50000;'          Set up jog at 50000 counts/sec

BG A;'              Begin the move

#loop;'             Loop until latch has occurred

JP #loop,(_ALA=1)

RL A;'              Transmit the latched position

EN;'                End of program

'Galil DMC Code Example REM Homing routine using the AL command to detect the Motor's index position #start AL TA;' Arm A-axis latch. Latch will trigger off the index pulse JG 50000;' Set up jog at 50000 counts/sec BG A;' Begin the move #loop;' Loop until latch has occurred JP #loop,(_ALA=1) STA;' Stop the jog AMA PAA=_RLA;' Set up a move to return to the latched position BGA AMA WT100;' Allow for settling. REM Checking that KI has eliminated error (TE) would be more thorough DP0;' Zero position MG "A Homed";' Report status EN;' End of program

'Galil DMC Code Example REM manual find index using latch off of index pulse using variable axes #index 'settings ~a=0;'axis number s1spd=10000;'stage 1 speed

#### AL

s2spd=1000;'stage 2 speed 'jog until index is latched

JG~a=s1spd;BG~a

WT1000

ALT~a

#A;JP#A,_AL~a=1

ST~a;AM~a

'Return to latched position

SP~a=s2spd

PA_RL~a;BG~a;MC~a

JS#STL;DP~a=0

EN

'wait for axis to settle #STL

WT2;JP#STL,@ABS[_TE~a]>2

WT2;JP#STL,@ABS[_TE~a]>0

WT2;JP#STL,@ABS[_TE~a]>0

EN

AL applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AL

### AM	Move

AM mm

AM mm

| Usage | AM mm | Argument is an axis mask |
| --- | --- | --- |

Description

The AM command is a trippoint used to control the timing of events. This command will hold up execution of the following commands until the current move on the specified axis or axes is completed. Any combination of axes or a motion sequence may be specified with the AM command.

For example, AM AB waits for motion on both the A and B axis to be complete. AM with no parameter specifies that motion on all axes to be complete.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to wait for profiled motion to complete |  |
|  | S |  | T | N/A | Multi-Axis Mask | Vector plane to wait for profiled motion to complete |  |

Remarks

AM is a very important command for controlling the timing between multiple move sequences.

For example, if the A-axis is in the middle of a position relative move (PR) you cannot make a position absolute move (PAA, BGA) until the first move is complete. Use AMA to halt the program sequence until the first profiled motion is complete.

AM tests for profile completion only. The actual motor may still be moving. To halt the program sequence until the actual physical motion has completed, use the MC command.

To test motion complete without halting the program sequence, use the operand _BGn, which will be zero when profiled motion is complete (see BG command).

Examples

'Galil DMC Code Example

#MOVE;                 'Program MOVE

PR 5000,5000,5000,5000;'Position relative moves

BG A;                  'Start the A-axis

AM A;                  'After the move is complete on A,

BG B;                  'Start the B-axis

AM B;                  'After the move is complete on B, BG C;                  'Start the C-axis

AM C;                  'After the move is complete on C

BG D;                  'Start the D-axis

AM D;                  'After the move is complete on D

EN;                    'End of Program

AM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AM

IO

### AO Analog Output

AO n

0

,n

1

AO n

0

,n

1

| Usage | AO n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The AO command sets the analog outputs on the Galil or for a Modbus Slave.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | 1,000 | 8,999 | N/A | 1 | Set Analog Output on Modbus Slave | See "Using AO with a Modbus Slave" in Remarks |
| n1 | -9.9998 | 9.9998 | N/A | 20/65,536 | Analog Output Voltage |  |

Remarks

#### Using AO with a Modbus Slave

RIO as Modbus Slave

3rd Party Modbus Slave Device n0 is the I/O number calculated using the following equations:

n0 = (HandleNum*1000) + ((Module-1)*4) + (Bitnum-1) HandleNum is the handle specifier from A to H.

Handle must be assigned to port 502 for Modbus comms (See IH) Module is the position of the module in the rack from 1 to 16. BitNum is the I/O point in the module from 1 to 4

Examples

'Galil DMC Code Example

:AO 3005,3.2;'          Outputs 3.2 Volts on Channel 5 of the Device connected to Handle C

AO applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,RIO47000,RIO57400,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AO

AP	Absolute Position

## APm= n AP n,n,n,n,n,n,n,n

| Usage | APm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | AP n ... | Arguments specified with an implicit, comma-separated order |

Description

The AP command will hold up the execution of the following command until the actual motor position crosses the specified position. This trippoint does not rely on the profiler, but on actual encoder position.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1 | Position trippoint value | Only one axis may be specified at a time |

Remarks

For AP command to clear, one of the following conditions have been met:

The actual motor position crosses the specified absolute position.

The motion profiling on the axis is complete.

The commanded motion is in the direction which moves away from the specified position.

The units of the command are quadrature counts.

When using a stepper motor, the AP trippoint condition is satisfied when the stepper position (TD) has crossed the specified position. For further information see Chapter 6 of the User Manual "Stepper Motor Operation".

Not valid for a slave during ECAM or Gearing - use MF and MR.

The motion profiler must be active before the AP command is used.

AP is accurate to the number of counts that occur in 2*TM msec

AP tests for absolute position. Use the AD command to measure incremental distances.

Examples

'Galil DMC Code Example

#TEST;' Program B

DP0;' Define zero

JG 1000;' Jog mode (speed of 1000 counts/sec)

BG A;' Begin move

AP 2000;' After passing the position 2000

V1=_TPA;' Assign V1 A position

MG "Position is", V1;' Print Message

ST;' Stop

EN;' End of Program

AP applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AP

IO

### AQ Analog Input Configuration

AQ n

0

,n

1

AQ n

0

,n

1

| Usage | AQ n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _AQ1 _AQ2 _AQ3 _AQ4 _AQ5 _AQ6 _AQ7 _AQ8 | Operand has special meaning, see Remarks |

Description

The AQ command is used to set the behavior of the analog inputs. This command will set the analog range and operation for the specified input.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 1 |  | 8 | N/A | 1 | Analog input channel |  |
| n1 | 1 |  | 4 | 2 | 1 | Analog range setting | See Table Below |
|  | -4 |  | -1 | N/A | 1 | Specify analog input is differential | See Remarks |

Remarks

AQ is a configuration command which must be set at the beginning of application code.

The usage of this command depends on the type of analog inputs present on the particular controller model, check the ID command to determine the hardware configuration.

Configurable Analog Input Settings

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n1 | 1 | -5V to +5V |  |
|  | 2 | -10V to +10V | Default |
|  | 3 | 0V to +5V |  |
|  | 4 | 0V to+10V |  |

Default resolution for analog inputs is 12bits. 16 bit is optional.

Operands _AQ1 through _AQ8 return the setting for the specified input

Setting a negative n1 for inputs 1,3,5 or 7, configures those inputs as the differential input relative to input 2,4,6 and 8 respectively.

#### Differential Input Mapping (-n1)

| Input (n0) | Compliment (n0 + 1) |
| --- | --- |
| 1 | 2 |
| 3 | 4 |
| 5 | 6 |
| 7 | 8 |

Position Range when in Analog Feedback by AQ

| Argument | Value | Analog Range | Position Range (12 bit) | Position Range (16 bit) |
| --- | --- | --- | --- | --- |
| n1 | 1 | -5V to +5V | -2048 to 2047 | -32,768 to 32767 |
|  | 2 | -10V to +10V | -2048 to 2047 | -32,768 to 32767 |
|  | 3 | 0V to 5V | 0 to 4095 | 0 to 65535 |
|  | 4 | 0V to10V | 0 to 4095 | 0 to 65535 |

Examples

'Galil DMC Code Example

:AQ2,3;'      Specify analog input 2 as 0-5V

:AQ1,-3;'     Specify analog input 1 as 0-5V and the differential input to analog input 2

:MG_AQ2

3.0000

AQ applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC52000,RIO57400,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AQ

AR	Relative Distance

## ARm= n AR n,n,n,n,n,n,n,n

| Usage | ARm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | AR n ... | Arguments specified with an implicit, comma-separated order |

Description

The After Relative (AR) command is a trippoint used to control the timing of events. This command will hold up the execution of the following command until one of the following conditions have been met:

- The commanded motor position crosses the specified relative distance from either the start of the move or the last AR or AD command.

- The motion profiling on the axis is complete.

- If in jog (JG) mode, the commanded motion is in the direction which moves away from the specified position.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1 | Relative position for trippoint | Only one axis may be specified at a time. |

Remarks

The units of the command are quadrature counts.

When using a stepper motor, this condition is satisfied when the stepper position (as determined by the output buffer) has crossed the specified Relative Position.

For further information see Chapter 6 of the User Manual "Stepper Motor Operation".

If the direction of the motion is reversed when in position tracking mode (see PT command), the starting point for the trippoint is reinitialized to the point at which the motion reversed.

The motion profiler must be active before the AR command is issued.

Not valid for a slave during ECAM or Gearing - use MF and MR.

Note: AR will be affected when the motion smoothing time constant, IT, is not 1. See IT command for further information. AP is accurate to the number of counts that occur in 2*TM msec AR is used to specify incremental distance from last AR or AD command.

Use AR if multiple position trippoints are needed in a single motion sequence.

Examples

'Galil DMC Code Example

#A;'              Begin Program

DP 0

JG 50000;'        Specify speed

BG A;'            Begin motion

#B;'              Label

AR 25000;'        After passing 25000 counts of relative distance on A-axis

MG "Passed";TPA;' Send message on A-axis

JP #B;'           Jump to Label #B

EN;'              End Program

AR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AR

### AS At Speed

AS mm

AS mm

| Usage | AS mm | Argument is an axis mask |
| --- | --- | --- |

Description

The AS command is a trippoint that occurs when the generated motion profile has reached the specified speed. This command will hold up execution of the following command until the commanded speed has been reached. The AS command will operate after either accelerating or decelerating.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGHST | ABCDEFGH | Multi-Axis Mask | Axes to use for AS trippoint |  |

Remarks

If the speed is not reached, the trippoint will be triggered after the speed begins diverging from the AS value.

'The AS command applies to a trapezoidal velocity profile only with linear acceleration. AS used with Smoothing profiling will be inaccurate.

Examples

'Galil DMC Code Example

#SPEED;'        Program

PR 100000;'     Specify position

SP 10000;'      Specify speed

BG A;'          Begin A

AS A;'          After speed is reached

MG "At Speed";' Print Message

EN;'            End programm

AS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AS

### AT At Time

AT n

0

,n

1

AT n

0

,n

1

| Usage | AT n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The AT command is a trippoint which is used to hold up execution of the next command until after the specified time has elapsed. The time is measured with respect to a defined reference time. AT 0 establishes the initial reference.

AT n,1 specifies n samples from the reference. This is useful when TM is lowered and faster application loop times are required.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -2,147,483,648 | 2,147,483,647 | 0 | 2 | Specify a wait time for AT trippoint | See Remarks |
| n1 | 0 | 1 | 0 | 1 | Specify time in samples or msecs | n1=0 for msecs. n1=1 for samples |

Remarks

n0 = 0 sets the reference time for AT to the current time.

n0 > 0 specifies the wait time as the absolute value of n0 from the reference time n0 < 0 specified the wait time as the absolute value of n0 from the reference time, and resets the reference time when the trippoint is complete to the current time.

AT -n0 is equivalent to AT n0; AT (old reference +n0)

Examples

'Galil DMC Code Example

'The following commands are sent sequentially

AT 0;' Establishes reference time 0 as current time

AT 50;' Waits 50 msec from reference 0

AT 100;' Waits 100 msec from reference 0

AT -150;' Waits 150 msec from reference 0 and sets new reference at 150

AT 80;' Waits 80 msec from new reference (total elapsed time is 230 msec)

'Galil DMC Code Example ' jog propotional to analog input example with AT in ms 'AT -n #main0 AT0;' set time reference for AT command JG0;BGX;' start Jog mode gain=1 #atloop jgspd=gain*@AN[1] JG jgspd AT-100;' wait 100 ms from last time reference (last AT-n or AT0) REM same functionality would be: REM AT -100,0 REM -or- REM AT 100,0;AT0 JP#atloop

'Galil DMC Code Example ' jog propotional to analog input example with AT in samples ' AT n,1 #main1 AT0;' set time reference for AT command JG0;BGX;' start Jog mode gain=1 #atloop jgspd=gain*@AN[1] JG jgspd AT -100,1;' wait 100 samples from last time reference (AT0) JP#atloop

AT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AT

FILTER/CONTROL

### AU Set amplifier current loop

AUm= n

AU n,n,n,n,n,n,n,n

AUm= n

AU n,n,n,n,n,n,n,n

| Usage | AUm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | AU n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _AUm | Operand holds the value last set by the command |

Description

The AU command sets the amplifier current loop gain for internal amplifiers.

For Galil trapezoidal amplifiers, the current loop is available in one of two settings. AU also sets the switching mode where available, Chopper vs. Inverter. For Galil sinusoidal amplifiers, AU sets the current loop gain as well as enables or disables the current loop integrator.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | Amplifier Specific | Amplifier Specific | Amplifier Specific | Amplifier Specific | Current Loop Gain Setting | See tables below for setting the n parameter by amplifier. |

#### D3040

| Argument | Value | Description | (24VDC Bus) Current loop setting | (48VDC Bus) Current loop setting | Notes |
| --- | --- | --- | --- | --- | --- |
| n | 0 | Inverter mode, Normal current loop gain | 0.5mH < L <5mH | 0.5mH < L <10mH | Default. |
|  | 0.5 | Chopper mode, Normal current loop gain | 0.2mH < L < 0.5mH | 0.2mH < L < 0.5mH |  |
|  | 1 | Inverter mode, Higher current loop gain | 5mH < L | 10mH < L |  |
|  | 1.5 | Chopper mode, Higher current loop gain | 5mH < L | 10mH < L |  |

#### D3240

| Argument | Value | Description | (24VDC Bus) Current loop setting | (48VDC Bus) Current loop setting | Notes |
| --- | --- | --- | --- | --- | --- |
| n | 0 | Chopper mode, Normal current loop gain | 0.5mH < L <5mH | 0.5mH < L <10mH | Default. |
|  | 1 | Chopper mode, Higher current loop gain | 5mH < L | 10mH < L |  |

#### D3540

| Argument | Value | (24VDC Bus) Current loop setting | (48VDC Bus) Current loop setting | Notes |
| --- | --- | --- | --- | --- |
| n | 1 | L < 1mH | L < 2.4mH | Default1 |
|  | 9 | L < 1mH | L < 2.4mH |  |
|  | 10 | 1mH < L < 2.3mH | 2.4mH < L < 4.2mH |  |
|  | 11 | 2.3mH < L < 4.2mH | 4.2mH< L < 7mH |  |
|  | 12 | 4.2mH < L | 7mH < L |  |

1. AU1 is default for backwards compatibility. AU 9 through 12 should be used for new applications.

#### D3547

| Argument | Value | (24VDC Bus) Current loop setting | (48VDC Bus) Current loop setting | Notes |
| --- | --- | --- | --- | --- |
| n | 9 | L < 1mH | L < 2.4mH | Default |
|  | 10 | 1mH < L < 2.3mH | 2.4mH < L < 4.2mH |  |
|  | 11 | 2.3mH < L < 4.2mH | 4.2mH < L < 7mH |  |
|  | 12 | 4.2mH < L | 7mH < L |  |

Remarks

The axis must be in the motor off state (MO) before setting AU

The AU settings for Galil sinusoidal amplifiers are only recommended values for the given bus voltages. For other bus voltages and their recommended settings, contact Galil.

Related Commands

#AMPERR - Amplifier error automatic subroutine

AG - Amplifier Gain

AZ - Clear Amplifier Errors

MT - Motor Type

#### AU

TA - Tell Amplifier Error

TK - Peak Torque Limit

TL - Torque Limit

Examples

'Galil DMC Code Example 'Example setting for Galil trapezoidal amplifier :AU1,0;' Sets A-axis to higher loop gain and B-axis to normal loop gain :AUB=?;' Query B-axis current loop gain 0 :MG_AUA;' Query A axis current loop gain 1

'Galil DMC Code Example 'Example setting for Galil sinusoidal amplifier 'inductance = 2.6mH :AU11;' Sets A-axis for motor inductance 2.6mH at 24V, current loop enabled :MG_AUA;' Query A axis current loop gain 11

AU applies to DMC50000,DMC4000,DMC4103,DMC30010,DMC2103,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AU

VECTOR/LINEAR, TRIPPOINT

### AV   After Vector Distance

AV n

0

,n

1

AV n

0

,n

1

| Usage | AV n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _AVS _AVT | Operand has special meaning, see Remarks |

Description

The AV command is used to hold up execution of the next command during coordinated moves such as VP,CR or LI. This trippoint occurs when the path distance of a sequence reaches the specified value. The distance is measured from the start of a coordinated move sequence or from the last AV command.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 |  | 2,147,483,647 | 0 | 1 | Vector distance to be executed in the S coordinate system |  |
| n1 | 0 |  | 2,147,483,647 | 0 | 1 | Vector distance to be executed in the T coordinate system |  |

Remarks

The units of the command are quadrature counts.

_AVS contains the vector distance from the start of the sequence in the S coordinate system

_AVT contains the vector distance from the start of the sequence in the T coordinate system.

Examples

'Galil DMC Code Example

#MOVE;'        Label to desginate start of program

DP 0,0;'       Define the A and B axis positions as 0

CAT;'          Specify the T coordinate system

LMAB;'         Linear move for A,B

LI 1000,2000;' Specify distance

LI 2000,3000;' Specify distance

LE

BGT;'          Begin motion in the T coordinate system

AV ,500;'      After path distance = 500,

TPAB;'         Print position of A and B axes

EN;'           End Program

'Vector Distance is calculated as the square root of the sum of the 'squared distance for each axis in the linear or vector mode.

AV applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AV

### AZ   Clear Latched Amplifier Errors

AZ n

AZ n

| Operands | _AZ2 | Operand has special meaning, see Remarks |
| --- | --- | --- |

Description

The AZ command is used to enable enhanced error reporting and clear any latched amplifier errors. When using certain amplifiers, error conditions will stay active until cleared. This persistent error state is defined as a latched error.

Arguments

| Argument | Value | Details |
| --- | --- | --- |
| n | 1 | Clear latched amplifier errors. |
|  | 2 | Enable Enhanced Error Clearing. |

Issue AZ2 to enable enhanced error reporting.

When all axes are in a motor off state (MO) and there are no latched errors, amplifier errors will be reported live by the TA command.

While an axis is in a servo here state (SH), the following amplifier errors will latch:

Over-Current

Over-Temperature

Under-Voltage

ELO

NOTE: The Over-Voltage amplifier error does not latch. While the amplifier is in an Over-Voltage condition, the amplifier will short the motor phases resulting in a drag being felt on the motor.

To clear latched amplifier errors, issue MO followed by AZ1.

Remarks

_AZ2 contains a 0 if the amplifier is in normal error reporting.

_AZ2 contains a 1 if the amplifier is in enhanced error reporting

Refer to the controller User Manual for more information on clearing amplifier errors.

| Amplifier | Enhanced Error Clearing Mode |
| --- | --- |
| D3540 | Disabled by default |
| D3547 | Always enabled |

Related Commands

#AMPERR - Amplifier error automatic subroutine

AG - Amplifier Gain

AU - Set amplifier current loop

MT - Motor Type

TA - Tell Amplifier Error

TK - Peak Torque Limit

TL - Torque Limit

Examples

'Galil DMC Code Example #AMPERR ST;' stop motion on all axes AM;' wait until motion is halted MO;WT2;' disable all axes IF((_TA0&1)¦(_TA0&16));' check if an Over-Current error occurred MG "Over-Current amplifier error" ENDIF IF((_TA0&2)¦(_TA0&32));' check if an Over-Voltage error occurred MG "Over-Voltage amplifier error" ENDIF IF((_TA0&4)¦(_TA0&64));' check if an Over-Temperature error occurred MG "Over-Temperature amplifier error" ENDIF IF((_TA0&8)¦(_TA0&128));' check if an Under-Voltage error occurred MG "Under-Voltage amplifier error" ENDIF IF((_TA3&1)¦(_TA3&2));' check if the ELO input was asserted MG "ELO input asserted" ENDIF IF((_TA3&64)¦(_TA3&128));' check if an amplifier error has latched MG "Amplifier error has latched"

#### AZ

AZ1;WT2;' clear latched amplifier errors ENDIF

RE

AZ applies to DMC4000,DMC4103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

AZ

### BA	Axis

BA mm

BA mm

| Usage | BA mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _BAm | Operand has special meaning, see Remarks |

Description

For axes equipped with a Galil sinusoidal amplifier, BA is used to configure the axis for sinusoidal commutation. In addition to BA, the BM command as well as either BX, BZ or BI/BC, must be used to initialize the axis for sinusoidal commutation.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | ABCDEFGH | N/A | Multi-Axis Mask | Axes to configure for sinusoidal commutation. |  |
|  | N |  | N | N/A | Multi-Axis Mask | Disable sine commutation for all axes. |  |

Remarks

_BAm will contain a 1 if the BA command has been issued for the speciifed axis, or a 0 if it has not.

There are several methods to initialize Galil's internal amplifiers for sinusoidal commutation. They are listed below: Initialization of a Galil Sinusoidal Amplifier

| Command | Description | Notes |
| --- | --- | --- |
| BI/BC | Uses hall sensors to estimate the commutation angle until a hall transition can be used to set the commutation angle precisely. | This is the recommended method if hall sensors are present. |
| BZ | Commands the axis to a specific commutation angle. | This is the recommended method if hall sensors are not present. |
| BX | Uses an algorithm to determine the commutation angle with minimal motion. | Should only be used if minimal motion is required during intialization and no hall sensors are present. |

Examples

'Galil DMC Code Example REM Example BZ initialization procedure BA A;' Designate sinusoidal commutation BM 2000;' Length of electrical cycle in counts--required setting for commutation BZ <1000>1500;' Set the first BZ time to 1500 msec, and second B time to 1000 msec. BZ 3;' Commutate motor using 3 V and timeout after 1000 msec SH A;' Enable motor, ready for commands EN

'Galil DMC Code Example REM Example BI/BC initialization procedure BAA;' Configure the A Axis for sinusoidal commutation. BMA=2000 BIA=-1;' Set estimated commutation based on current hall state BCA;' Enable brushless calibration hall=_QHA;' Store hall state SHA;' Enable amplifier JGA=500;' Slow jog so that the commutation angle is set precisely at ' the next hall transition. BGA;' Begin jog #hall;WT2;JP#hall,_QHA=hall;' Wait for a hall transition ' At this point, a precise commutation angle is set ' and the axis is fully configured for sinusoidal commutation STA AMA EN

BA applies to DMC4000,DMC4103,DMC30010,DMC50000,EDD37010,DMC1802,DMC1806,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BA

### BC	Calibration

BC mm

BC mm

| Usage | BC mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _BCm | Operand has special meaning, see Remarks |

Description

The BC command is used along with the BI command to initialize an axis for sinusoidal commutation using hall sensors. BC monitors the status of the hall sensors, and precisely sets the commutation angle upon detecting a hall sensor transition. In addition to BI and BC, the BA and BM commands must be used to initialize the axis for sinusoidal commutation.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to initialize using hall sensors. |  |

Remarks

GDK's Step-By-Step tool can be utilized for automatic setup and configuration of brushless motor with Galil internal sinusoidal amplifiers. When performing the BI/BC initialization method, the axis should be moved at a low speed until the first hall transition occurs. This ensures the commutation angle is accurately set.

BI/BC initialization is valid with Galil internal sinusoidal amplifiers.

There are several methods to initialize Galil's internal amplifiers for sinusoidal commutation. They are listed below: Initialization of a Galil Sinusoidal Amplifier

| Command | Description | Notes |
| --- | --- | --- |
| BI/BC | Uses hall sensors to estimate the commutation angle until a hall transition can be used to set the commutation angle precisely. | This is the recommended method if hall sensors are present. |
| BZ | Commands the axis to a specific commutation angle. | This is the recommended method if hall sensors are not present. |
| BX | Uses an algorithm to determine the commutation angle with minimal motion. | Should only be used if minimal motion is required during intialization and no hall sensors are present. |

#### Steps for BI/BC sinusoidal initialization

- Specify the axis/axes for sinusoidal initialization with the BA command.

- Specify the number of encoder counts per magnetic cycle of the motor with the BM command.

- Issue BI to set an approximate commutation angle based on the current hall state.

- Issue the BC command.

- Enable the axis and issue a low speed jog until a hall transition occurs.

- The motor is now fully initialized for sinusoidal commutation.

#### Operand Usage

_BCm contains the state of the Hall sensor inputs. This value should be between 1 and 6. 0 and 7 are invalid hall states.

Examples

'Galil DMC Code Example REM Example BI/BC initialization procedure BAA;' Configure the A Axis for sinusoidal commutation. BMA=2000 BIA=-1;' Set estimated commutation based on current hall state BCA;' enable brushless calibration hall=_QHA;' store hall state SHA;' enable amplifier JGA=500;' slow jog so that the commutation angle is set precisely at ' the next hall transition. BGA;' begin jog #hall;WT2;JP#hall,_QHA=hall;' wait for a hall transition ' At this point, a precise commutation angle is set ' and the axis is fully configured for sinusoidal commutation STA AMA EN EN

BC applies to DMC4000,DMC4103,DMC30010,DMC50000,EDD37010,DMC1802,DMC1806,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BC

### BD	Degrees

BDm= n

BD n,n,n,n,n,n,n,n

BDm= n

BD n,n,n,n,n,n,n,n

| Usage | BDm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | BD n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _BDm | Operand has special meaning, see Remarks |

Description

The BD command sets the commutation angle of a sinusoidally commutated axis. This command should not be used except when the user is creating a specialized sinusoidal initialization procedure. The commutation angle is set automatically when using Galil's built-in sinusoidal initialization methods - BX, BZ and BI/BC.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 360 | 6 | 1/32 | Commutation angle in degrees |  |

Remarks

Using BD to set a commutation angle overrides the current value set by the BX, BZ, or BI/BC initialization methods.

Once initialized, BD is updated by the controller automatically based on the encoder position and the value of the brushless modulus, BM. n = ? queries the current commutation angle

_BDm contains the commutation angle of the specified axis.

Examples

'Galil DMC Code Example

BDA=100;'     Set the commutation angle for A axis to 100

MG_BDA;'      Report the commutation angle for A axis

BD applies to DMC4000,DMC4103,DMC30010,DMC50000,EDD37010,DMC1802,DMC1806,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BD

INDEPENDENT MOTION

### BG Begin

BG mm

BG mm

| Usage | BG mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _BGm | Operand has special meaning, see Remarks |

Description

The BG command starts a motion on the specified axis or sequence.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to begin motion | Any combination of axes is acceptable. BG with no arguments begins motion on all axes |
|  | S |  | T | N/A | Multi-Axis Mask | Vector plane axes to begin motion | Any combination of axes is acceptable |
|  | M |  | N | N/A | Multi-Axis Mask | Virtual axis to begin motion | Any combination of axes is acceptable |

Remarks

Any combination of Axes, Vector Planes, and Virtual Axes may be mixed to begin motion.

A BG command cannot be executed for any axis in which motion has not completed.

Slaving to a master in gearing mode is an exception. Gearing does not require the axis to profile a motion and therefore Independent moves may be superimposed on top of gearing.

Use the AM trippoint to wait for motion complete between moves from embedded code. From host code, use one of the following methods to determine if motion is complete:

Poll MG_BGm.

Use the data record (DR/QR).

Use interrupts (EI), if available.

#### Operands

_BGm contains a '0' if motion complete on the specified axis or coordinate system, otherwise contains a '1'

_BGm can be used from host programs to determine if motion is complete by polling the axes of interest

Examples

'Galil DMC Code Example

PR 2000,3000,,5000;'  Set up for a relative move

BG ;'                 Start the A,B and D motors moving

'Galil DMC Code Example

HM ;'   Set up for the homing

BG A;'  Start only the A-axis moving

'Galil DMC Code Example

JG 1000,4000;'   Set up for jog

BG B;'           Start only the B-axis moving

'Galil DMC Code Example

bstate= _BGB;'   Assign a 1 to bstate if the B-axis is performing a move

'Galil DMC Code Example

VM AB;'         Vector Mode

VP 1000,2000;'  Specify vector position

VS 20000;'      Specify vector velocity

BG S;'          Begin coordinated sequence

VP 4000,-1000;' Specify vector position

VE;'            Vector End

BG applies to DMC4000,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC4200,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BG

SINE COMMUTATION

### BI Brushless Inputs

BIm= n

BI n,n,n,n,n,n,n,n

BIm= n

BI n,n,n,n,n,n,n,n

| Usage | BIm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | BI n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _BIm | Operand holds the value last set by the command |

Description

The BI command is used along with the BC command to initialize an axis for sinusoidal commutation using hall sensors. When this command is issued, BI sets an approximate commutation angle based on the current hall state. In addition to BI and BC, the BA and BM commands must be used to initialize the axis for sinusoidal commutation.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to initialize using hall sensors. |  |
| n | -1 |  | 0 |  | 0 | 1 |  | n = -1 uses dedicated hall inputs. n = 0 clears configuration. |

Remarks

GDK's Step-By-Step tool can be utilized for automatic setup and configuration of brushless motor with Galil internal sinusoidal amplifiers.

There are several methods to initialize Galil's internal amplifiers for sinusoidal commutation. They are listed below: Initialization of a Galil Sinusoidal Amplifier

| Command | Description | Notes |
| --- | --- | --- |
| BI/BC | Uses hall sensors to estimate the commutation angle until a hall transition can be used to set the commutation angle precisely. | This is the recommended method if hall sensors are present. |
| BX | Uses an algorithm to determine the commutation angle with minimal motion. | Should only be used if minimal motion is required during intialization and no hall sensors are present. |
| BZ | Commands the axis to a specific commutation angle. | This is the recommended method if hall sensors are not present. |

#### Steps for BI/BC sinusoidal initialization

- Specify the axis/axes for sinusoidal initialization with the BA command.

- Specify the number of encoder counts per magnetic cycle of the motor with the BM command.

- Issue BI to set an approximate commutation angle based on the current hall state.

- Issue the BC command.

- Enable the axis and issue a low speed jog until a hall transition occurs.

- The motor is now fully initialized for sinusoidal commutation.

Examples

'Galil DMC Code Example REM Example BI/BC initialization procedure REM Example BI/BC initialization procedure BAA;' Configure the A Axis for sinusoidal commutation. BMA=2000 BIA=-1;' Set estimated commutation based on current hall state BCA;' enable brushless calibration hall=_QHA;' store hall state SHA;' enable amplifier JGA=500;' slow jog so that the commutation angle is set precisely at ' the next hall transition. BGA;' begin jog #hall;WT2;JP#hall,_QHA=hall;' wait for a hall transition ' At this point, a precise commutation angle is set ' and the axis is fully configured for sinusoidal commutation STA AMA EN

BI applies to DMC4000,DMC4103,DMC30010,DMC50000,EDD37010,DMC1802,DMC1806,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BI

PROGRAMMING

### BK Breakpoint

BK n

0

,n

1

BK n

0

,n

1

| Usage | BK n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _BK | Operand has special meaning, see Remarks |

Description

The BK command causes the controller to pause execution of the given thread at the given program line number. When that line is reached, program execution halts before the line is executed, while all other threads continue running. After a breakpoint is encountered, a new breakpoint can be armed (to continue execution to the new breakpoint) or BK will resume program execution. The SL command can be used to single step from the breakpoint.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 |  | 3,999 | N/A | 1 | Line number to set breakpoint | Firmware Rev 1.2a and later. n = null resumes execution |
| n0 | 0 |  | 1,999 | N/A | 1 | Line number to set breakpoint | n = null resumes execution |
| n1 | 0 |  | 7 | 0 | 1 | Thread number to set breakpoint | If n omitted, default value used. |

Remarks

Only one breakpoint may be armed at any time.

BK can be armed before or during thread execution.

#### Operand Usage

_BK will tell whether a breakpoint has been armed, whether it has been encountered, and the program line number of the breakpoint:

= -LineNumber: breakpoint armed

= LineNumber: breakpoint encountered

= -2147483648: breakpoint not armed

Examples

'Galil DMC Code Example

:BK 3;'    Pause at line 3 (the 4th line) in thread 0

:BK 5;'    Continue to line 5

:SL;'      Execute the next line

:SL 3;'    Execute the next 3 lines

:BK;'      Resume normal execution

BK applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BK

ERROR CONTROL

### BL Reverse Software Limit

BLm= n

BL n,n,n,n,n,n,n,n

BLm= n

BL n,n,n,n,n,n,n,n

| Usage | BLm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | BL n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _BLm | Operand holds the value last set by the command |

Description

The BL command sets the reverse software limit. If this limit is exceeded during motion, motion on that axis will decelerate to a stop. Reverse motion beyond this limit is not permitted.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -2,147,483,648 | 2,147,483,647 | -2,147,483,648 | 1 | Position for reverse soft limit |  |

Remarks

The reverse limit is activated at the position n-1. n = -2147483648 effectively disables the reverse soft limit The software limit is specified in counts for a servo system or in microsteps for a stepper system.

When the reverse software limit is activated, the automatic subroutine #LIMSWI will be executed if it is included in the program.

If motion is commanded when the axis is already passed the BL value, the controller will return error code 22. See TC for details.

Examples

'Galil DMC Code Example

#TEST;'  Test Program

AC 1000000;' Acceleration Rate

DC 1000000;' Deceleration Rate

BL -15000;' Set Reverse Limit

JG  -5000;' Jog Reverse

BGA;'    Begin Motion

AMA;'    After Motion (limit occurred)

TPA;'    Tell Position

EN;'     End Program

'

'Galil Controllers also provide hardware limits.

BL applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BL

SINE COMMUTATION

### BM Brushless Modulo

BMm= n

BM n,n,n,n,n,n,n,n

BMm= n

BM n,n,n,n,n,n,n,n

| Usage | BMm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | BM n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _BMm | Operand holds the value last set by the command |

Description

The BM command defines the length of the motors magnetic cycle in encoder counts. This value must be specified correctly for sinusidal commutation. In addition to BM, the BA command as well as either BX, BZ or BI/BC, must be used to initialize the axis for sinusoidal commutation.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value. |  |
| n | 1 |  | 10,000,000 | 2,000 | 1/65,536 | Encoder counts per magnetic cycle. |  |

Remarks

For rotary motors, the magnetic cycle (BM value) is calculated by: BM = encoder counts per revolution / # of pole pairs

When calculating BM, always use the controller to do the math. For example, "BMA=5000/3".

Examples

'Galil DMC Code Example REM Example BZ initialization procedure BA A;' Designate sinusoidal commutation BM 4000;' Length of electrical cycle in counts--required setting for commutation BZ <1000>1500;' Set the first BZ time to 1500 msec, and second B time to 1000 msec. BZ 3;' Commutate motor using 3 V and timeout after 1000 msec SH A;' Enable motor, ready for commands EN

'Galil DMC Code Example REM Example BI/BC initialization procedure BAA;' Configure the A Axis for sinusoidal commutation. BMA=6000;' Length of electrical cycle in counts--required setting for commutation BIA=-1;' Set estimated commutation based on current hall state BCA;' Enable brushless calibration hall=_QHA;' Store hall state SHA;' Enable amplifier JGA=500;' Slow jog so that the commutation angle is set precisely at ' the next hall transition. BGA;' Begin jog #hall;WT2;JP#hall,_QHA=hall;' Wait for a hall transition ' At this point, a precise commutation angle is set ' and the axis is fully configured for sinusoidal commutation STA;' Stop motion on axis A AMA;' Pause code execution until after motion A is complete EN

BM applies to DMC4000,DMC4103,DMC30010,DMC50000,EDD37010,DMC1802,DMC1806,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BM

SYSTEM CONFIG

#### BN   Burn

BN

BN

| Usage | BN | Command takes no arguments |
| --- | --- | --- |
| Operands | _BN | Operand has special meaning, see Remarks |

Description

The BN command saves certain board parameters in non-volatile EEPROM memory. Once written to the memory, all parameters which can be burned will persist through a software reset (RS command), hardware reset (reset button) or power cycle. This command typically takes 1 second to execute and must not be interrupted. The controller returns a colon (:) when the Burn is complete. All parameters which have been burned into memory can be restored to their factory defaults through a master reset.

This command reference will denote comands that can and cannot be burned with BN with the following usage icons.

Burnable with BN icon

Not burnable with BN icon

Arguments

The BN command has no arguments

Remarks

Issuing this command will pause the output of the Data Record until the command is completed.

The following table shows the commands that have their parameters saved with the BN command:

Parameters saved during burn

| AC | BO | EO | IK | MO | OT | TM |
| --- | --- | --- | --- | --- | --- | --- |
| AF | BR | ER | IL | MT | OV | TR |
| AG | BW | FA | IT | MU | PF | VA |
| AQ | CB | FL | KD | NB | PL | VD |
| AU | CE | FV | KI | NF | PW | VF |
| BA | CN | GA | KP | NZ | SB | VS |
| BB | CW | GM | KS | OA | SM | YA |
| BI | DC | GR | LC | OE | SP | YB |
| BL | DH | HV | LD | OF | TK | YC |
| BM | DV | IA | LZ | OP | TL |  |

Operand Usage  _BN contains the serial number of the processor board.

Examples

'Galil DMC Code Example

SB1;'   Set bit 1

CB2;'   Clear bit 2

CW1;'   Set data adjustment bit

BN;' Burn all parameter states

BN applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BN

SYSTEM CONFIG

### BP Burn Program

BP

BP

| Usage | BP | Command takes no arguments |
| --- | --- | --- |

Description

The BP command saves the application program in non-volatile EEPROM memory. This command may take several seconds to execute and must not be interrupted. The controller returns a : when the Burn is complete.

Arguments

The BP command has no arguments

Remarks

Issuing this command will pause the output of the Data Record until the command is completed.

Legacy Software Note: This command may cause the Galil software to issue the following warning "A time-out occurred while waiting for a response from the controller". This warning is normal and is designed to warn the user when the controller does not respond to a command within the timeout period.

The timeout can be changed in the Galil software but this warning does not affect the operation of the controller or software.

Examples

'Galil DMC Code Example

:BP;'    Burn in program to controller

:'       Get colon response when done

BP applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BP

SYSTEM CONFIG

### BR BrushAxis

BRm= n

BR n,n,n,n,n,n,n,n,n

BRm= n

BR n,n,n,n,n,n,n,n,n

| Usage | BRm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | BR n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _BRm | Operand holds the value last set by the command |

Description

The BR command configures the motor configuration and type for an axis.

The BR command is used with internal Galil amplifiers to enable which axes will be set as brush-type servos or to configure the firmware to use external drives instead of the internal channel.

Arguments

| Argument |  | Min |  | Max | Default | Resolution |  | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis |  | Axis to assign value |  |
| Argument | Value |  | Description |  |  |  | Notes |  |  |  |
| n | -1 |  | Configured for external drive |  |  |  | Use for external drives with internal sine amps -D3640, -D3540 and -D3520 |  |  |  |
|  | 0 |  | Configured for Brushless servo |  |  |  | Default |  |  |  |
|  | 1 |  | Configured for Brush-type servo |  |  |  | Use for axes with external drives on -D3040 and -D3020 to avoid hall errors |  |  |  |

Remarks

If an axis has Off-On-Error(OE) set to 1, an amplifier error will occur on an axis if there are no halls and BR is set to 0. Set BR to 1 to avoid an amplifier error state.

The hall error bits cannot cause #AMPERR events if an axis is configured as brush-type.

On axes with an internal amplifier present, when BR1 is set the hall inputs are available for general use via the QH command.

Note: If the controller has been previously configured with the BA command for sinusoidal commutation with a Galil internal amplifier, the command "BA N" must be issued prior to setting the axis to brushed mode.

Examples

'Galil DMC Code Example

BR 1,0,0;'     Sets X-axis to brush-type, Y and Z to brushless

'Galil DMC Code Example

BR 1;'       Set to brush type, ignore hall errors

BR -1;'      Set to external amp

BR applies to DMC50000,DMC4000,DMC4103,DMC30010,DMC2103,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BR

PVT MODE

### BT BeginPVT Motion

BT mm

BT mm

| Usage | BT mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _BTm | Operand has special meaning, see Remarks |

Description

The BT command begins PVT motion on the specified axes. All axes specified will begin at the same time. For more details on PVT mode see the user manual.

Arguments

| Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- |
| A | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to begin PVT motion |  |

Remarks

For more details on PVT mode see the user manual.

_BTm contains the number of PV segments that have executed.

Examples

'Galil DMC Code Example :MG _BTA;' Query number of PVT segments executed 0.0000 :PVA= 100,200,100;' Command X axis to move 100 counts reaching an ending speed of 200c/s in 100 samples :PVA= 100,0,100;' Command X axis to move another 100 counts reaching an ending speed of 0c/s in 100 samples :PVA= ,,0;' Command X axis to exit PVT mode :BT A;' Begin PVT mode :MG _BTA;' Query number of PVT segments executed 3.0000 :

BT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BT

SYSTEM CONFIG

### BV BurnVariables and Array

BV

BV

| Usage | BV | Command takes no arguments |
| --- | --- | --- |
| Operands | _BV | Operand has special meaning, see Remarks |

Description

The BV command saves the controller variables and arrays in non-volatile EEPROM memory. This command typically takes up to 2 seconds to execute and must not be interrupted. The controller returns a : when the Burn is complete.

Arguments

The BV command has no arguments

Remarks

_BV returns the number of controller axes.

This command will store the ECAM table values in non-volatile EEPROM memory.

This command may cause the Galil software to timeout. This warning is normal and is designed to warn the user when the controller does not respond to a command within the timeout period. This occurs because this command takes more time than the default timeout period. The timeout can be changed in the Galil software. This warning does not affect the operation of the board or software.

Issuing this command will pause the output of the Data Record until the command is completed.

Examples

'Galil DMC Code Example

:BV;'  burn in variables

:'     colon response returned

BV applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BV

SYSTEM CONFIG, IO

### BW   Brake Wait

BWm= n

BW n,n,n,n,n,n,n,n

BWm= n

BW n,n,n,n,n,n,n,n

| Usage | BWm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | BW n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _BWm | Operand holds the value last set by the command |

Description

The BW command sets the delay between when the brake is turned on (1) and when the amp is turned off (2). When the controller goes into a motor-off (MO) state, this is the time (in samples) between when the brake digital output changes state and when the amp enable digital output changes state. The brake is actuated immediately upon MO and the delay is to account for the time it takes for the brake to engage mechanically once it is energized electrically. The brake is released immediately upon SH.

#### Brake Wait Timing

Brake Wait Timing Diagram

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 4,096 | 0 | 1 | Specify brake wait time, in samples. | 0 = Turn brake function off |

Remarks

The Brake Wait does not apply when the motor is shut off due to OE1 (Off on Error). In this case (position error exceeded or Abort triggered) the motor off and brake output will be applied simultaneously.

SB,CB and OP have no effect on outputs mapped to BW. In order to toggle brake outputs without engaging the servo (e.g. for maintenance), set BWm=0 and then use SB and CB as necessary.

The state of the output configured as a brake cannot be querried with the @OUT[] command.

Outputs 1-8 are used for Axes A-H, where output 1 is the brake for axis A and output 2 is the brake for axis B and so on.

When using the brake outputs, it is recommended to order the controller with 500mA sourcing output option (HSRC).

Examples

'Galil DMC Code Example

BW 100;' Set brake delay to 100 ms (TM1000) for the A axis

BW applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BW

SINE COMMUTATION

### BX   Sine Amp Initialization

BXm= n

BX< o

BXm= n

BX< o

| Usage | BXm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
| Operands | _BXm | Operand has special meaning, see Remarks |

Description

The BX command uses a method to initialize an axis with limited movement of the motor. It is expected to move no more than 10 degrees of the magnetic cycle. The last stage of the BX command will lock the motor into the nearest 15 degree increment. In addition to BX , the BA and BM commands must be used to initialize the axis for sinusoidal commutation.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to initialize |  |
| n | -4.998 | 4.998 | 0 | 20/65,536 | Torque command voltage to be applied during initialization | -n = end BX with SH. +n = end BX with MO |
| o | 100 | 5,000 | 1,000 | 1 | Number of samples for BX to hold final torque pulse. | See Remarks |

#### Calculating the 'n' parameter

Use equation below to determine the maximum n parameter where Im (in Amps) is the continuous current rating of the motor and G is the current gain of the amplifier (in Amps/Volt).

A conservative starting point for the BX command is 0.5* n max but may be increased up to n max as needed.

Equation for maximum 'n' value

Remarks

GDK's Step-By-Step tool can be utilized for automatic setup and configuration of brushless motor with Galil internal sinusoidal amplifiers.

_BXm contains 0 if axis m is not a sinusoidal axis, contains 1 if axis m is an uninitialized sinusoidal axis, and contains 3 if axis m is an initialized sinusoidal axis

The BX command may be given while the motor is off.

While the BX command is executing, DMC code, data records, and communication from the controller will pause until completion.

BX initialization is only valid with Galil internal sinusoidal amplifiers.

There are several methods to initialize Galil's internal amplifiers for sinusoidal commutation. They are listed below: Initialization of a Galil Sinusoidal Amplifier

| Command | Description | Notes |
| --- | --- | --- |
| BI/BC | Uses hall sensors to estimate the commutation angle until a hall transition can be used to set the commutation angle precisely. | This is the recommended method if hall sensors are present. |
| BZ | Commands the axis to a specific commutation angle. | This is the recommended method if hall sensors are not present. |
| BX | Uses an algorithm to determine the commutation angle with minimal motion. | Should only be used if minimal motion is required during intialization and no hall sensors are present. |

#### BX Initialization Steps

- Specify the axis/axes for sinusoidal initialization with the BA command.

- Specify the number of encoder counts per magnetic cycle of the motor with the BM command.

- Set the desired hold time BX<o.

- Initialize using BX command with n <= n max .

- The motor is now fully initialized for sinusoidal commutation.

Examples

'Galil DMC Code Example

REM Initialize A axis for internal sine commutation.

BA A;'          Configure axis A for sine amp

BMA = 2000;'    Length of electrical cycle in counts

BX <1000;'      Set hold time to 1000 ms

BXA = 3;'       Initialize the motor with 3V torque command.

SH A;'          Enable motor, ready for commands EN

BX applies to DMC50000,DMC4000,DMC4103,DMC30010,EDD37010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

BX

SINE COMMUTATION

### BZ   Brushless Zero

BZm= n

BZ <o>p

BZm= n

BZ <o>p

| Usage | BZm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
| Operands | _BZm | Operand has special meaning, see Remarks |

Description

The BZ command is used to initialize axes which are configured for sinusoidal commutation. To do this, BZ drives the motor to two different magnetic positions and then sets an appropriate commutation angle. In addition to BZ , the BA and BM commands must be used to initialize the axis for sinusoidal commutation.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to initialize. |  |
| n | -4.998 | 4.998 | 0 | 20/65,536 | Torque command voltage to be applied during initialization. | -n = end BZ with SH. +n = end BZ with MO. |
| o | 100 | 32,767 | 200 | 1 | Time in milliseconds for BZ to hold at second magnetic position. | See Remarks. |
| p | 100 | 32,767 | 100 | 1 | Time in milliseconds for BZ to hold at first magnetic position. | See Remarks. |

Remarks

GDK's Step-By-Step tool can be utilized for automatic setup and configuration of brushless motor with Galil internal sinusoidal amplifiers.

The BZ hold times should be lengthened to ensure that any oscillations introduced by the BZ command fully settle in order to accurately set the commutation angle.

The o and p parameters can be interrogated with BZ <? and BZ >? respectively.

The BZ command may be given when the motor is off.

_BZm contains the un-signed distance in encoder counts from the motor's current position to the position of magnetic zero for the specified axis. The value is only vaild after succesfully initializing with BZ.

While the BZ command is executing, DMC code, data records, and communication from the controller will pause until completion.

Use equation below to determine the maximum n paramete where Im (in Amps) is the continuous current rating of the motor and G is the current gain of the amplifier (in Amps/Volt).

A conservative starting point for the BX command is 0.5* n max but may be increased up to n max as needed.

Equation for maximum 'n' value

BZ initialization is only valid with Galil internal sinusoidal amplifiers.

There are several methods to initialize Galil's internal amplifiers for sinusoidal commutation. They are listed below: Initialization of a Galil Sinusoidal Amplifier

| Command | Description | Notes |
| --- | --- | --- |
| BI/BC | Uses hall sensors to estimate the commutation angle until a hall transition can be used to set the commutation angle precisely. | This is the recommended method if hall sensors are present. |
| BZ | Commands the axis to a specific commutation angle. | This is the recommended method if hall sensors are not present. |
| BX | Uses an algorithm to determine the commutation angle with minimal motion. | Should only be used if minimal motion is required during intialization and no hall sensors are present. |

#### BZ Initialization Steps

- Specify the axis/axes for sinusoidal initialization with the BA command.

- Specify the number of encoder counts per magnetic cycle of the motor with the BM command.

- Set the desired hold times BZ<o>p.

- Initialize using BZ command with n <= n max .

- The motor is now fully initialized for sinusoidal commutation.

Examples

'Galil DMC Code Example REM Initialize A axis for internal sine commutation. BA A;' Configure axis A for sine amp BMA = 2000;' Length of electrical cycle in counts BZ <1000>1500;' Set the first BZ time to 1500 msec, and second B time to 1000 msec. BZA = 3;' Initialize the motor with 3V motor command. SH A;' Enable motor, ready for commands EN

#### BZ

BZ applies to DMC4000,DMC4103,DMC30010,DMC50000,EDD37010,DMC1802,DMC1806,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### BZ

VECTOR/LINEAR

##### CA   Coordinate Axes

CA m

CA m

| Usage | CA mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _CAm | Operand has special meaning, see Remarks |

#### Description

The CA command specifies the coordinate system (S or T) which will be used by proceeding vector commands. The following commands apply to the active coordinate system as set by the CA command:

| CR | ES | LE | LI | LM |
| --- | --- | --- | --- | --- |
| TN | VE | VM | VP |  |

#### Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | S |  | T |  | S | Axis | Coordinate plane to specify |  |

#### Remarks

CA ? returns a 0 if the S coordinate system is active and a 1 if the T coordinate system is active.

_CA contains a 0 if the S coordinate system is active and a 1 if the T coordinate system is active.

#### Examples

'Galil DMC Code Example CAT;' Specify T coordinate system VMAB;' Specify vector motion in the A and B plane VST= 10000;' Specify vector speed CR 1000,0,360;' Generate circle with radius of 1000 counts, start at 0 degrees and complete one circle in counterclockwise direction. VE;' End Sequence BGT;' Start motion of T coordinate system

CA applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC50000,DMC52000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CA

IO

#### CB   Clear Bit

CB n

CB n

| Usage | CB n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The CB command clears a particular digital output. The SB and CB (Clear Bit) instructions can be used to control the state of output lines.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 1 | 16 | N/A | 1 | General output bit to be cleared | Max value is 8 for 1-4 axis controllers |
| n | 1,000 | 8,999 | N/A | 1 | Clear Modbus slave bit | See "CB via Modbus Slave" in Remarks |

Remarks

The state of the output can be read with the @OUT[] command

Using CB with a Modbus Slave n = (SlaveAddress*10000) + (HandleNum*1000) + ((Module-1)*4) + (Bitnum-1)

Slave Address is used when the ModBus device has slave devices connected to it and specified as Addresses 0 to 255. Please note that the use of slave devices for modbus are very rare and this number will usually be 0.

HandleNum is the handle specifier where A is 1, B is 2 and so on.

Module is the position of the module in the rack from 1 to 16.

BitNum is the I/O point in the module from 1 to 4

For detailed information on connecting to a Modbus slave, see:

https://www.galil.com/news/dmc-programming-io-control/setting-rio-pocket-plc-or-generic-modbus-slave-extended-io

Examples

'Galil DMC Code Example

#main

SB 5;'     Set digital output 5

SB 1;'     Set digital output 1

CB 5;'     Clear digital output 5

CB 1;'     Clear digital output 1

EN

'Galil DMC Code Example #modbus REM connect to modbus slave at IP address 192.168.42.50 IHH=192,168,42,50<502>2 WT100 SB 8001;'set bit 1 on modbus slave WT 10 CB 8003;'clear bit 3 on modbus slave EN

'Galil DMC Code Example

:SB 18;'     Set digital output 18

:SB 21;'     Set digital output 21

:CB 18;'     Clear digital output 18

:CB 21;'     Clear digital output 21

For detailed information on connecting to a Modbus slave, see: https://www.galil.com/news/dmc-programming-io-control/setting-rio-pocket-plc-or-generic-modbus-slave-extended-io

CB applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CB

### CC	Communications Port 2

CC n

0

,n

1

,n

2

,n

3

CC n

0

,n

1

,n

2

,n

3

| Usage | CC n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The CC command configures baud rate, handshake, mode, and echo for the AUX SERIAL PORT, referred to as Port 2. This command must be given before using the MG, or CI commands with Port 2.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | 9,600 | 19,200 | N/A | 9,600 | Baud rate | 9600 and 19200 are valid baud rates |
| n1 | 0 | 1 | N/A | 1 | Handshake setting | n1=0 turns off handshaking. n1=1 turns handshaking on |
| n2 | 0 | 1 | N/A | 1 | Enable aux serial port | n2=0 disables port. n2=1 enables port |
| n3 | 0 | 1 | N/A | 1 | Echo setting | n3=0 for echo off. n3=1 for echo on |

Remarks

The Aux port is not an interpreted port. It cannot receive Galil commands directly. Instead, use CI, #COMINT, and the P2 operands to handle received data.

Examples

'Galil DMC Code Example

:CC 9600,0,1,0;' 9600 baud, no handshake, enable, echo off.

:'         Typical setting with TERM-P or TERM-H.

CC applies to DMC50000,DMC4000,DMC4200,DMC4103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CC

CONTOUR MODE

### CD Contour Distance

CDm= n

CD n,n,n,n,n,n,n,n=t

CDm= n

CD n,n,n,n,n,n,n,n=t

| Usage | CDm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | CD n ... | Arguments specified with an implicit, comma-separated order |

Description

The CD command specifies the incremental position on contour axes. This command is used only in the Contour Mode (CM). The incremental position will be executed over the time period specified by the command DT (ranging from 2 to 256 servo updates)

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -32,768 | 32,767 | 0 | 1 | Contour position segment | Incremental position move |
| t | 1 | 8 | 0 | 1 | Time override option | t = 1-8 specifies 2^n samples for the given interval. |
|  | 0 | 0 | 0 | 0 | Time override option | t=0 with n=0 disables Contour mode. See Remarks |
|  | -1 | -1 | 0 | 0 | Time override option | Pauses contour buffer at the segment with t=-1. Reissue DT to re-engage contour mode. |

Remarks

The units of the command are in encoder counts.

The = operator can be used to override the global DT time by transmitting the time in a CD with the position data. n=t=0 terminates Contour mode similar to VE or LE for vector mode and linear interpolation mode.

Example. CMBC is terminated with CD 0,0=0.

The user must have a space after CD in order to terminate the Contour Mode correctly.

The command CD0=0 (no space) will assign a variable CD0 the value of 0 rather than terminate Contour mode.

Examples

'Galil DMC Code Example

#contour;'                Program Label

CMAB;'                    Enter Contour Mode

DT 4;'                    Set time interval

CD 1000,2000;'            Specify data

CD 2000,4000;'            Next data

CD 0,0=0;'                End of Contour Buffer

#wait;'                   Wait for all segments to process (buffer to empty)

WT 16,1;'                 wait for 1 DT time segment (2^4)

JP#Wait,(_CM<>511)

EN;'                      End Program

'Galil DMC Code Example

#contour;'                Program Label

CMA;'                    Enter Contour Mode

DT 4;'                    Set time interval

CD 1000;'                Specify data

CD 2000;'               Next data

CD 0=0;'                End of Contour Buffer

#wait;'                   Wait for all segments to process (buffer to empty)

WT 16,1;'                 wait for 1 DT time segment (2^4)

JP#Wait,(_CM<>31)

EN;'                      End Program

CD applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CD

### CE	Encoder

CEm= n

CE n,n,n,n,n,n,n,n

CEm= n

CE n,n,n,n,n,n,n,n

| Usage | CEm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | CE n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _CEm | Operand holds the value last set by the command |

Description

The CE command configures the encoder to quadrature type or pulse and direction type. It also allows inverting the polarity of the encoders which reverses the direction of the feedback. The configuration applies independently to the main axes encoders and the auxiliary encoders.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 15 | 0 | 1 | Encoder configuration setting | n is the sum of 2 integers M and N which configure main and auxiliary encoders. See table below for configuration description. |

Configure Encoder Types. Add value from Column 1 and Column 2 to make n

| Column 1 | Main Encoder Type | Column 2 | Auxiliary Encoder Type |
| --- | --- | --- | --- |
| 0 | Normal quadrature | 0 | Normal quadrature |
| 1 | Normal pulse and direction | 4 | Normal pulse and direction |
| 2 | Reversed quadrature | 8 | Reversed quadrature |
| 3 | Reversed pulse and direction | 12 | Reversed pulse and direction |

For example: n = 10 implies 2 + 8, thus both encoders are reversed quadrature.

Remarks

When using a servo motor, changing the CE type can cause the motor to run away.

When the MT command is configured for a stepper motor, the auxiliary encoder (used to count stepper pulses) will be forced to pulse and direction. When using pulse and direction encoders, the pulse signal is connected to CHA and the direction signal is connected to CHB.

Examples

'Galil DMC Code Example

:CE 0, 3, 6, 2;' Configure encoders

:CE ?,?,?,?;'     Interrogate configuration

0,3,6,2

:V = _CEB;'       Assign configuration to a variable :V = ?  3 :

CE applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CE

### CF	Unsolicited Messages Handle

CF m

CF m

| Usage | CF mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _CF | Operand has special meaning, see Remarks |

Description

The CF command sets the port for unsolicited messages. The CF command directs the controller to send unsolicited responses to the Main or Aux Serial Port (If equipped), or to an Ethernet handle. An unsolicited message is data generated by the controller which is not in response to a command sent by the host.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | S | Handle | Ethernet Handle to assign as unsolicited message port | See Remarks |
|  | I |  | I |  | S | Handle | Set the port that sent the command as the unsolicited message port | Not valid in program |
|  | S |  | T |  | S | Handle | Set serial port as unsolicited message port | m=S is Main serial port. m=T is Aux Serial port |

Remarks

Examples of application code commands that will generate unsolicited messages follow.

'Galil DMC Code Example MG"Hello";' A message (MG) TC1;' A command that returns a response TP;' " RPA;' " var=?;' A variable interogation var=;' " thisIsAnError;' A dmc error will generate an error message

#### Ethernet Handle as Unsolicited Message Port

When communicating over Ethernet, two Ethernet handles should be used:

- The first handle should be used for command-and-response traffic. This is the primary handle that the host uses to communicate to thecontroller.

- The second handle should be used for unsolicited traffic. This is the primary handle that the controller uses to asynchronously communicateto the host. Use CF to point unsolicited traffic to this handle.

It is NOT recommended to use one Ethernet handle for both command-and-response, and unsolicited messages.

#### Operand Usage

_CF contains the decimal value of the ASCII letter where unsolicited messages are currently routed.

Examples

'Galil DMC Code Example

:CFI;' send unsolicited traffic to the terminal that sent the command

'Galil DMC Code Example 'main handle is seperate from the unsolicited handle 'Note the connection indicators IHA and IHB in the following: '192.168.1.3, RIO47102 Rev 1.0c, 1480, IHA IHB :TH CONTROLLER IP ADDRESS 192,168,1,3 ETHERNET ADDRESS 00-50-4C-28-05-C8 IHA TCP PORT 23 TO IP ADDRESS 192,168,1,100 PORT 2420 IHB UDP PORT 60007 TO IP ADDRESS 192,168,1,100 PORT 2421 IHC AVAILABLE IHD AVAILABLE IHE AVAILABLE :WH IHA :'Main handle is A :MG_CF 66.0000 :'Unsolicited handle. 66 is ASCII for "B" :

CF applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CF

### CI	Communication Interrupt

CI n

CI n

| Usage | CI n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The CI command configures program interrupts based on input of characters over the communication port.

The command configures a program interrupt based on characters received on communications port 2, the AUX serial port. An interrupt causes program flow to jump to the #COMINT subroutine. If multiple program threads are used, the #COMINT subroutine runs in thread 0 and the remaining threads continue to run without interruption. The characters received can be accessed via the operands P2CH, P2ST, P2NM, P2CD.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | -1 | Clear interrupt data buffer |  |
|  | 0 | Do not interrupt | Default |
|  | 1 | Interrupt on carriage return |  |
|  | 2 | Interrupt on any character |  |

Remarks

For more, see Operator Data Entry Mode in the user manual.

It is the user's responsiblity to ensure the communication buffer is not filled when in this mode, otherwise the controller will report error code 5 "Input Buffer Full". The buffer on the controller is cleared when either of the two following conditions are met: A carriage return is recieved on the communication port. CI -1 is called.

Examples

'Galil DMC Code Example

:CI 1;'     Interrupt when the  key is received on port 2 :CI 2;'     Interrupt on a single character received on Port 2 :

CI applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### CI

CONTOUR MODE

##### CM Contour Mode

CM mm

CM mm

| Usage | CM mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _CM | Operand has special meaning, see Remarks |

Description

Contour Mode is initiated by the instruction CM. This mode allows the generation of an arbitrary motion trajectory with any of the axes. The CD command specifies the position interval between subsequent contour segments. The DT command specifies the time interval between subsequent contour segments.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | N/A | Multi-Axis Mask | Axes to initialize to Contour mode | Disabled by default |

Remarks

CM? Returns the number of available contour seqments (0 means the buffer is full).

_CM contains the number of available contour segments (0 means the buffer is full).

Issuing the CM command will clear the contour buffer when contour mode is not running.

Examples

'Galil DMC Code Example #cont0;' Define label #cont0 CM ABCD;' Specify Contour Mode Axes ABCD DT 4;' Specify time increment for contour (2^4 servo loops, 16ms at TM1000) CD 200,350,-150,500;' Specify incremental positions on A,B,C and D axes ' A-axis moves 200 counts B-axis moves 350 counts C- ' axis moves -150 counts D-axis moves 500 counts ' CD 100,200,300,400;' Next position data CD 0,0,0,0=0;' Special syntax to terminate Contour mode #Wait;JP#Wait,_CM<>511;' Spin on #Wait label until buffer is empty ' End of Contour Buffer/Sequence EN;' End program ' ' #cont1;' Define label #cont1 CM ABC;' Specify Contour Mode DT 8;' Specify time increment for contour (2^8 servo loops, 256ms at TM1000) CD 100,100,100;' New position data CD 100,100,100;' New position data CD 0,0,0 =-1;' Pause contour buffer set DT to resume CD 100,100,100;' New position data CD 100,100,100;' New position data CD 0,0,0,0=0;' Special syntax to terminate Contour mode #wait2;JP#wait2,_CM<>511;'Spin on #wait2 label until buffer is empty 'End of Contour Buffer/Sequence EN

CM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CM

SYSTEM CONFIG, IO

## CN

CN n

0

,n

1

,n

2

,n

3

,n

4

CN n

0

,n

1

,n

2

,n

3

,n

4

| Usage | CN n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _CN0 _CN1 _CN2 _CN3 _CN4 | Operand holds the value last set by the command |

Description

The CN command configures the polarity of the limit switches, home switches, latch inputs, the selective abort function, and the program termination behavior of the abort input.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n0 | -1 | Limit switches active low | Default |
|  | 1 | Limit switches active high |  |
| n1 | -1 | _HM is 0 when grounded (or active-opto), and 1 when pull-up (non-active opto). Affects direction of travel for HM and FE | Default |
|  | 1 | _HM is 1 when grounded (or active-opto), and 0 when pull-up (non-active opto). Affects direction of travel for HM and FE | See HM and FE commands |
| n2 | -1 | Latch input triggers on falling edge | Default |
|  | 1 | Latch input triggers on rising edge |  |
| n3 | 0 | Inputs 5,6,7,8,13,14,15,16 are configured as general use inputs | Default |
|  | 1 | Configures inputs 5,6,7,8,13,14,15,16 as selective abort inputs for axes A,B,C,D,E,F,G,and H respectively | Will also trigger #POSERR automatic subroutine if program is running. |
| n4 | 0 | Abort input will terminate program execution | Default |
|  | 1 | Abort input will not terminate program execution |  |

Remarks  n0 is useful for testing the operation of the #LIMSWI automatic subroutine. See example below.

Examples

'Galil DMC Code Example

CN 1,1;'   Sets limit and home switches to active high

CN ,, -1;' Sets input latch active low

'Galil DMC Code Example REM n0 is useful for testing the operation of the #LIMSWI automatic subroutine #test CN -1;' Switches are active low JGA= 100 BG A;' Start a slow jog move WT1000 CN 1;' Cause a limit fault by inverting the limit polarity EN ' #LIMSWI;' Automatic sub will automatically launch on limit detection MG "Limit Switch Routine" WT100 CN -1;' Return to correct polarity RE

CN applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CN

FILTER/CONTROL

### CP Dead band within which the motor is shut off (MO)

CPm= n

CP n,n,n,n,n,n,n,n

CPm= n

CP n,n,n,n,n,n,n,n

| Usage | CPm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | CP n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _CPm | Operand holds the value last set by the command |

Description

The CP command sets the deadband within which the motor is shut off. After a move is complete (_BGn = 0) and the absolute value of the position error TE becomes less than the dead band CP, the motor is turned off. SH must be issued before further motion can be commanded. CT can be used to increment the integrator limit to ensure that the motor reaches the dead band.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | -1 |  | 32,767 | -1 | 1 | Set the position deadband for motor off. | n=-1 disables the feature |

Remarks

Valid for -NAN or -CER firmware

Examples

'Galil DMC Code Example :^R^V DMC1842 Rev 1.0n-CM-F :^R^S :ED - #L - MG _RPX, _TEX, _ILX, _MOX - WT100 - JP#L 4 :IL0 :CP100 :KI0.01 :PR1000 :BG :XQ : 0.0000 19.0000 0.0000 0.0000 934.0000 669.0000 0.0000 0.0000 1000.0000 684.0000 2.0406 0.0000 1000.0000 656.0000 4.6783 0.0000 1000.0000 429.0000 7.2166 0.0000 1000.0000 340.0000 9.8544 0.0000 1000.0000 83.0000 9.9982 1.0000 1000.0000 82.0000 9.9982 1.0000

CP applies to CER,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### CP

VECTOR/LINEAR

##### CR Circle

CR n

0

, n

1

, n

2

< o > p

CR n

0

, n

1

, n

2

< o > p

| Usage | CR n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

When using the vector mode (VM), the CR command specifies a 2-dimensional arc segment. The VE command must be used to denote the end of the motion sequence after all CR and VP segments are specified. The BG (Begin Sequence) command is used to start the motion sequence. Parameters for radius, starting angle and traverse angle must all be entered for each CR command.

Arguments

| Argument | Min | Max | Default | Resolution | Description |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 10 | 6,000,000 | N/A | 1 | Radius of circle segment |  |  |  |
| n1 | -32,000 | 32,000 | N/A | 1/65,536 | Starting angle of circle segment |  |  |  |
| n2 | -32,000 | 32,000 | N/A | 1/65,536 | Degrees to traverse for circle segment |  |  |  |
| o | 2 | 15,000,000 | N/A | 2 | Specifies the vector speed to be commanded at the beginning of the segment. The controller will start accelerating or decelerating at the start of the sequence to this speed. |  |  | For MT 1,-1 |
|  | 2 | 3,000,000 | N/A | 2 | Specifies the vector speed to be commanded at the beginning of the segment. The controller will start accelerating or decelerating at the start of the sequence to this speed. |  |  | For MT 2,2,2.5,-2.5 |
| p | 2 | 15,000,000 | N/A | 2 | Specifies the vector speed to be achieved at the end of the segment. The controller will decelerate or accelerate during the segment and will reach the specified speed at the end of the segment. |  |  | For MT 1,-1 |
|  | 2 | 3,000,000 | N/A | 2 | Specifies the vector speed to be achieved at the end of the segment. The controller will decelerate or accelerate during the segment and will reach the specified speed at the end of the segment. |  |  | For MT 2,2,2.5,-2.5 |
| Argument | Value | Description |  |  |  | Notes |
| o | -1 | Specifies vector speed to be set by Vector Speed Variable (VV command) |  |  |  | See VV command |

Remarks

The product of n0 * n2 must be less than 450,000,000

A positive n2 denotes counterclockwise traverse, -n2 denotes clockwise.

n0 units are in quadrature counts. n1 and n2 have units of degrees.

Examples

'Galil DMC Code Example

'A starting position of zero degrees denotes that the radius lies along 'a vector following the positive X axis, on a 2D Cartesian space:

VMXY

CR 1000,0,270

VE

BGS

EN

'The 2-d map out the position output can be seen below

#### CR

'Galil DMC Code Example

VMAB;'                Specify vector motion in the A and B plane

VS 1000;'             Specify vector speed

CR 1000,0,360;'       Generate circle with radius of 1000 counts, start at

'                     0 degrees and complete one circle in counterclockwise

'                     direction.

CR 1000,0,360 < 40000;' Generate circle with radius of 1000 counts, start

'                at 0 degrees and complete one circle in counterclockwise

'                direction and use a vector speed of 40000.

VE;'             End Sequence

BGS;'            Start motion

'Galil DMC Code Example 'Generate a sine wave output on the A axis VMAN;' Specify vector motion in the A and N plane VS 1000;' Specify vector speed CR 1000,0,360;' Generate sine wave with amplitude of 1000 counts ' start at 0 degrees and complete one cycle CR 1000,0,360<40000;' Generate same sine wave with same amplitude ' but run at faster speed (higher frequency) ' VE;' End Sequence BGS;' Start motion

CR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CR

VECTOR/LINEAR

### CS Clear Sequence

CS mm

CS mm

| Usage | CS mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _CSm | Operand has special meaning, see Remarks |

Description

The CS command will remove VP, CR or LI commands stored in a motion sequence for a coordinated axis. After a sequence has been executed, the CS command is not necessary to put in a new sequence. This command is useful when you have incorrectly specified VP, CR or LI commands.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | S |  | T |  | N/A | Axis | Coordinate plane specified to clear buffer |  |

Remarks

_CSm contains the segment number in the sequence specified by m= S or T. This operand is valid in the Linear mode, LM, and Vector mode, VM.

Examples

'Galil DMC Code Example

#CLEAR;' Label

CAT;' Specify the T coordinate system vector points

VP 1000,2000;' Vector Position

VP 4000,8000;' Vector Position

CST;' Clear vectors specified in T coordinate system

CAS;' Specify the T coordinate system vector points

VP 1000,5000;' New vector

VP 8000,9000;' New vector

CSS;' Clear vectors specified in S coordinate system

CS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

CS

SYSTEM CONFIG

### CW   Copyright information and Data Adjustment bit on/off

CW n

0

,n

1

CW n

0

,n

1

| Usage | CW n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The CW command will return the copyright information when the argument, n, is 0 or is omitted. Otherwise, the CW command is used as a

communications enhancement for use by the Galil terminal software programs. When turned on, the most significant bit of unsolicited ASCII characters is set to 1. Unsolicited ASCII characters are characters that are returned from a program running on the controller (usually from the MG command). This command does not affect solicited characters, which are characters that are returned as a response to a command sent from a host PC (e.g. TP).

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n0 | 0 | Causes controller to return a copyright information string | Equivalent to n0 = ? |
|  | 1 | Controller will set the MSB of unsolicited message characters |  |
|  | 2 | Controller will not set the MSB of unsolicited message characters | Default. Must be set when viewing unsolicited messages from non-Galil software |
| n1 | 0 | Controller will set error code 131 when hardware handshaking disables character transmissions | Serial timeout will cause long code execution time. |
|  | 1 | Controller will not set error code 131 when hardware handshaking disables character transmissions | Default. Data will be lost if the receiving hardware doesn't service the hardware handshaking lines. |

Remarks

Galil software packages automatically sends CW 1 during connection to a controller.

If reading unsolicited data through a non-Galil software (eg. Hyperterminal), issue CW 2

#### Operand Usage

_CW contains the value set for n0

_CW4 contains the value set for n1

Examples

'Galil DMC Code Example

CW1;'     Set CW to Galil Driver mode (MSB set on unsolicited characters)

'         The CW command can cause garbled (non-ASCII) characters to be returned

'         by the controller when using third-party software. Use CW2.

CW2;'     Set CW to third-party device mode (normal ASCII on unsoliticed characters)

CW applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,RIO47000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### CW

PROGRAMMING DA Deallocate Variables and Arrays

DA str,str,...

| Usage | DA n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _DAm | Operand has special meaning, see Remarks |

Description

The DA command frees the array and/or variable memory space. In this command, more than one array or variable can be specified for memory deallocation. Different arrays and variables are separated by comma when specified in one command.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 8 chars | N/A | String | Array name to deallocate | If str = *, deallocate all arrays |
|  | 1 char | 8 chars | N/A | String | Variable name to deallocate | If str = *, deallocate all variables |

where

str[] - Defined array name str - Defined variable name str = * deallocates all the variables str = *[] - Deallocates all the arrays

Remarks

_DA contains the total number of arrays available.

DA ? returns the total number of arrays available.

Since this command deallocates the spaces and compacts the array spaces in the memory it is possible that execution of this command may take longer time than a standard command.

Variables and arrays that are deallocated are not set to zero. A routine that writes zeros to the array and/or variables should be created if this is desired.

Examples

'Galil DMC Code Example

'Cars' and 'Salesmen' are arrays, and 'Total' is a variable.

DM Cars[40],Salesmen[50];'        Dimension 2 arrays

Total=70;'                        Assign 70 to the variable Total

DA Cars[0],Salesmen[0],Total;'    Deallocate the 2 arrays & variable

DA*[0];'                          Deallocate all arrays

DA *,*[0];'                       Deallocate all variables and all arrays

DA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DA

FILTER/CONTROL

### DB Range in which PID and antifriction bias are turned on (on band)

DBm= n

DB n,n,n,n,n,n,n,n

DBm= n

DB n,n,n,n,n,n,n,n

| Usage | DBm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | DB n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _DBm | Operand holds the value last set by the command |

Description

When the controller is holding position, and the absolute value of the error TE is greater than DB, the PID control loop, as well as anti-friction biases ZP and ZN, will be enabled. This is used in combination with the DS command to avoid oscillation when holding position. DS and DB are disabled during motion, and only apply when holding position.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 32,767 | 0 | 1 | Position setpoint for PID deadband |  |

Remarks

DB should be set greater than or equal to DS.

Examples

'Galil DMC Code Example

DSA=100;'   set off band on A axis to +/-100 counts

DBA=200;'   set on band on A axis to +/-200 counts

DB applies to CER,CLS,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### DB

INDEPENDENT MOTION DC    Deceleration

### DC m= n DC n,n,n,n,n,n,n,n

| Usage | DC m= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | DC n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _DC m | Operand holds the value last set by the command |

Description

The Deceleration command (DC) sets the linear deceleration of the motors for independent moves such as PR, PA, and JG moves. The parameters will be rounded down to the nearest factor of 1024 and have units of counts per second squared.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
|  | M | N | N/A | Axis | Virtual axis to assign value |  |
| n | 1,024 | 1,073,740,800 | 256,000 | 1,024 | Deceleration rate | At TM 1000. See Remarks for resolution details. |

Remarks

The AC command is used to designate acceleration

Specify realistic deceleration rates based on physical system parameters such as:

motor torque rating loads

amplifier current rating

Specifying an excessive deceleration will cause a large following error during deceleration and the motor will not follow the commanded profile DC may be changed during a move in Jog mode, but not in a PA or PR move

However, directly following an axis stop (i.e. ST m or a limit switch), the DC value of a PA or PR move may be changed while the axis is still decelerating

#### Resolution

The resolution of the DC command is dependent on the sampling period of the control loop (TM). With the default rate of TM 1000 the resolution is 1024 counts/second^2. The equation to calculate the resolution of the DC command is:

resolution = min = 1024*(1000/TM)^2 Example:

With TM 500 the minimum DC setting and resolution is 4096 counts/second^2. resolution = 1024*(1000/500)^2 = 4096

Examples

'Galil DMC Code Example

PR 10000;' Specify position

AC 2000000;' Specify acceleration rate

DC 1000000;' Specify deceleration rate

SP 5000;' Specify slew speed

BG;'     Begin motion

DC applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DC

SYSTEM CONFIG

#### DE Dual (Auxiliary) Encoder Position

DEm= n

DE n,n,n,n,n,n,n,n

DEm= n

DE n,n,n,n,n,n,n,n

| Usage | DEm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | DE n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _DEm | Operand has special meaning, see Remarks |

Description

The DE command defines the position of the auxiliary (dual) encoders.

Dual encoders are useful when you need an encoder on the motor and on the load. The encoder on the load is typically the auxiliary encoder and is used to verify the true load position. Any error in load position is used to correct the motor position.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -2,147,483,648 | 2,147,483,647 | 0 | 1 | Position set for auxiliary encoders | For MT 1,-1,1.5,-1.5 |
|  | -2,147,483,648 | 2,147,483,647 | 0 | 1 | Position set for main encoders | For MT 2,-2,2.5,-2.5 |

Remarks

When using stepper motors, the DE command defines the main encoder position.

The auxiliary encoders are not available for the stepper axis or for any axis where output compare is active.

The operand _DEm, as well as _TDm, holds the current aux encoder position. n=? will return the encoder position, as returned by TD.

Examples

'Galil DMC Code Example

DE 0,100,200,400;' Set the current auxiliary encoder position to 0,100,200,400 on A,B,C and D axes DE ?,?,?,?;'      Return auxiliary encoder positions duala= _DEA;'     Assign auxiliary encoder position of A-axis to the variable duala

DE applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DE

SYSTEM CONFIG

#### DF Dual Feedback (DV feedback swap)

DFm= n

DF n,n,n,n,n,n,n,n

DFm= n

DF n,n,n,n,n,n,n,n

| Usage | DFm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | DF n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _DFm | Operand holds the value last set by the command |

Description

The DF command allows configuration of BiSS or SSI feedback in Dual Loop mode as the load encoder. Issuing the DF command will swap the main and auxiliary position registers, such that the encoder wired into the main encoder terminals will report its position in TD and the encoder wired into the auxiliary encoder terminals will report in TP.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Disable feedback swap | Default |
|  | 1 | Enable feedback swap |  |

Remarks

When using this command, wire the motor's incremental encoder into the main encoder terminals. The load encoder should be wired to the auxiliary encoder terminals.

Once wired, configure the serial encoder as an auxiliary encoder. See SI or SS for configuration information.

Examples

'Galil DMC Code Example

MOA;'                 Disable motor on X

SIA= 2,25,0,0<9>1;'   Setup SSI encoder to fill the Aux encoder register

DF1;'                 Enable Dual Feedback Swap

DV1;'                 Enable Dual Loop mode

SHA;'                 Enable servo with new configuration

DF applies to DMC50000,SER

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DF

ETHERNET

#### DH DHCP Client Enable

DH n

DH n

| Usage | DH n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The DH command configures the DHCP or BOOT-P functionality on the controller for Server IP addressing.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Enable BOOT-P and disable DHCP | Allows IP assignment through IA command. |
|  | 1 | Disable BOOT-P and enable DHCP | Default. Allows IP assignment through DHCP server. |

Remarks

DH 0 must be set to manually assign and burn in an IP address. With DH 1 set, the IA command will return an error if used to set the IP address.

Related Commands

IA - IP Address

IH - Open IP Handle

TH - Tell Ethernet Handle

WH - Which Handle

Examples

'Galil DMC Code Example

DH 1;' Sets the DHCP function on.  IA assignment will no longer work.

DH 0;' Sets the DHCP function off, and the Boot-P function on.

DH applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

##### DH

PROGRAMMING

###### DL Download

DL n

DL #str

DL s

DL n

DL #str

DL s

| Usage | DL n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The DL command transfers a data file from the host computer to the controller. Instructions in the file will be accepted as a data stream without line numbers. The file is terminated using <control> Z, <control> Q, <control> D, or \.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 0 | 3,999 | 0 | 1 | Line number to begin program download | Firmware Rev 1.2a and later |
| n | 0 | 1,999 | 0 | 1 | Line number to begin program download |  |
| str | 1 char | 8 chars | "" | String | Name of label in RAM to begin download from. | If str = "", download begins at the end of the current program in RAM |
| s | # | # | N/A | Symbol | Begins download at end of program in RAM |  |

Remarks

Do not insert spaces before label declarations.

_DL gives the number of available labels.

Issuing this command will pause the output of the Data Record until the command is completed.

This command will be rejected by Galil software if sent via the terminal. In order to download a program using a Galil software package, use that package's prescribed programming interface (I.E. GDK's Editor Tool).

Examples

'Galil DMC Code Example

:DL;'      Begin Download

#A;PR 4000;BGA

AMA;MG DONE

EN

\

:'End download

DL applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DL

PROGRAMMING

##### DM   Dimension Array

DM str[n],str[n],...

| Usage | DM n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

#### Description

The DM command defines a single-dimensional array with a name and n total elements. The first element of the defined array starts with element number 0 and the last element is at n-1.

#### Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 8 chars | N/A | String | Name of array to dimension |  |
| n | 1 | 24,000 | N/A | 1 | Number of array elements to assign to dimensioned array | Firmware Rev 1.2a and later |
| n | 1 | 16,000 | N/A | 1 | Number of array elements to assign to dimensioned array |  |

#### Remarks

Typing in array name with [-1] element marked reports the number of elements for that array.

The first character of str must be alphabetic. The rest can be any alphanumeric characters.

When assigning array elements, the number specified must be less than the current available array space _DM contains the available array space.

DM ? returns the available array space.

The DM command can allocate any number of array in a single command up to the maximum command line length of the controller being used.

#### Examples

'Galil DMC Code Example DM Pets[5],Dogs[2],Cats[3];' Define dimension of arrays, Pets with 5 elements, Dogs with 2 elements, Cats with 3 elements DM Tests[100];' Define dimension of array Tests with 100 elements

'Galil DMC Code Example :DM? 16000 :DM MyArray[1000] :DM? 15000 :'DMC-4000 and 30010 provide length of array with array[-1] :MG "MyArray contains",MyArray[-1]," elements" MyArray contains 1000.0000 elements :

DM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

##### DM

SYSTEM CONFIG

###### DP   Define Position

DPm= n

DP n,n,n,n,n,n,n,n

DPm= n

DP n,n,n,n,n,n,n,n

| Usage | DPm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | DP n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _DPm | Operand has special meaning, see Remarks |

##### Description

The DP command sets the current motor position and current command positions to a user specified value. The units are in quadrature counts. This command will set both the TP and RP values.

##### Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
|  | M | N | N/A | Axis | Virtual axis to assign value |  |
| n | - 2,147,483,648 | 2,147,483,647 | 0 | 1 | Value assigned to motor/commanded position (RP and TP registers) | For MT 1,1,1.5,-1.5 |
|  | - 2,147,483,648 | 2,147,483,647 | 0 | 1 | Value assigned to step/commanded position (RP and TD registers) | For MT 2,2,2.5,-2.5 |

##### Remarks

The DP command sets the commanded reference position for axes configured as steppers. The units are in steps.

Example: "DP 0" This will set the registers for TD and RP to zero, but will not effect the TP register value. When equipped with an encoder, use the DE command to set the encoder position for stepper mode.

The DP command is useful to redefine the absolute position.

For example, you can manually position the motor by hand using the Motor Off command, MO. Turn the servo motors back on with SH and then use DP0 to redefine the new position as your absolute zero. The operand _DPm, as well as _TPm, holds the current main encoder position. n=? will return the encoder position, as returned by TP.

##### Examples

'Galil DMC Code Example :DP 0,100,200,400;' Sets the current position of the A-axis to 0, the B-axis to 100, the C-axis to 200, and the D-axis to 400 :DP ,-50000;' Sets the current position of B-axis to -50000. The A, C and D axes remain unchanged. :DP ?,?,?,?;' Interrogate the position of A, B, C and D axis. 0, -50000, 200, 400 :DP ?;' Interrogate the position of A axis 0 :

'Galil DMC Code Example :DP 0;' Sets the current position of the A-axis to 0 :DP -50000;' Sets the current position of A-axis to -50000. :DP ?;' Interrogate the position of A -50000

DP applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DP

SYSTEM CONFIG

##### DR Configures I O Data Record Update Rate

DR n

0

, n

1

DR n

0

, n

1

| Usage | DR n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _DR0 _DR1 | Operand has special meaning, see Remarks |

Description

DR specifies and enables the rate for the controller to output its data record.

For ethernet-based controllers, the controller creates a QR record and sends it to the unsolicited UDP Ethernet Handle at the specified rate. See the User Manual for the data record map.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 8 |  | 30,000 | 0 | 2 | Data update rate specified in samples between packets. |  |
|  | 0 |  | 0 | 0 | 0 | Turn off data record output |  |
| n1 | 0 |  | 7 | see Notes | 1 | Ethernet handle to output data record packet | 0=A,1=B,2=C,3=D,4=E,5=F,6=G,7=H. |

Remarks

If a small sample period and a small update rate is used, the controller may become noticeably slower as a result of maintaining a high update rate.

If n1 is omitted, then the CF unsolicited message port is used by default.

The DR port specified with n1 must be a UDP handle.

_DR0 contains the data record update rate (n0).

_DR1 contains the specified handle (n1). Will return an integer 0-7 for handles A-H.

Issuing any of the following commands will pause the output of the data record until the command is complete: BN, BP, BV, BX, BZ, DL, LS, LV, QD, QU, UL

Examples

'Galil DMC Code Example

:WH

IHA

:DR1000,0

G​x​~​P​

_​`​@~​P​

_​H​`~​P​

_​0​~​P​

:DR0

'Note:  The data record is in a binary, non-printable format

'(the output above is normal when printing to the terminal)

DR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DR

FILTER/CONTROL

##### DS Range in which PID and antifriction bias are turned off (off band)

DSm= n

DS n,n,n,n,n,n,n,n

DSm= n

DS n,n,n,n,n,n,n,n

| Usage | DSm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | DS n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _DSm | Operand holds the value last set by the command |

Description

When the controller is holding position, and the absolute value of the error TE is less than DS, the PID control loop, as well as anti-friction biases ZP and ZN, will be disabled. This is used in combination with DB to avoid oscillation when holding position. DS and DB are disabled during motion, and only apply when holding position.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 32,767 | 0 | 1 | Position setpoint for PID deadband |  |

Remarks

DB should be set greater than or equal to DS.

Examples

'Galil DMC Code Example

DSA=100;'   set off band on the A axis to +/-100 counts

DBA=200;'   set on band on the A axis to +/-200 counts

DS applies to CER,CLS,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DS

CONTOUR MODE

##### DT Delta Time

DT n

DT n

| Usage | DT n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The DT command sets the time interval for Contour Mode. The time interval is 2N samples. With TM 1000, there are 1024 samples per second. Sending the DT command once will set the time interval for all contour data until a new DT command (or CDm=n) is sent.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n | 1 |  | 8 | 0 | 1 | Set time interval for contour mode in 2^n samples. |  |
|  | -1 |  | -1 | N/A | 0 | n=-1 to pause the contour mode | See Remarks. |

Remarks

By default the sample period is 1 msec (set by the TM command); with n=1, the time interval would be 2 msec

n = -1 allows a pre-load of the contour buffer or to asynchronously pause the contour buffer. DT-1 during contour mode will pause the contour buffer (and commanded movement).

A positive DT will resume contour mode from paused position of buffer. DT can be overridden with the =t parameter within a CD segment.

Examples

'Galil DMC Code Example :DT 4;' Specifies time interval to be 16 msec (TM1000) :DT 7;' Specifies time interval to be 128 msec :

'Galil DMC Code Example

REM basic contour example

#Cont0;'                Define label #Cont0

CM ABCD;'               Specify Contour Mode

DT 4;'                  Specify time increment for contour

CD 200,350,-150,500;'   Specify incremental positions on A,B,C and C axes

'                       A-axis moves 200 counts B-axis moves 350 counts C-

'                       axis moves -150 counts C-axis moves 500 counts

CD 100,200,300,400 ;'   New position data

CD 0,0,0,0=0;'          End of Contour Buffer/Sequence

#Wait;'                 Wait for all segments to process (buffer to empty)

WT 16,1;'               wait for 1 DT time segment (2^4)

JP#Wait,(_CM<>511)

EN;'                    End program

'Galil DMC Code Example

REM contour example for pre-loading of contour buffer

#Cont1;'                Define label #Cont1

CM AB;'                 Specify Contour Mode

DT -1;'                 Pause Contour Mode to allow pre-load of buffer

CD 100,200;'            Countour Data pre-loaded in buffer

CD 400,200;'            Countour Data pre-loaded in buffer

CD 200,100;'            Countour Data pre-loaded in buffer

CD 300,50;'             Countour Data pre-loaded in buffer

AI -1;'                 Wait for Analog input 1 to go low

DT 8;'                  Set positive DT to start contour mode

CD 0,0,0,0=0;'          End of Contour Buffer/Sequence

#Wait;'                 Wait for all segments to process (buffer to empty)

WT 16,1;'               wait for 1 DT time segment (2^4)

JP#Wait,(_CM<>511)

EN;'                    End program

DT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DT

FILTER/CONTROL

##### DV Dual Velocity (Dual Loop)

DVm= n

DV n,n,n,n,n,n,n,n

DVm= n

DV n,n,n,n,n,n,n,n

| Usage | DVm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | DV n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _DVm | Operand holds the value last set by the command |

Description

The DV function changes the operation of the PID filter. When DV is enabled the KD (derivative) term operates on the auxiliary encoder instead of the main encoder.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 1 |  | 0 | 1 | State of dual loop mode | n = 0 disables Dual loop. n = 1 enables Dual loop |

Remarks

The DV command is useful in backlash and resonance compensation.

DV must be set properly for commutation to be successful with internal sine drives.

When DVm=0, the controller will use the main encoder for sine drive commutation. When DVm=1, the controller will use the aux encoder for sine drive commutation.

###### Correcting for Positive Feedback

With motor off (MO) check the motor encoder with TD and load encoder with TP. Manually move the motor/load and reissue the TD and TP commands to confirm both encoders count in the same direction.

If the encoders count in opposing directions, change the polarity of one encoder using the CE command or by changing the wiring. Consult user manual.

If positive feedback still persists, switch the motor polarity or reverse the direction of both encoders.

Off on error (OE) and error limits (ER) can be used to shut down the motor in the event of a runaway.

###### Using DV with Large motor/load encoder ratio

When using Dual Loop mode with a large motor:load ratio and/or running at high velocities where low position error at speed is required, FV should be used to compensate for the derivative contribution from the higher resolution motor encoder.

The estimated FV setting required to compensate for the derivative contribution can be calculated by the equation: FV = (KD/4)*(motor/load)

motor/load = effective motor to load ratio

For example: KD = 200, motor encoder changes 5000 counts per 1000 counts of load encoder (motor/load = 5/1) FV = (200/4)*(5/1) = 250

Examples

'Galil DMC Code Example

DV 1,1,1,1;' Enables dual loop on all axes

DV 0;' Disables DV on A axis

DV,,1,1;' Enables dual loop on C axis and D axis.  Other axes remain unchanged.

DV 1,0,1,0;' Enables dual loop on A and C axis.  Disables dual loop on B and D axis.

MG_DVA;' Returns state of dual velocity mode for A axis

DV applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

DV

##### EA Choose ECAM master

EA m

EA m

| Usage | EA mm | Argument is an axis mask |
| --- | --- | --- |

Description

The EA command selects the master axis for the electronic cam mode. Any axis may be chosen.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign as ECAM master |  |
|  | M |  | N |  | N/A | Axis | Virtual axis to assign as ECAM master | N is default |

Remarks

The ECAM mode runs off of the master's main encoder (TP) even when the axis is running in stepper mode. When using the M or N imaginary axes, the commanded position is used.

Examples

'Galil DMC Code Example REM example using A axis as ECAM master and B axis as ECAM slave #CAMONE master=400 slave=8192 EB0; 'Disable ECAM Mode EA A; 'Set Master Axis as A EM master,slave EP master/4,0 ET[0]=,0 ET[1]=,2048 ET[2]=,4096 ET[3]=,6144 ET[4]=,8192 DP0,0 SHAB 'NOTE - (EP Value)*(# of Cam Points) must be >= to Master Modulus JG100;BGA EB1 EG,0; 'Start ECAM profile EN

EA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EA

##### EB Enable ECAM

EB n

EB n

| Usage | EB n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _EB | Operand has special meaning, see Remarks |

Description

The EB function enables or disables the cam mode. In this mode, the starting position of the master axis is specified within the cycle.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Stop ECAM mode | Default |
|  | 1 | Start ECAM mode |  |

Remarks

When the EB command is given, the master axis position is modularized. _EB holds the enabled state, 1 or 0

Examples

'Galil DMC Code Example EB1;' Starts ECAM mode EB0;' Stops ECAM mode var = _EB;' Return status of cam mode

EB applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EB

##### EC ECAM Counter

EC n

EC n

| Usage | EC n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _EC | Operand has special meaning, see Remarks |

Description

The EC function sets the index into the ECAM table. This command is only useful when entering ECAM table values without index values. See the command, ET.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  | 256 | 0 | 1 | Set the ECAM table index |  |

Remarks

_EC contains the current value of the index into the ECAM table.

Examples

'Galil DMC Code Example

EC0;'  Set ECAM index to 0

ET 200,400;' Set first ECAM table entries to 200,400 ET 400,800;' Set second ECAM table entries to 400,800 var=_EC;'    Set the ECAM index value to a variable

EC applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EC

PROGRAMMING

##### ED Edit

ED n

ED n

| Usage | ED n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _ED _ED1 _ED4 | Operand has special meaning, see Remarks |

Description

The ED command puts the controller into the Edit subsystem. The ED command is used when using Telnet style interface (not Galil Software). In the Edit subsystem, programs can be created, changed, or destroyed.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  | 3,999 | see Notes | 1 | Line number to begin editing | Firmware Rev 1.2a and later. Default n is the last line of program space with commands. |
| n | 0 |  | 1,999 | see Notes | 1 | Line number to begin editing | Default n is the last line of program space with commands. |

Remarks

This command will be rejected by Galil software if sent to via the terminal. In order to edit a program using a Galil software package, use that package's prescribed programming interface (I.E. GDK's Editor Tool). The commands in the Edit subsystem are the following.

ED Commands

| Key Combination | Function |
| --- | --- |
| <ctrl>D | Deletes a Line |
| <ctrl>I | Inserts a line before the current |
| <ctrl>P | Displays the previous line |
| <ctrl>Q | Exits the ED subsystem |
| Enter | Saves a line and moves cursor to next |

###### Operand Usage

_ED0 contains the line number of the last line to have an error.

_ED1 contains the number of the thread where the error occurred (for multitasking).

_ED0 returns 0 if no error has occurred.

_ED1 returns -1 if no error has occurred.

_ED4 when evaluated in an embedded code thread, this operand will contain the thread id of the calling thread. This is useful for DMC code to determine which thread it is running in. See example below.

Examples

'Galil DMC Code Example :ED #START PR 2000 BGA xx;' bad command line EN #CMDERR Routine which occurs upon a command error V=_ED0 MG "An error has occurred" {n} MG "In line", V{F3.0} ST ZS0 EN ctrl-Q :'Hint: Remember to quit the Edit Mode prior to executing or listing a program.

ED applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ED

ECAM/GEARING

##### EG ECAM go (engage)

EGm= n

EG n,n,n,n,n,n,n,n

EGm= n

EG n,n,n,n,n,n,n,n

| Usage | EGm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | EG n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _EGm | Operand has special meaning, see Remarks |

Description

The EG command engages an ECAM slave axis at a specified position of the master. Once a slave motor is engaged, its position is redefined to fit within the cycle.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | - 2,147,483,648 | 2,147,483,647 | 0 | 1 | Master position to engage ECAM slave | n = outside of master axis position range causes slave to engage immediately. |

Remarks

_EGm contains ECAM status for specified slave axis. 0 = axis is not engaged, 1 = axis is engaged. n = ? Returns 1 if specified axis is engaged and 0 if disengaged.

This command is not a trippoint. This command will not hold the execution of the program flow. If the execution needs to be held until master position is reached, use MF or MR command.

Examples

'Galil DMC Code Example

EG 700,1300;' Engages the A and B axes at the master position 700 and 1300 respectively.

B = _EGB;' Return the status of B axis, 1 if engaged

EG applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EG

SYSTEM CONFIG

##### EI   Event Interrupts

EI n

1

,n

2

,n

3

EI n

1

,n

2

,n

3

| Usage | EI n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _EI | Operand has special meaning, see Remarks |

Description

The EI command is used to enable interrupts on events. EI enables interrupts for the predefined event conditions in the table below. When a condition (e.g. Axis A profiled motion complete) occurs after EI is armed, a particular status byte value (e.g. $D0 or 208) is delivered to the host PC along with the interrupt.

Interrupts are issued as automatically dispatched UDP packets.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 |  | 65,535 | 0 | 1 | 16-bit interrupt mask | 0 turns off interrupts. See Remarks for bit mask |
| n2 | 0 |  | 255 | 0 | 1 | 8-bit input mask | Used to select the specific digital input trigger. Bit 15 of n1 must be set for the n2 mask to be used. |
| n3 | -1 |  | 7 | -1 | 1 | Preconfigured UDP handle for interrupt transmission | -1 disabled, 0-7 indicate Handles A-H, respectively |

Remarks

_EI contains the interrupt mask n1

n1 = 0 means "don't interrupt" and clears the queue when issued

The interrupts marked with * in the table below must be re-enabled with EI after each occurrence Bit 15 of n1 must be set for the n2 input mask to be used

If the handle specified by n3 is not UDP or is not initialized, an error will occur

###### n1 Bit Mask

Interrupt Bits

| bit | n1=2^bit Hex (decimal) | Status Byte Hex (decimal) | Condition |
| --- | --- | --- | --- |
| 0 | $0001 (1) | $D0 (208) | Axis A profiled motion complete _BGA = 0 |
| 1 | $0002 (2) | $D1 (209) | Axis B profiled motion complete _BGB = 0 |
| 2 | $0004 (4) | $D2 (210) | Axis C profiled motion complete _BGC = 0 |
| 3 | $0008 (8) | $D3 (211) | Axis D profiled motion complete _BGD = 0 |
| 4 | $0010 (16) | $D4 (212) | Axis E profiled motion complete _BGE = 0 |
| 5 | $0020 (32) | $D5 (213) | Axis F profiled motion complete _BGF = 0 |
| 6 | $0040 (64) | $D6 (214) | Axis G profiled motion complete _BGG = 0 |
| 7 | $0080 (128) | $D7 (215) | Axis H profiled motion complete _BGH = 0 |
| 8 | $0100 (256) | $D8 (216) | All axes profiled motion complete |
| 9 | $0200 (512) | $C8 (200) | * Excess position error _TEm >= _ERm |
| 10 | $0400 (1024) | $C0 (192) | * Limit switch _LFm=0 /_LRm=0 Must be profiling motion in direction of activated limit switch for interrupt to occur. |
| 11 | $0800 (2048) | $D9 (217) | Reserved |
| 12 | $1000 (4096) |  | Reserved |
| 13 | $2000 (8192) | $DB (219) | Application program stopped _XQn = -1 |
| 14 | $4000 (16384) | $DA (218) | Reserved |
| 15 | $8000 (32768) | $E1-$E8 (225-232) | * Digital input(s) 1-8 low (use n2 for mask) |
|  | UI, user interrupt command | $F0-$FF (240-255) | User Interrupt, See UI command |

###### n2 Bit Mask

Input Interrupts

| bit | n2 = 2^bit hex (decimal) | Status Byte hex (decimal) | Condition |
| --- | --- | --- | --- |
| 0 | $01 (1) | $E1 (225) | * Digital input 1 is low @IN[1] = 0 |
| 1 | $02 (2) | $E2 (226) | * Digital input 2 is low @IN[2] = 0 |
| 2 | $04 (4) | $E3 (227) | * Digital input 3 is low @IN[3] = 0 |
| 3 | $08 (8) | $E4 (228) | * Digital input 4 is low @IN[4] = 0 |
| 4 | $10 (16) | $E5 (229) | * Digital input 5 is low @IN[5] = 0 |
| 5 | $20 (32) | $E6 (230) | * Digital input 6 is low @IN[6] = 0 |
| 6 | $40 (64) | $E7 (231) | * Digital input 7 is low @IN[7] = 0 |
| 7 | $80 (128) | $E8 (232) | * Digital input 8 is low @IN[8] = 0 |

###### EI

UDP Interrupts Framing

The UDP packet can contain up to 16 individual status bytes and is framed as follows

| Format | Header (Fixed Byte) | Status Byte (1-16 bytes) | Payload Byte Count (0003 - 0x12) [Includes header and footer in count] |
| --- | --- | --- | --- |
| Example | 0001 | 0xD0F1DBE1 | 0006 |
| Example Decoded | Interrupt Packet Indicator | Axis A Profiled Motion Complete; User Interrupt 1; Application Program Stopped; Digital Input 1 is low | 6 bytes in payload |

Examples

'Galil DMC Code Example

'Interrupt when motion is complete on all axes OR if a limit switch is hit:

'From the table, enable bits 8 and 10.  n1 = 256 + 1024 = 1280

EI  1280

'

'Interrupt when digital input 3 is low.

'Enable bit 15 of n1 and bit 2 of n2.

EI 32768,4

EI applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806,DMC1802

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EI

PROGRAMMING

##### ELSE   Else function for use with IF conditional statement

ELSE

ELSE

| Usage | ELSE | Command takes no arguments |
| --- | --- | --- |

Description

The ELSE command is an optional part of an IF conditional statement. The ELSE command must occur after an IF command and it has no arguments. It allows for the execution of a command only when the argument of the IF command evaluates False. If the argument of the IF command evaluates false, the controller will skip commands until the ELSE command. If the argument for the IF command evaluates true, the controller will execute the commands between the IF and ELSE command.

Arguments

ELSE is a command with no parameters

Remarks

None

Examples

'Galil DMC Code Example

IF (@IN[1]=0);'                 IF conditional statement based on input 1

IF (@IN[2]=0);'                2nd IF conditional statement executed if 1st IF conditional true

MG "IN1 AND IN2 ARE ACTIVE";' Message to be executed if 2nd IF conditional is true

ELSE;'                         ELSE command for 2nd IF conditional statement

MG "ONLY IN1 IS ACTIVE";'     Message to be executed if 2nd IF conditional is false

ENDIF;'                        End of 2nd conditional statement

ELSE;'                          ELSE command for 1st IF conditional statement

IF (@IN[2]=0);'             3rd IF conditional statement executed if 1st IF conditional false

MG "ONLY IN2 IS ACTIVE";'  Message to be executed if 3rd IF conditional statement is true

ELSE;'                      ELSE command for 3rd conditional statement

MG "IN1 AND IN2 INACTIVE";'Message to be executed if 3rd IF conditional statement is false

ENDIF;'                     End of 3rd conditional statement

ENDIF;'                      End of 1st conditional statement

ELSE applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ELSE

ECAM/GEARING

##### EM   Ecam modulus

EMm= n

EM n,n,n,n,n,n,n,n

EMm= n

EM n,n,n,n,n,n,n,n

| Usage | EMm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | EM n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _EMm | Operand holds the value last set by the command |

Description

The EM command defines the change in position over one complete cycle of the master.

The field for the master axis is the cycle of the master position. For the slaves, the field defines the net change in one cycle.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
|  | M |  | N | N/A | Axis | Virtual axis to assign value | Virtual axes are only valid as ECAM masters |
| n | 2 |  | 8,388,607 | N/A | 1 | Position change over one full ECAM cycle | For defining master axis |
|  | 0 |  | 2,147,483,647 | N/A | 1 | Position change over one full ECAM cycle | For defining slave axis |

Remarks

If a slave will return to its original position at the end of the cycle, then n=0. If the change is negative, specify the absolute value for n.

Examples

'Galil DMC Code Example

REM example using A axis main encoder as master B axis main encoder as the slave #cam

REM define A axis encoder as master for ECAM

EA A

REM

REM EM command options

REM ----

REM define slave modulus as 0 (returns to original position)

REM and define master modulus as 4000

EM 4000,0

REM

REM another valid EM settings for this configuration

REM ----

'EMA= 4000;' define A axis master modulus as 0

'EMB= 0;'    define B axis slave modulus as 0

REM

REM ----

REM define master increment as 1000 counts/table entry

EP 1000

ET[0]= , 0

ET[1]= , 1000

ET[2]= , 2000

ET[3]= , 1000

ET[4]= , 0

REM enable ECAM mode

EB 1

REM engage when master is at 0 position

EG 0,0

EN

'Galil DMC Code Example

EAC;' Select C axis as master for ECAM.

EM 0,3000,2000;' Define the changes in A and B to be 0 and 3000 respectively.  Define master cycle as 2000.

V = _EMA;' Return cycle of A

EM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EM

PROGRAMMING

##### EN End

EN n

0

,n

1

,n

2

EN n

0

,n

1

,n

2

| Usage | EN n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The EN command is used to designate the end of a program or subroutine. If a subroutine was called by the JS command, the EN command ends the subroutine and returns program flow to the point just after the JS command.

A return parameter can be specified to EN from a subroutine to return a value from the subroutine to the calling stack.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 | 1 | 0 | 1 | Specify trippoint status when returning from subroutine | n0=1 restores trippoints. n0=0 does not restore trippoints |
| n1 | 0 | 1 | 0 | 1 | Set status of CI interrupt when returning from #COMINT | n1=1 restores CI interrupt. n1=0 does not restore CI interrupt |
| n2 | - 2,147,483,648 | 2,147,483,647 | 0 | 1 | Return a value from a subroutine. | Accesible from the calling program with _JS. See JS for more information |

Remarks

The EN command is used to end the automatic subroutines #MCTIME #COMINT and #CMDERR.

Use the RE command to end the #POSERR and #LIMSWI subroutines. Use the RI command to end the #ININT subroutine

Examples

'Galil DMC Code Example

#A;'     Program A

PR 500;' Move A axis forward 500 counts

BGA;'    Begin motion

AMA;'    Pause the program until the A axis completes the motion

EN;'     End of Program

'Galil DMC Code Example #example 'test program showing restoring trippoints with EN XQ#err,1;' Execute thread to generate error AI1;' Wait for input 1 to trigger MG"hello";' After input, message out EN #err 'dummy thread that runs to cause an error XX123;' Invalid command 'causes CMDERR to be called, interrupting thread 0 EN #CMDERR 'error subroutine running on thread 0 tc=_TC;' Save error code EN1;' End routine, restore AI trippoint.

EN applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EN

PROGRAMMING

##### ENDIF   End of IF conditional statement

ENDIF

ENDIF

| Usage | ENDIF n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The ENDIF command is used to designate the end of an IF conditional statement. An IF conditional statement is formed by the combination of an IF and ENDIF command. An ENDIF command must always be executed for every IF command that has been executed. It is recommended that the user not include jump commands inside IF conditional statements since this causes re-direction of command execution. In this case, the command interpreter may not execute an ENDIF command.

Arguments

ENDIF is a command with no parameters

Remarks

None

Examples

'Galil DMC Code Example

IF (@IN[1]=0);'                 IF conditional statement based on input 1

IF (@IN[2]=0);'                2nd IF conditional statement executed if 1st IF conditional true

MG "IN1 AND IN2 ARE ACTIVE";' Message to be executed if 2nd IF conditional is true

ELSE;'                         ELSE command for 2nd IF conditional statement

MG "ONLY IN1 IS ACTIVE";'     Message to be executed if 2nd IF conditional is false

ENDIF;'                        End of 2nd conditional statement

ELSE;'                          ELSE command for 1st IF conditional statement

IF (@IN[2]=0);'             3rd IF conditional statement executed if 1st IF conditional false

MG "ONLY IN2 IS ACTIVE";'  Message to be executed if 3rd IF conditional statement is true

ELSE;'                      ELSE command for 3rd conditional statement

MG "IN1 AND IN2 INACTIVE";'Message to be executed if 3rd IF conditional statement is false

ENDIF;'                     End of 3rd conditional statement

ENDIF;'                      End of 1st conditional statement

ENDIF applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ENDIF

SYSTEM CONFIG

##### EO Echo

EO n

EO n

| Usage | EO n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _EO | Operand holds the value last set by the command |

Description

The EO command turns the echo on or off. If the echo is off, characters input over the bus will not be echoed back.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Echo Off |  |
|  | 1 | Echo On | Default |

Remarks

This command is defaulted to EO1. Galil software upon connection will set EO0

The EO command is accepted over the serial port only.

The ethernet port will not echo commands

Examples

'Galil DMC Code Example

EO 0;' Turns echo off

EO 1;' Turns echo on

EO applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EO

ECAM/GEARING

##### EP   Cam table master interval and phase shift

EP n

0

,n

1

EP n

0

,n

1

| Usage | EP n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _EP | Operand holds the value last set by the command |

Description

The EP command defines the ECAM table intervals and offset. The offset is the master position of the first ECAM table entry. The interval is the difference of the master position between 2 consecutive table entries. This command effectively defines the size of the ECAM table. Up to 257 points may be specified.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | 1 | 32,767 | 256 | 1 | Master position interval | Cannot be changed while ECAM is running |
| n1 | -2,147,483,648 | 2,147,483,647 | 0 | 1 | ECAM table phase shift | Can be modified during ECAM |

Remarks

_EP contains the value of the interval n0.

The offset parameter 'n1' can also be used to instantaneously phase shift the graph of the slave position verses the master position. This can be used to make on-the-fly corrections to the slaves.

See application note #2502 for more details. http://www.galil.com/download/application-note/note2502.pdf

Examples

'Galil DMC Code Example

EP 20;'           Sets the cam master points to 0,20,40 . . . d =  _EP;'        Set the variable d equal to the ECAM internal master interval

EP,100;'          Phase shift all slaves by 100 master counts

EP applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EP

ECAM/GEARING

##### EQ ECAM quit (disengage)

EQm= n

EQ n,n,n,n,n,n,n,n

EQm= n

EQ n,n,n,n,n,n,n,n

| Usage | EQm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | EQ n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _EQm | Operand has special meaning, see Remarks |

Description

The EQ command disengages an electronic cam slave axis at the specified master position. Separate points can be specified for each axis. If a value is specified outside of the master's range, the slave will disengage immediately.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | - 2,147,483,648 | 2,147,483,647 | N/A | 1 | Master position to disengage the slave axis specified. | If n = outside of master position range, disengage slave axis immediately. |

Remarks

_EQn contains 1 if engage command issued and axis is waiting to engage, 2 if disengage command issued and axis is waiting to disengage, and 0 if ECAM engaged or disengaged.

n = ? Returns 1 if engage command issued and axis is waiting to engage, 2 if disengage command issued and axis is waiting to disengage, and 0 if ECAM engaged or disengaged.

This command is not a trippoint. This command will not hold the execution of the program flow.

If the execution needs to be held until master position is reached, use MF or MR command.

Examples

'Galil DMC Code Example

EQ 300,700;' Disengages the A and B motors at master positions 300 and 700 respectively.

'Galil DMC Code Example

EQ 300;' Disengages the A motor at master position 300.

EQ applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

###### EQ

ERROR CONTROL

ER   Error Limit

ERm= n

ER n,n,n,n,n,n,n,n

ERm= n

ER n,n,n,n,n,n,n,n

| Usage | ERm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | ER n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _ERm | Operand holds the value last set by the command |

Description

The ER command sets the magnitude of the position errors for each axis that will trigger an error condition. When the limit is exceeded, the Error output will go low (true) and the controller's red light will be turned on. If the Off On Error (OE1) command is active, the motors will be disabled.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | -1 |  | 2,147,483,647 | 16,384 | 1 | Set the position error limit in counts | n=0 enables Error output. n=-1 disables Error output. |

Remarks

The error limit specified by ER should be high enough as not to be reached during normal operation.

Examples of exceeding the error limit would be a mechanical jam, or a fault in a system component such as encoder or amplifier For debugging purposes, ER0 and ER-1 can be used to turn the red LED on and off.

Examples

'Galil DMC Code Example

:ER 200,300,400,600;' Set the A-axis error limit to 200, the B-axis error limit to 300, the C-axis error limit to 400, and the D-axis error limit to 600. :ER ,1000;' Sets the B-axis error limit to 1000, leave the A-axis error limit unchanged.

:ER ?,?,?,?;' Return A,B,C and D values

200,1000,400,600

:ER ?;' Return A value

200

:V1=_ERA;' Assigns V1 value of ERA

:MG V1;' Returns V1

200

ER applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ER

VECTOR/LINEAR

##### ES Ellipse Scale

ES n

0

,n

1

ES n

0

,n

1

| Usage | ES n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The ES command divides the resolution of one of the axes in a vector mode (VM). This function allows for the generation of circular motion when encoder resolutions differ. It also allows for the generation of an ellipse instead of a circle. The resolution change applies for the purpose of generating the VP and CR commands, effectively changing the axis with the higher resolution to match the coarser resolution.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 1 |  | 65,535 | 1 | 1 | First value used for resolution scaling | See Remarks for usage |
| n1 | 1 |  | 65,535 | 1 | 1 | Second value used for resolution scaling | See Remarks for usage |

Remarks

For VM xy

When n0 > n1, the resolution of x will be multiplied by n0/n1

When n0 < n1, the resolution of y will be multiplied by n1/n0

The ES command will apply to the selected coordinate system, S or T. To select the coordinate system, use the command CAS or CAT.

Examples

'Galil DMC Code Example

VMAB;ES 3,4;' Divide B resolution by 4/3

VMCA;ES 2,3;' Divide A resolution by 3/2

VMAC;ES 3,2;' Divide A Resolution by 3/2 'Note:  ES must be issued after VM.

'Galil DMC Code Example

VMAN;ES 3,2;' Divide A Resolution by 3/2 'Note:  ES must be issued after VM.

ES applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

###### ES

ECAM/GEARING

ET   Electronic cam table

ET n,n,n,n,n,n,n,n

ET[n0] = n,n,n,n,n,n,n,n

| Usage | ET n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

#### Description

The ET command sets the ECAM table entries for the slave axes. The values of the master axes are not required. The slave entry (n) is the position of the slave axes when the master is at the point (m i) + o, where i is the interval and o is the offset as determined by the EP command.

#### Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 | 256 | N/A | 1 | Index of the ECAM table entry |  |
| n | -2,147,483,648 | 2,147,483,647 | 0 | 1 | Position of the slave axis at the specified table point. |  |

#### Remarks

[n0] can be omitted only if EC has initialized the index count. In this case, each ET command will increment the index counter by 1. n=? Returns the slave position for the specified point.

#### Examples

'Galil DMC Code Example

ET[0]= 0,,0;'      Specifies the position of the slave axes A and C to be synchronized with the starting point of the master.

ET[1]= 1200,,400;' Specifies the position of the slave axes A and C to be synchronized with the second point of the master

EC 0;'              Set the table index value to 0, the first element in the table

ET 0,,0;'          Specifies the position of the slave axes A and C to be synchronized with the starting point of the master.

ET 1200,,400;'     Specifies the position of the slave axes A and C to be synchronized with the second point of the master

ET applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

##### ET

ECAM/GEARING EW ECAM Widen Segment

EW n

0

,n

1

,n

2

,n

3

EW n

0

,n

1

,n

2

,n

3

| Usage | EW n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _EW0 _EW1 _EW2 _EW3 | Operand has special meaning, see Remarks |

Description

The EW command allows widening the length of one or two ECAM segments beyond the width specified by EP. For ECAM tables with one or two long linear sections, this allows placing more points in the curved sections of the table. There are only two widened segments, and if used they are common for all ECAM axes.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 1 |  | 255 | -1 | 1 | Index of first widened segment | If n0 = -1, no segment is widened |
| n1 | 1 |  | 2,147,483,647 | 0 | 1 | Length of first widened segment | In master counts |
| n2 | 3 |  | 255 | -1 | 1 | Index of second widened segment | If n2 = -1, no segment is widened. n2 must be > n0 |
| n3 | 1 |  | 2,147,483,647 | 0 | 1 | Length of second widened segment | In master counts |

Remarks

Remember that the widened segment lengths must be taken into account when determining the modulus (EM) for the master.

The second widened segment cannot be used unless the first widened segment is also being used. The segments chosen should not be the first or last segments, or consecutive segments.

###### Operand Usage

_EW0 contains n0, the index of the first widened segment.

_EW1 contains n1, the length of the first widened segment.

_EW2 contains n2, the index of the second widened segment

_EW3 contains n3, the length of the second widened segment.

Examples

'Galil DMC Code Example

EW 41, 688;'            Widen segment 41 to 688 master counts

EW 41, 688, 124, 688;'   Widen segments 41 and 124 to 688 master counts

EW applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

EW

ECAM/GEARING

##### EY ECAM Cycle Count

EY n

EY n

| Usage | EY n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _EY | Operand holds the value last set by the command |

Description

The EY command sets or gets the ECAM cycle count. This is the number of times that the ECAM axes have exceeded their modulus as defined by the EM command. EY will increment by one each time the master exceeds its modulus in the positive direction, and EY will decrement by one each time the master exceeds its modulus in the negative direction.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | -2,147,483,648 | 2,147,483,647 | 0 | 1 | Current ECAM cycle count |  |

Remarks

_EY returns the current cycle count

EY can be used to calculate the absolute position of an axis with the following equation: Absolute position = EY * EM + TP

Examples

'Galil DMC Code Example

MG _EY * _EMY + _TPY;'         print absolute position of master (Y)

EY applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

###### EY

FILTER/CONTROL FA Acceleration Feedforward

### FAm= n FA n,n,n,n,n,n,n,n

| Usage | FAm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | FA n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _FAm | Operand holds the value last set by the command |

Description

The FA command sets the acceleration feedforward coefficient. This coefficient is scaled by the set acceleration and adds a torque bias voltage during the acceleration phase and subtracts the bias during the deceleration phase of a motion.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 8,191 | 0 | 1/4 | Value of proportional term |  |

Remarks

The Feedforward Bias product is limited to 10 Volts.

If the feedforward coefficient is changed during a move, then the change will not take effect until the next move.

FA operates on PA, PR, IP, JG and PVT mode.

FA does not operate in: Contour Mode (CM)

Axis is Gearing or ECAM slave

Coordinated motion (LM, VM)

Acceleration Feedforward Bias = FA * AC * (1.5 10-7) * ((TM/1000)^2)

Deceleration Feedforward Bias = FA * DC * (1.5 10-7) * ((TM/1000)^2)

Examples

'Galil DMC Code Example

'Set feedforward coefficient to 10 for the A-axis 'and 15 for the B-axis.  The effective bias will 'be 0.75V for A and 2.25V for B.

:AC 500000,1000000

:FA 10,15

:MG _FAA,_FAB

10 15

FA applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

FA

FILTER/CONTROL

#### FC Distance-selectable feedforward gain

FCm= n

FC n,n,n,n,n,n,n,n

FCm= n

FC n,n,n,n,n,n,n,n

| Usage | FCm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | FC n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _FCm | Operand holds the value last set by the command |

Description

Adds a bias to the torque output TT proportional to the commanded velocity if the distance from the end of the move is less than FN. FC is the same as FV but activated FN counts from the end of the move and both positive and negative values are allowed.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -8,191 | 8,191 | 0 | 1 | Value of distance selectable feedforward gain |  |

Remarks

Valid only in -NAN and -CER firmware

Bias in volts = 1.22 E-6 * FC . (commanded Velocity in counts/s)

Examples

'Galil DMC Code Example

SPA=100000;'set speed to 100,000 cnts/second

FCA=10;'    set distance-selectable velocity feedforward gain to 10

FNA=50000;'  set distance from end of move when FC is engaged to 5000 counts

PRA=100000;' command move of 10,000 counts

BGA;'       begin move

EN

' Move shown below with KP 0,KD 0,KI 0,K1 0,K2 0,K3 0

FC applies to CER,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

FC

INDEPENDENT MOTION

#### FE Find Edge

FE mm

FE mm

| Usage | FE mm | Argument is an axis mask |
| --- | --- | --- |

Description

The FE command moves a motor until a transition is seen on the home input for that axis. The direction of motion depends on the initial state of the homing input (use the CN command to configure the polarity of the home input). Once the transition is detected, the motor decelerates to a stop. This command is useful for creating custom homing sequences.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | N/A | Multi-Axis Mask | Axes to Find Edge |  |

Remarks

Find Edge only searches for a change in state on the Home Input. Use FI (Find Index) to search for the encoder index. Use HM (Home) to search for both the Home input and the Index.

Remember to specify BG after each of these commands

Speed of Find Edge is set with the SP command and should be low enough to allow for a minimum of a 2 sample period pulse width on the home signal. With TM 1000, the pulse width must be at least 2ms.

Direction of Travel

| _CN1 value | Home input digital state | _HMn state | Direction of travel if FE begun in this state |
| --- | --- | --- | --- |
| -1 | pull-up or non-active opto | 1 | Backward |
| -1 | grounded or active opto | 0 | Forward |
| 1 | pull-up or non-active opto | 0 | Forward |
| 1 | grounded or active opto | 1 | Backward |

Examples

'Galil DMC Code Example

:FEB;' Only find edge on B

:BGB;

:FECD;' Find edge on C and D

:BGCD;

FE applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

FE

INDEPENDENT MOTION

#### FI Find Index

FI mm

FI mm

| Usage | FI mm | Argument is an axis mask |
| --- | --- | --- |

Description

The FI and BG commands move the motor until an encoder index pulse is detected.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | N/A | Multi-Axis Mask | Axes to Find Index |  |

Remarks

The controller looks for a transition from low to high. There are 2 stages to the FI command. The first stage jogs the motor at the speed and direction of the JG command until a transition is detected on the index line. When the transition is detected, the position is latched and the motor will decelerate to a stop. In the second stage, the motor will reverse direction and move to the latched position of the index pulse at the speed set by the HV command. At the conclusion of FI, the position is defined as zero.

Find Index only searches for a change in state on the Index. Use FE to search for the Home. Use HM (Home) to search for both the Home input and the Index. Remember to specify BG after each of these commands.

Examples

'Galil DMC Code Example #HOME;' Home Routine JG 1000,-2000;' Set the speed and direction for the first phase of the FI move HV 500,500;' Set the speed for the second phase of the FI move FI AB;' Queue up a find edge move on the A and B axes ' Direction of phase 2 is opposite of phase 1. BG B;' Begin FI move on B axis AM B;' After the move has finished on axis B, BG A;' Begin FI move on the A axis AM A;' After the move has finished on axis A, MG "FI done";' Output a message indicating the FI move is complete. EN

FI applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

FI

ERROR CONTROL

#### FL Forward Software Limit

FLm= n

FL n,n,n,n,n,n,n,n

FLm= n

FL n,n,n,n,n,n,n,n

| Usage | FLm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | FL n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _FLm | Operand has special meaning, see Remarks |

Description

The FL command sets the forward software position limit. If this limit is exceeded during motion, motion on that axis will decelerate to a stop. Forward motion beyond this limit is not permitted.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -2,147,483,648 | 2,147,483,647 | 2,147,483,647 | 1 | Value of software forward limit | 2147483647 turns off forward limit |

Remarks

The forward limit is activated at n+1. n = 2147483647 effectively disables the forward soft limit.

The software limit is specified in counts for a servo system or in microsteps for a stepper system.

When the forward software limit is activated, the automatic subroutine #LIMSWI will be executed if it is included in the program. If motion is commanded when the axis is already passed the FL value, the controller will return error code 22. See TC for details.

Examples

'Galil DMC Code Example #TEST;' Test Program AC 1000000;' Acceleration Rate DC 1000000;' Deceleration Rate FL 15000;' Forward Limit JG 5000;' Jog Forward BGA;' Begin AMA;' After Limit RPA;' Tell Position EN;' End 'Hint: Galil controllers also provide hardware limits.

FL applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

FL

FILTER/CONTROL

#### FN Distance from end of move when FC is engaged

FNm= n

FN n,n,n,n,n,n,n,n

FNm= n

FN n,n,n,n,n,n,n,n

| Usage | FNm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | FN n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _FNm | Operand holds the value last set by the command |

Description

Adds a bias to the torque output TT proportional to the commanded velocity if the distance from the end of the move is less than FN. FC is the same as FV but activated FN counts from the end of the move and both positive and negative values are allowed.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 2,147,483,647 | 0 | 1 | Distance from end of move for FC to engage |  |

Remarks

Valid only in -NAN and -CER firmware

Bias in volts = 1.22 . E-6 . FC . (commanded Velocity in counts/s)

Examples

'Galil DMC Code Example

SPA=100000;'set speed to 100,000 cnts/second

FCA=10;'    set distance-selectable velocity feedforward gain to 10

FNA=50000;'  set distance from end of move when FC is engaged to 5000 counts

PRA=100000;' command move of 10,000 counts

BGA;'       begin move

EN

'Move shown below with KP 0,KD 0,KI 0,K1 0,K2 0,K3 0

FN applies to CER,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

FN

FILTER/CONTROL FV Velocity Feedforward

### FVm= n FV n,n,n,n,n,n,n,n

| Usage | FVm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | FV n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _FVm | Operand has special meaning, see Remarks |

Description

The FV command sets the velocity feedforward coefficient. This coefficient generates an output bias signal in proportions to the sample to sample change in reference position (RP).

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 8,191.75 | 0 | 0.25 | Value of proportional term |  |
|  | 0 |  | 8,191 | 0 | 1 | Value of proportional term | -CER firmware only. |

Remarks

FV also applies to Contour Mode (CM) and in gearing when an axis is a slave

Velocity feedforward bias = FV * (Velocity [cts/s]) * (1.20 10-6) * (TM/1000)

With FVA=10, TM 1000 and the velocity is 200,000 count/s, the velocity feedforward bias equals 2.40 volts

Examples

'Galil DMC Code Example

'Set feedforward coefficients to 10 and 20 for A and B respectively. 'This effective bias will be 0.366 volts for A and 1.95 volts for B.

:FV 10,20

:JG 30000,80000

:MG _FVA,_FVB

10 20

FV applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

FV

ECAM/GEARING

#### GA   Master Axis for Gearing

GAm0= m

GA m,m,m,m,m,m,m,m

| Usage | GAm0= m | Arguments are single axis masks and are specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | GA m ... | Arguments are single axis masks specified with an implicit, comma-separated order |

#### Description

The GA command specifies the master axes for electronic gearing. Multiple masters for gearing may be specified. A slave axis may have only one master. The masters may be the main encoder input, auxiliary encoder input, or the commanded position of any axis. The master may also be the commanded vector move in a coordinated motion of LM or VM type. When the master is a simple axis, it may move in any direction and the slave follows. When the master is a commanded vector move, the vector move is considered positive and the slave will move forward if the gear ratio is positive, and backward if the gear ratio is negative. The slave axes and ratios are specified with the GR command and gearing is turned off by the command GR0.

#### Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m0 | A |  | H | N/A | Axis | Slave axis to assign master | m0<>m |
| m | A |  | H | N/A | Axis | Master axis main encoder as the slave's master |  |
|  | CA |  | CH | N/A | Axis | Master axis commanded position as the slave's master | Valid arguments: CA,CB,CC,CD,CE,CF,CG,CH |
|  | DA |  | DH | N/A | Axis | Master axis aux encoder as the slave's master | Valid arguments: DA,DB,DC,DD,DE,DF,DG,DH |
|  | S |  | T | N/A | Axis | Vector plane as the slave's master |  |
|  | M |  | N | N/A | Axis | Virtual axis as the slave's master |  |

#### Remarks

m=? returns the GA setting

When gearing is used in a gantry application, gearing off of the commanded position is recommended

When an axis is geared to a master axis, the slave's geared profile will be superimposed to the slave's commanded profile Gearing is disabled in the following conditions:

The gear ratio is set to 0

A limit switch is reached

The axis is commanded to stop with the ST command

If it is desired that gearing is not disabled when a limit switch is reached or an ST command is issued, enable Gantry Mode (GM command).

#### Examples

'Galil DMC Code Example REM setup gearing where B axis is master for A and C axes. #gear MO B;' Turn off servo to B motor GA B,,B;' Specify master axis as B on A and C GR .25,,-5;' Specify A and C gear ratios SH B;' Enable B axis PRB= 1000;BG B;' Move B axis 1000 counts ' A axis will be commanded to move 250 counts positive ' C axis will be commanded to move -5000 counts EN;' End program

'Galil DMC Code Example

REM imaginary axis example

#imag

GAA= N;'         set the imaginary N axis as the master of the A axis

GRA= 2.5;'       set the gear ratio for the A axis as 2.5

PRN= 1000;'      Move N axis 1000 counts

PRA= 1000;'      Move A axis 1000 counts (will be superimposed to the profiled position due to gearing)

BG AN

'                 (A axis will be commanded to move 3500 counts positive >> 2500 due to gearing + 1000 due to commanded move) EN;'             End Program

GA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

GA

##### GD Gear Distance

GD n,n,n,n,n,n,n,n

GDm= n

GD n,n,n,n,n,n,n,n

GDm= n

| Usage | GD n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _GDm | Operand holds the value last set by the command |

Description

The GD command sets the distance of the master axis over which the specified slave will be engaged, disengaged or changed to a new gear setting. Using this command will cause the master axis and the slave axis positions to differ due to the gradual gear shift. To correct for this, see the _GP operand

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Slave axis to assign value |  |
| n | 0 |  | 32,767 | 0 | 1 | Absolute Value of Gearing Distance | 0 engages gearing instantly |

Remarks

The distance is entered as an absolute value, the motion of the master may be in either direction. If the distance is set to 0, then the gearing will engage instantly.

Examples

'Galil DMC Code Example #A GA ,A;' Sets the A axis as the gearing master for the B axis GD,5000;' Set distance over which gearing is engaged to 5000 counts of the master axis. JG 5000;' Set the A axis jog speed to 5000 cts/sec BG A;' Begin motion on the A axis AS A;' Wait until A axis reaches the set speed of 5000 counts/sec GR ,1;' Engage gearing on the B axis with a ratio of 1:1, the 'distance to fully engage gearing will be 5000 counts of the master axis WT 1000;' Wait 1 second GR ,3;' Set the gear ratio to three. The ratio will be changed 'over the distance set by the GD command WT 1000;' Wait 1 second GR ,0;' Disengage the gearing between the B axis slave and the 'master. The gearing will be disengaged over the number of 'counts of the master specified with the GD command above EN;' End program

'Galil DMC Code Example #A GA DA;' Set the aux encoder input as the gearing master GD 5000;' Set distance over which gearing is engaged to 5000 counts of the master axis. GR 1;' Set a gear ratio of 1:1, the distance to fully 'engage gearing will be 5000 counts of the master axis WT 1000;' Wait 1 second GR 3;' Set the gear ratio to three. The ratio will be changed 'over the distance set by the GD command WT 1000;' Wait 1 second GR 0;' Disengage the gearing between the axis aux encoder 'The gearing will be disengaged over the number of 'counts of the master specified with the GD command above EN;' End program

GD applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

GD

##### GM Gantry mode

GMm= n

GM n,n,n,n,n,n,n,n

GMm= n

GM n,n,n,n,n,n,n,n

| Usage | GMm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | GM n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _GMm | Operand holds the value last set by the command |

Description

The GM command specifies the axes in which the gearing function is performed in the Gantry mode. In this mode, the geared slaves will not be stopped by the ST command or by limit switches.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 1 |  | 0 | 1 | Value of GM command | 1 Enables Gantry Mode, 0 disables Gantry Mode |

Remarks

Only setting Gantry Mode of the slave to 0 (GMm= 0) will disable Gantry Mode

Examples

'Galil DMC Code Example

GM 1,1,1,1;'     Enable GM on all axes

GM 0;'           Disable GM on A-axis, other axes remain unchanged

GM ,,1,1;'       Enable GM on C-axis and D-axis, other axes remain unchanged

GM 1,0,1,0;'     Enable GM on A and C-axis, disable GM on B and D axis

'Galil DMC Code Example GA DA;' Set master for A axis to the A axis Aux encoder input GM 1;' Enable Gantry Mode on A axis GR 1;' Set Gear Ratio to 1 WT 1000 ST;' Axis will still be in gearing Mode WT 1000 GM 0;' Disable Gantry Mode (Axis still gearing) WT 1000 ST;' Will clear gearing mode EN

GM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

GM

##### GR Gear Ratio

GRm= n

GR n,n,n,n,n,n,n,n

GRm= n

GR n,n,n,n,n,n,n,n

| Usage | GRm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | GR n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _GRm | Operand holds the value last set by the command |

Description

GR specifies the Gear Ratios for the geared axes in the electronic gearing mode. The master axis is defined by the GA command.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Slave axis to assign gear ratio |  |
| n | -127 | 127 | 0 | 1/65,536 | Value of Gear Ratio of Slave | n = 0 disables gearing |

Remarks

The gear ratio may be different for each geared axis.

The master can go in both directions.

Gearing is disabled in the following conditions:

The gear ratio is set to 0

A limit switch is reached

The axis is commanded to stop with the ST command

If it it is desired that gearing is not disabled when a limit switch is reached or an ST command is issued, enable Gantry Mode (GM command).

Examples

'Galil DMC Code Example

REM setup gearing where B axis is master for A and C axes.

#gear

MOB;'          Turn off servo to B motor

GAB,,B;'       Specify master axis as B

GR .25,,-5;'   Specify A and C gear ratios

SHB;'          Enable B axis

PRB=1000;BGB;' Move B axis 1000 counts

'              A axis will be commanded to move 250 counts positive

'              C axis will be commanded to move 5000 counts negative (-5000)

EN;'           End program

'Galil DMC Code Example REM setup gearing where virtual axis, N, is master for axis A. #gear GA N;' Specify master axis as N (imaginary Axis) GR -2;' Specify gear ratio or -2 PRN=1000;BGN;' Move N axis 1000 counts WT 1000 MG _RPA,_RPN;' will indicate -2000 on A and 1000 on N EN;' End program :'execution of gearing example :XQ : -2000.0000 1000.0000 :

GR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

GR

INDEPENDENT MOTION

##### HM Home

HM mm

HM mm

| Usage | HM mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _HMm | Operand has special meaning, see Remarks |

Description

The HM command performs a three stage homing sequence for servo systems and a two stage sequence for stepper motors.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axis to perform Homing Routine | No argument homes all axes |

Remarks

The HM command is derived of FE and FI commands. Custom homing sequences can be created by using the FE (Find Edge) and FI (Find Index) commands.

The sequence of FE and FI commands varies depending upon if the axis is configured for a stepper or servo

###### Step One. Servos and Steppers

During the first stage of the homing sequence, the motor moves at the user-programmed speed until detecting a transition on the homing input for that axis. The speed for step one is set with the SP command.

The direction for this first stage is determined by the initial state of the homing input. The state of the homing input can be configured using the second field of the CN command.

Once the homing input changes state, the motor decelerates to a stop.

###### Step Two. Servos and Steppers

At the second stage, the motor changes directions and approaches the transition again at the speed set with the HV command. When the transition is detected, the motor is stopped instantaneously.

###### Step Three. Servos only

At the third stage, the motor moves in the positive direction at the speed set with the HV command until it detects an index pulse via latch from the encoder. It returns to the latched position and defines it as position 0.

###### Operand

_HMm state as a function of CN,n and Home digital input

| _CN1 value | Home input digital state | _HMn state | Direction of travel if HM begun in this state |
| --- | --- | --- | --- |
| -1 | pull-up or non-active opto | 1 | Backward |
| -1 | grounded or active opto | 0 | Forward |
| 1 | pull-up or non-active opto | 0 | Forward |
| 1 | grounded or active opto | 1 | Backward |

Examples

'Galil DMC Code Example

:HM;'     Set Homing Mode for all axes :BG;'     Home all axes

:HMA;'     Set Homing Mode for axis A

:BGA;'     Home only the A-axis

HM applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

HM

ETHERNET

###### HS   Handle Assignment Switch

HSm0= m

HSm0= m

| Usage | HSm0= m | Arguments are single axis masks and are specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | HS m ... | Arguments are single axis masks specified with an implicit, comma-separated order |

Description

The HS command is used to switch the ethernet handle assignments between two handles. Handles are opened when a connection is established by an external client (TCP or UDP), or when a handle is assigned explicitly with the IH command. Should those assignments need modifications, the HS command allows the handles to be reassigned.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m0 | A |  | H |  | N/A | Handle | First handle to switch |  |
|  | S |  | S |  | N/A | Handle | First handle to switch | S = current handle sending command. Not valid in program |
| m | A |  | H |  | N/A | Handle | Second handle to switch |  |
|  | S |  | S |  | N/A | Handle | Second handle to switch | S = current handle sending command. Not valid in program |

Remarks

A handle encapsulates the following 4 pieces of information:

- Local IP address (same for all handles)

- Remote IP address

- Local Port

- Remote Port

Handles are used as a pointer to the network socket in commands such as SAh, MBh, {Eh}, and IHh where h is the handle letter

Examples

'Galil DMC Code Example

:HSC= D;' Connection for handle C is assigned to handle D.  Connection for handle D is assigned to handle C.

:HSS= E;' Executing handle connection is assigned to handle E.  Connection for handle E is assigned to executing handle.

HS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

HS

INDEPENDENT MOTION HV   Homing Velocity

### HVm= n HV n,n,n,n,n,n,n,n

| Usage | HVm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | HV n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _HVm | Operand holds the value last set by the command |

Description

Sets the slew speed for the FI final move to the index and all but the first stage of HM.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 15,000,000 | 256 | 2 | Value of Homing Velocity in cnts/second | For MT settings of 1,-1,1.5 and -1.5 (Servos) |
|  | 0 |  | 3,000,000 | 256 | 2 | Value of Homing Velocity in cnts/second | For MT settings of 2,-2,2.5 and -2.5 (Steppers) |

Remarks

None

Examples

'Galil DMC Code Example

HVA=1000;'  set homing speed

HMA;'       home to home switch then index

BGA;'       begin motion

AMA;'       wait for motion complete

EN;'        end program

HV applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

HV

PROGRAMMING

#### HX   Halt Execution

HX n

HX n

| Usage | HX n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _HX0 _HX1 _HX2 _HX3 _HX4 _HX5 _HX6 _HX7 | Operand has special meaning, see Remarks |

Description

The HX command halts the execution of any program that is running.The parameter n specifies the thread to be halted.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  | 7 |  | N/A | 1 | Thread number to halt | If n omitted, all threads are halted. |

Remarks

When used as an operand, _HXn contains the running status of thread n with:

- Thread not running

- Thread is running

- Thread has stopped at trippoint

Examples

'Galil DMC Code Example

XQ #A;'    Execute program #A, thread zero

XQ #B,3;' Execute program #B, thread three

HX0;'     Halt thread zero

HX3;'    Halt thread three

HX applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

HX

ETHERNET

#### IA   IP Address

IA n

0

, n

1

, n

2

, n

3

IA n

4

IA < o

IA n

0

, n

1

, n

2

, n

3

IA n

4

IA < o

| Usage | IA n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _IA0 _IA1 _IA2 _IA3 _IA4 _IA5 | Operand has special meaning, see Remarks |

Description

The IA command assigns the controller IP address and the TCP time out. The IP address can also be assigned via Galil software or from an external server. The controller defaults to DHCP and will receive an IP address from a DHCP server if present. To manually set an IP address over the serial connection, send DH0 to disable DHCP prior to setting the new IP address with IA.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 | 255 | 0 | 1 | Byte 3 of the IP address |  |
| n1 | 0 | 255 | 0 | 1 | Byte 2 of the IP address |  |
| n2 | 0 | 255 | 0 | 1 | Byte 1 of the IP address |  |
| n3 | 0 | 255 | 0 | 1 | Byte 0 of the IP address |  |
| n4 | - 2,147,483,648 | 2,147,483,647 | 0 | 1 | The full IP address specified as a signed 32 bit two's complement integer |  |
| o | 1 | 2,147,483,647 | 250 | 1 | The time in update samples between TCP retries | Up to 5 retries occur |

Remarks

When specifying the IP address with IA, remember to use commas as delimeters instead of periods.

n4 = ? will return the IP address of the controller in comma seperated format.

Setting the IP address over Ethernet to a new value will cause an immediate disconnect/timeout. Reconnect to the controller on the new IP address and issue a BN to save the new value to flash.

To change the IP address manually over Ethernet on a controller which was initially assigned via DHCP, send "DH 0;IA n0,n1,n2,n3" as one command line. Reconnect on the new IP and issue BN to save.

##### Operands

_IA0 contains the IP address representing a 32 bit signed number (Two's complement). See the example below. _IA1 contains the value for o (retry time).

_IA2 contains the number of available handles.

_IA3 contains the number of the handle using this operand where the number is 0 to 7. 0 represents handle A, 1 handle B, etc. This is used by a remote device to detect its outgoing handle (see WH).

_IA4 contains the number of the handle that lost communication last, contains a -1 on reset to indicate no handles lost.

_IA5 returns autonegotiation Ethernet speed. Returns 10 for 10-Base T and returns 100 for 100-Base T, it will return -1 if there is no physical link.

Related Commands

DH - DHCP Client Enable

IH - Open IP Handle

TH - Tell Ethernet Handle

WH - Which Handle

Examples

'Galil DMC Code Example

IA 151,12,53,89;' Assigns the controller with the address 151.12.53.89

IA 2534159705;'   Assigns the controller with the address 151.12.53.89

IA < 500;'        Sets the timeout value to 500 msec

'Galil DMC Code Example

REM The individual IP address bytes can be derived within embedded code using _IA0 a=@INT[(_IA0&($FF000000))/$1000000]&$FF b=@INT[(_IA0&($00FF0000))/$10000] c=@INT[(_IA0&($0000FF00))/$100] d=@INT[(_IA0&($000000FF))]

REM IP address = a.b.c.d

##### IA

IA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

IA

INTERROGATION

#### ID   Identify

ID

ID

| Usage | ID | Command takes no arguments |
| --- | --- | --- |

Description

The ID command is used to query the controller for the hardware configuration and factory programming.

Arguments

ID is a command with no arguments

Remarks

Refer to the Examples section for actual controller responses The following are descriptions of the ID response The ID command follows this pattern:

:ID

FW, firmware revision

DMC, 4103, options, Rev #, nre #

AMP1, Amplifier model # for axes A-D, options, Rev #, nre # AMP2, Amplifier model # for axes E-H, options, Rev #, nre # :

Where the firmware revision is the string returned by the ^R^V command, options are any ordered options associated with that hardware component,

model # refers to the Galil part number for that product, Rev # is the hardware revision for that component,

nre # is the NRE number for that component. This field will not be present if there is no NRE number, and can appear on any component after the Rev field.

If a particular component is not present - i.e. the second amplifier on a one axis controller, it will not be listed in ID.

Examples

'Galil DMC Code Example 'Part Number: DMC-4143-BOX4-D3040 :ID FW, DMC4143 Rev 1.3a DMC, 4103, Rev 11 AMP1, 43040, Rev 6 :

ID applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,RIO47000,RIO57400

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ID

PROGRAMMING

#### IF IF conditional statement

IF (ex)

IF (ex)

| Usage | IF n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The IF command is used in conjunction with an ENDIF command to form an IF conditional statement. The arguments consist of one or more conditional statements and each condition must be enclosed with parenthesis (). If the conditional statement(s) evaluates true, the command interpreter will continue executing commands which follow the IF command. If the conditional statement evaluates false, the controller will ignore commands until the associated ENDIF command or an ELSE command occurs in the program.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| ex | N/A | N/A | N/A | Expression | Conditional statement for IF statement | See Remarks |

Remarks

Conditions are tested with the following logical operators:

< less than or equal to

> greater than

= equal to

<= less than or equal to

>= greater than or equal to

<> not equal

Bit wise operators | and & can be used to evaluate multiple conditions.

A true condition = 1 and a false condition = 0.

Each condition must be placed in parenthesis for proper evaluation by the controller.

'Galil DMC Code Example

IF((var0=1)&(var1=2));' valid IF statement

IF var0=1&var1=2;'      invalid IF statement

IF (var0=1&var1=2);'    invalid IF statement

Examples

'Galil DMC Code Example

#A

IF (_TEA<1000);'  IF conditional statement based on a motor position

MG "Motor is within 1000 counts of zero";' Message to be executed for true

ENDIF;'           End of IF conditional statement

EN;'              End Program

'Galil DMC Code Example

#input

IF (@IN[1]=0);'          IF conditional statement based on input 1

MG "Input 1 is Low";'   Message to be executed if "IF" statement is true ENDIF;'                  End of IF conditional statement

EN

'Galil DMC Code Example #var v1=@AN[1]*5;' some calculation for variable v1 IF((v1>25)&(@IN[4]=1));' Conditions based on V1 variable and input 4 status MG "Conditions met";' Message to be executed if "IF" statement is true ENDIF;' End of IF statement EN

'Galil DMC Code Example REM The conditions of an if statement can be simplied with the fact that REM a true condition = 1 and a false condition = 0. #true v1=1 IF (v1) MG "True v1=",v1 ENDIF #false v1=0 IF (v1) 'if statement evaluates false ELSE MG "False v1=",0 ENDIF EN

IF applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

IF

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

##### IF

ETHERNET IH Open IP Handle

### IHm= n0,n1,n2,n3 <o >p IHm= n <o >p IHm= >p

| Usage | IHm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
| Operands | _IHm0 _IHm1 _IHm2 _IHm3 _IHm4 | Operand has special meaning, see Remarks |

Description

The IH command is used when the controller is operated as a master (client) to open a handle and connect to a slave (server).

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to assign connection |  |
|  | S | T | N/A | Handle | Special handle designator used when closing handles | See Remarks |
| n0 | 0 | 255 | 0 | 1 | Byte 3 of the slave IP address |  |
| n1 | 0 | 255 | 0 | 1 | Byte 2 of the slave IP address |  |
| n2 | 0 | 255 | 0 | 1 | Byte 1 of the slave IP address |  |
| n3 | 0 | 255 | 0 | 1 | Byte 0 of the slave IP address |  |
| n4 | - 2,147,483,648 | 2,147,483,647 | 0 | 1 | Slave IP address as a 32 bit value |  |
| o | 0 | 65,535 | see Notes | 1 | Specify the slave port to connect over | If o is omitted, the controller selects the port starting at 1000 |
| p | 1 | 2 | 2 | 1 | Specify the connection type to open | n = 2 is TCP. n = 1 is UDP. |
|  | -3 | -1 | N/A | 1 | Specify the connection type to close when closing a handle | See Remarks |

Remarks

All 4 bytes must be assigned for an IP address to be valid.

IHm=? returns the IP address as 4, 1-byte numbers.

Use the following equation to change the 4 byte IP (n0,n1,n2,n3) to a single 32 bit number, n.

n = (n0*2^24) + (n1*2^16) + (n2*2^8) + n3.

When using Modbus, port 502, note that Galil Modbus supports one master per slave.

#### Opening a Handle

To open a handle, the user must specify:

The IP address of the slave.

(optional) The port number of the slave. If not specified, the firmware will choose a port.

(optional)The connection type as TCP/IP or UDP/IP. If not specified, the controller will make a TCP connection.

Issue the IH command on an available handle with the correct settings for IP (n0-n3), port (o) and connection type (p). See TH to list handle status.

Modbus connections must always be specified as port 502.

#### Closing a Handle

Closing multiple handles is done with the T handle identifier along with a connection type p selector.

IHT => p closes all handles matching the p type selector, where p = -1 closes UDP handles, p = -2 closes TCP handles and p = -3 closes all handle types.

Closing individual handles, other than the handle being used to send the IH command, is done with IHN => -1 where n is the handle to close (A-H).

Closing the handle that sent the command is done with the S handle identifier, along with connection type p selector.

IHS => p closes the handle that sent the command if its type matches the p selector,where p = -1 closes UDP handles, p = -2 closes TCP handles and p = -3 closes all handle types.

#### Operand Usage

| Operand | Reported Value | Description of Value | Notes |
| --- | --- | --- | --- |
| _IHm0 | -2147483648 to 2147483648 | IP address of handle m as a 32 bit number (n) |  |
| _IHm1 | 0 to 65535 | Slave port number for handle m |  |
| _IHm2 | 0 | Handle is free | Handle 'Available' in TH |

#### IH

|  | 1 | Handle connected as UDP slave |  |
| --- | --- | --- | --- |
|  | 2 | Handle connected as TCP slave |  |
|  | -1 | Handle connected as UDP master |  |
|  | -2 | Handle connected as TCP master |  |
|  | -5 | Attempting to establish UDP handle |  |
|  | -6 | Attempting to establish TCP handle |  |
| _IHm3 | 0 | ARP was successful |  |
|  | 1 | ARP failed or still in progress |  |
| _IHm4 | 1 | Waiting for ACK from slave controller after issuing a command |  |
|  | 2 | Received ":" as response to a command |  |
|  | 3 | Received "?" as response to a command |  |
|  | 4 | Connection timed-out waiting for a response to a command |  |

Related Commands

DH - DHCP Client Enable

IA - IP Address

TH - Tell Ethernet Handle

WH - Which Handle

Examples

'Galil DMC Code Example

IHA=251,29,51,1;'     Open handle A at IP address 251.29.51.1

'TCP is used as default

IHA= -2095238399;'    Open handle A at IP address 251.29.51.1

'When the IH command is given,

'the controller initializes an ARP

'on the slave device before opening a handle.

'This operation can cause a small time delay

'before the controller responds

'Galil DMC Code Example

'setting up a modbus handle

MW1;'                        setup modbus wait

IHE= 192,168,100,200<502>2;' setup a modbus handle to slave

#wt;'                        wait for handle to be connected

WT2;'                        before issuing a command JP#wt,_IHE2<>-2;'

SB5003;'                     Set output 3 on slave

WT1000;'                     1 second wait

MBE= ,5,3,0;'                Clear output 3 using MB command

EN

IH applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

IH

IO

#### II Input Interrupt

II n

0

,n

1

,n

2

,n

3

II n

0

,n

1

,n

2

,n

3

| Usage | II n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The II command enables the input interrupt function for the specified inputs.

The II command is used along with the #ININT subroutine to execute specific code when inputs specified by II are in the desired state.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 |  | 8 | 0 | 1 | Lowest input to use for interrupt trigger | n0=0 disables input interrupt |
| n1 | 1 |  | 8 | N/A | 1 | Highest input to use for interrupt trigger | n1 must be >= n0, If omitted n1=n0 |
| n2 | 1 |  | 255 | N/A | 1 | Use bitmask as alternative selection of input interrupt triggers | If n0 and n1 are used, n2 is ignored, see Remarks |
| n3 | 0 |  | 255 | 0 | 1 | Bitmask specifying required input state for interrupt trigger | Default=interrupt triggers on low inputs, see Remarks |

Remarks

The argument n2 is a bitmask for the inputs selected for the input interrupt function. This field is ignored if n0 and n1 are used.

For example, if n2 = 15, the binary equivalent is 00001111. This means that inputs 1-4 would be selectd by the II function, and 5-8 would not be.

This argument n3 is bistmask showing which state the input must be in for the II function to trigger.

For example, if n0=1 and n1=4, the inputs 1,2,3 and 4 have been activated. If the value for n3 is 2 (the binary equivalent of 2 is 00000010), then input 2 must be a '1' and inputs 1,3, and 4 must be a "0", for II to trigger the #ININT subroutine. The RI command is used to return from the #ININT routine.

Examples

'Galil DMC Code Example

II 1,1,,0;'                 Specify interrupt on input 1 only, and triggers when input 1 = 0. EN;'                        End Program

#ININT;'                   Interrupt subroutine

'                           The code the user wants to run when II triggers goes here.

WT100;'                     Debounce the input.

RI 1;'                      Return to main program, re-enabling trip point.

'                           Specify RI 0 if it is not desired to re-enable trip points.

II applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

II

ETHERNET

#### IK Block Ethernet ports

IK n

IK n

| Usage | IK n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The IK command blocks client connections to the controller on most ports below port number 1000. Specific port numbers and ports above 1000 are unaffected.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Allow controller to receive Ethernet packets on any port |  |
|  | 1 | Blocks Ethernet packets on ports lower than 1000. | Default. Ports 0,23,68, and 502 are unaffected. |

Remarks

A Galil Ethernet controller simultaneusly operates as a server (listening for Ethernet connections from a client) and a client (able to create connections to a server).

Ports 0, 23, 68 and 502 are used for standard client connections to the controller.

Examples

'Galil DMC Code Example

:IK1;' Blocks undesirable port communication :IK0;' Allows all Ethernet ports to be used :

IK applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

IK

FILTER/CONTROL

#### IL Integrator Limit

ILm= n

IL n,n,n,n,n,n,n,n

ILm= n

IL n,n,n,n,n,n,n,n

| Usage | ILm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | IL n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _ILm | Operand holds the value last set by the command |

Description

The IL command limits the effect of the integrator gain in the filter to a certain voltage.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -9.9982 | 9.9982 | 9.9982 | 20/65,536 | Value of Integrator limit in volts | n< 0 (negative value) freezes the effect of the integrator during the move |

Remarks

IL is the absolute value of the integrator limit. For example:

ILA= 2 limits the output of the integrator of the A-axis to the +/-2 Volt range.

KD and KP terms remain active in any case. The output from the KD and KP terms is not affected.

A negative parameter will freeze the effect of the integrator during the move. For Example:

ILA= -3 limits the integrator output of the A axis to +/-3V but freezes the contribution of the Integrator loop during motion.

If, at the start of the motion, the integrator output is 1.6 Volts, that level will be maintained through the move and the integrator will not accumulate during the move.

Once the profiled move has completed (RP has reached final commanded position), the integrator loop will be enabled.

When using the -CER firmware, the default value of IL is -9.9982.

Examples

'Galil DMC Code Example

KI 2,3,5,8;' Integrator constants

IL 3,2,7,2;' Integrator limits

IL ?;'   Returns the A-axis limit

IL applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

IL

INDEPENDENT MOTION

#### IP Increment Position

IPm= n

IP n,n,n,n,n,n,n,n

IPm= n

IP n,n,n,n,n,n,n,n

| Usage | IPm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | IP n ... | Arguments specified with an implicit, comma-separated order |

Description

The IP command allows for a change in the command position while the motor is moving. This command does not require a BG.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -2,147,483,648 | 2,147,483,647 | N/A | 1 | Value of incremental move |  |

Remarks

_IPm contains the current position of the motor

The IP command has four effects depending on the mode of motion being executed. IP operation based upon modes of motion

| Case | Equivalent Commands | Description |
| --- | --- | --- |
| Motor is standing still | IPm=n Equivalent to PRm=n;BGm | Motor will move to specified position with the predefined AC,DC,SP values. |
| Motor is moving toward position n | PRm=n0; BGm;IPm=n1 Equivalent to PRm= (n0+n1); BGm | Motor will move a relative move of (n0+n1). |
| Motor is in Jog Mode | JGm=n0;BGm;IPm=n1 Equivalent to Continuing jog from (current position + n1) | The motor will instantly try to servo to a position which is the current instantaneous position plus the specified IP position. SP and AC parameters have no effect. This command is useful when synchronizing 2 axes in which one of the axis' speed is indeterminate due to a variable diameter pulley. |
| Motor is a slave in gearing mode | GAm= m0; GRm=n0; IPm=n1 Equivalent to GAm= m0; GRm=n0; PRm=n1; BGm | The motor will move with the predefined AC,DC,SP values superimposed on top of the existing gearing motion. |

Examples

'Galil DMC Code Example

IP 50;'  50 counts with set acceleration and speed

#CORRECT;' Label

AC 100000;' Set acceleration

JG 10000;BGA;' Jog at 10000 counts/sec rate

WT 1000;' Wait 1000 msec

IP 10;'   Move the motor 10 counts instantaneously

STA;'    Stop Motion

EN

IP applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

IP

INDEPENDENT MOTION, SYSTEM CONFIG

#### IT Independent Time Constant - Smoothing Function

ITm= n

IT n,n,n,n,n,n,n,n

ITm= n

IT n,n,n,n,n,n,n,n

| Usage | ITm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | IT n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _ITm | Operand holds the value last set by the command |

Description

The IT command filters the acceleration and deceleration functions of independent moves such as JG, PR, PA to produce a smooth velocity profile. The resulting profile, known as smoothing, has continuous acceleration and results in reduced mechanical vibrations. IT sets the bandwidth of the filter where 1 means no filtering and 0.004 means maximum filtering.

The IT command also filters the individual axes during Vector Mode (VM) and Linear Interpolation Mode (LM).

Arguments

| Argument | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A | H |  | N/A | Axis | Axis to assign value |  |
| n | 0.004 | 1 |  | 1 | 1/256 | Value of independent smoothing function | 1 = no filtering, 0.004 = maximum filtering |

Remarks

The IT filtering results in longer motion time.

The use of IT will not effect the trippoints AR and AD.

The trippoints AR & AD monitor the profile prior to the IT filter and therefore can be satisfied before the actual distance has been reached if IT is NOT 1.

Details on the IT filtering can be found in Application Note #3412 https://www.galil.com/download/application-note/note3412.pdf

Examples

'Galil DMC Code Example :IT 0.8, 0.6, 0.9, 0.1;' Set independent time constants for a,b,c,d axes :IT ?;' Return independent time constant for A-axis 0.8000

'Galil DMC Code Example REM example showing increased time due to IT filtering #move IT 1 t=TIME;'store time reference PR 1000 BGA;AMA MG TIME-t;'display move time IT 0.01 t=TIME;'store time reference PR 1000 BGA;AMA MG TIME-t;'display move time EN :'program execution output :XQ : 508.0000 1112.0000 :

IT applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

IT

INDEPENDENT MOTION

JG Jog

### JGm= n JG n,n,n,n,n,n,n,n

| Usage | JGm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | JG n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _JGm | Operand has special meaning, see Remarks |

Description

The JG command sets the jog mode and the jog slew speed of the axes.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
|  | N | N | N/A | Axis | Virtual axis to assign value |  |
| n | - 15,000,000 | 15,000,000 | 25,000 | 2 | Value of jog speed in cnts/second | For MT settings of 1,-1,1.5 and -1.5 (Servos) |
|  | -3,000,000 | 3,000,000 | 25,000 | 2 | Value of jog speed in cnts/second | For MT settings of 2,-2,2.5 and -2.5 (Steppers) |

Remarks

When jogging, the motion controller profiles a continuous move at the commanded speed. To stop the motion, use the ST command.

JG 2 is the minimum non-zero speed

_JGm contains the absolute value of the jog speed for the specified axis.

The JG command will set the SP register with the absolute value of the 'n' value.

#### Resolution

The resolution of the JG command is dependent upon the update rate setting (TM).

With the default rate of TM 1000 the resolution is 2 cnts/second.

The equation to calculate the resolution of the JG command is:

resolution = 2*(1000/TM)

example:

With TM 250 the resolution of the JG command is 8 cnts/second resolution = 2*(1000/250) = 8

Examples

'Galil DMC Code Example #jg JG 100,500,2000,5000 ' Sets for jog mode with a slew speed of 100 counts/sec for the A-axis, ' 500 counts/sec for the B-axis, ' 2000 counts/sec for the C-axis, ' and 5000 counts/sec for D-axis. BG;' Begin Motion WT 1000;' Wait one second JG ,,-2000;' Change the C-axis to slew in the negative direction at -2000 counts/sec. EN

JG applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### JG

PROGRAMMING JP Jump to Program Location

### JP #str,(ex) JP n,(ex)

| Usage | JP n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The JP command causes a jump to a program location on a specified condition. The program location may be any program line number or label. A jump is taken if the specified condition is true. Multiple conditions can be used in a single jump statement.

JP can be used for relative jumps and for jump tables, see Examples.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 7 chars | N/A | String | Label name for jump destination | Must be a valid label in application code |
| n | 0 | see Notes | N/A | 1 | Line number for jump destination | Maximum is number of lines of controller program memory - 1 |
| ex | N/A | N/A | N/A | Expression | Conditional statement/s that must evaluate true for jump to occur | If omitted, JP automatically evaluates as true |

Remarks

The logical operators that can be used in the conditional statement are: < less than

> greater than

= equal to

<= less than or equal to

>= greater than or equal to

<> not equal to

The conditional statements are combined in pairs using the operands "&" and "|".

The "&" operand between any two conditions requires that both statements must be true for the combined statement to be true. The "|" operand between any two conditions requires that only one statement be true for the combined statement to be true. Each condition must be placed in parentheses for proper evaluation by the controller.

'Galil DMC Code Example

REM Use of parentheses

JP#a,((var0=1)&(var1=2));' valid conditional jump

JP#a,var0=1&var1=2;'       invalid conditional jump

Examples

'Galil DMC Code Example

JP #POS1,(V1<5);'   Jump to label #POS1 if variable V1 is less than 5

JP #A,((V7*V8)=0);' Jump to #A if V7 times V8 equals 0

JP #B,(@IN[1]=1);'  Jump to #B if input 1 = 1

JP #C;'             Jump to #C unconditionally

#### Jump Table

'Galil DMC Code Example REM Example of jumping to a label plus an offset REM #error is a subroutine that prints an error REM message based on the value of an error REM variable, ecode #a REM Set error code and then JS to sub ecode = 1 JS #error ecode = 3 JS #error ecode = 56;' bad error code JS #error EN ' '*************** 'Example of a Jump table #error REM First check that ecode is valid IF (ecode < 0) ecode = 4 ENDIF IF (ecode > 4) ecode = 4 ENDIF REM Call the helper label with an offset JP#error_h + ecode

#### JP

'CRITICAL! Do not change line ' spacing in following text #error_h;MG "No error, zero";EN MG "Error code 1, foo";EN MG "Error code 2, bar";EN MG "Error code 3, baz";EN MG "Invalid error code";EN REM ecode indexes the line to execute REM above, relative to #error_h REM REM Returned messages: REM Error code 1, foo REM Error code 3, baz REM Invalid error code

##### Relative Jump

'Galil DMC Code Example REM A loop for delaying 1000 samples (~ 1 sec) REM sample time MG "Relative jump" t=TIME REM print sampled time MG t REM loop until TIME increments 1000 samples REM _XQ0-1 points back to the beginning of the line JP _XQ0-1,(TIME < (t+1000)) REM print current time MG TIME REM This is NOT thread safe as REM _XQ0 refers to thread 0 only REM For easier readability and stability, use labels REM wherever possible MG "Label-based jump" t=TIME MG t #wait JP#wait, (TIME < (t+1000)) MG TIME REM Also, where possible use trippoints MG "Trippoint" t=TIME MG t WT 1000;' see WT for units MG TIME EN REM Relative jump REM 3459.0000 REM 4459.0000 REM Label-based jump REM 4461.0000 REM 5461.0000 REM Trippoint REM 5463.0000 REM 6464.0000

JP applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

JP

PROGRAMMING JS Jump to Subroutine

### JS #str(arg,arg,arg,arg,arg,arg,arg,arg), (ex) JS n(arg,arg,arg,arg,arg,arg,arg,arg), (ex)

| Usage | JS n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _JS | Operand has special meaning, see Remarks |

Description

Allows the program to jump to a subroutine and return back after completion. This command is often used to call reusable code.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 7 chars | N/A | String | Label Name for jump destination | Must be a valid label in application code |
| n | 0 | 3,999 | N/A | 1 | Line number for jump destination | Firmware Rev 1.2a and later. May be a value or a variable, but not an evaluated statement with parenthesis |
| n | 0 | 1,999 | N/A | 1 | Line number for jump destination | May be a value or a variable, but not an evaluated statement with parenthesis |
| ex | N/A | N/A | N/A | N/A | Conditional statement/s that must evaluate true for jump to occur | If omitted, the jump is taken |
| arg | N/A | N/A | N/A | N/A | A value, variable, or array to pass to the subroutine being called | referenced from within the subroutine as ^a-^h, respectively. See Remarks for a table of valid args |

Remarks

JS can be nested, called up to 16 deep

When used after JS is called, the _JS operand contains the returned value of the subroutine called by JS

#### Basic Usage

The JS command will change the sequential order of execution of commands in a program

If the jump is taken, program execution will continue at the line specified by the destination parameter, which can be either a line number or label. A variable holding a line number or an expression resulting in the calculation of a line number can also be used

The line number of the calling JS command is saved and after an EN command is encountered (End of subroutine), program execution will continue with the instruction following the calling JS command.

A jump is taken if the specified condition is true. Each condition must be placed in parenthesis for proper evaluation by the controller.

Code flexibility/reuse. A single subroutine can be written and called many times and from various locations in code. The stack "remembers" where to return when completed. This is opposite from a "blind jump" (JP).

Conditional Syntax

| Condition | Validity |
| --- | --- |
| JS#A,(var1=0)&(var2=1) | This conditional statement is valid |
| JS#A,var1=0&var2=1 | This conditional statement is not valid |

#### Passing Values on the Stack

Parameters can be passed on the subroutine stack

Passing parameters in a subroutine has many advantages including the following

Variable Scope/ Local variables. A subroutine can run with a protected variable space. Local variables exist only in the extent of the subroutine, and no external thread or stack level can access local variables. Local variables can be used for counters, indices, and other helper variables Each thread has its own stack, therefore subroutines are reentrant. In other words, multiple threads can be running the same subroutine simultaneously at various stack depths.

Support for recursion. Although the subroutine stack is only 16 deep, recursion is possible. A stack depth of 16 is sufficient for many recursive tasks. E.G. recursing axes, handles, and thread status.

Parameter passing. A calling command can explicitly specify the inputs to a subroutine. The subroutine can pass one value back to the calling command. More returns are possible with pass by reference and array passing.

Constants, Variables, and Arrays may be passed up a subroutine stack.

Variables may be passed by value or by reference. If passed by value, a copy is made in the subroutine stack, leaving the original variable immutable. If passed by reference, the original variable's value will be changed when the subroutine writes to its local variable. This is similar, but not exactly analogous to a C pointer.

A variable passed by reference is automatically dereferenced; the variable pointer is not exposed to the user. Following the C syntax, a by-reference pass is accomplished with the ampersand (&) in the invoking call.

IMPORTANT NOTE: When passing a variable by reference, do not allocate any new variables in the called subroutine.

Arrays can be passed in the stack, though only by reference. No "&" is used when passing arrays, by-reference is assumed. To pass an array, use its name in quotations.

IMPORTANT NOTE: Arrays to be passed must have names that are 6 characters or less. The number of elements in an array is returned by reading index -1, e.g. array[-1].

To return a value on the stack, write the value in the EN command upon ending the subroutine. The parent stack can access this value via _JS. Examples of valid args (see examples for demo of each concept)

| What to pass | arg | Example |
| --- | --- | --- |
| Value | the value | JS #square(7) |
| Variable's value | variable name | JS #sub(var) |
| Variable by reference | ampersand + variable name | JS #sub(&var) |
| Array by reference | array name in quotes | JS#sub("array") |

Examples

'Galil DMC Code Example REM Example of pulsing an output pulse=0 JS#pulse,(pulse > 0);'JS not taken WT 2000 pulse=3 JS#pulse,(pulse > 0);'JS taken WT 2000 pulse=5 JS#pulse;'unconditionally take jump EN ' ' REM Subroutine called after REM setting pulse variable #pulse SB 1;' set bit 1 WT 500;' delay 500 ms CB 1;' clear bit 1 WT 500;' delay 500 ms pulse=pulse-1;' decrement pulse JP#pulse,pulse>0;' continue till zero EN;' return to calling JS

#### Advanced Usage Examples

'Galil DMC Code Example REM Run all examples #all JS#val JS#var JS#varref JS#array EN REM Example for each way to pass to a subroutine REM ******************************************** REM Pass a Value #val JS #square(3) MG _JS EN #square REM Return the passed value squared EN ,,(^a*^a) REM ******************************************** REM Pass a variable's value #var val= 7 REM call the same sub above JS #square(val) MG _JS EN REM ******************************************** REM Pass a variable by reference #varref val= 9 JS #square2(&val) MG val EN #square2 REM change the value of the variable ^a= ^a*^a REM don't return anything EN REM ******************************************** REM Pass an array by reference #array DM array[100] array[42]= 11 JS #square3("array") MG array[42] EN #square3 REM change the array element ^a[42]= ^a[42]*^a[42] REM don't return anything EN

REM ******************************************** REM Controller Response REM :XQ#all REM : REM 9.0000 REM 49.0000 REM 81.0000 REM 121.0000

'Galil DMC Code Example #ADD JS#SUM(1,2,3,4,5,6,7,8);' Call subroutine, pass values MG_JS;' Print return value, will print 36.0000 EN #SUM;' Sums values passed to it. Expects 8 numbers EN,,^a+^b+^c+^d+^e+^f+^g+^h;' Return the sum

'Galil DMC Code Example 'Dimension two arrays DM array1[10] DM array2[100] 'Zero the contents of each array JS#ZeroAry("array1", 0) JS#ZeroAry("array2", 0) EN 'Zero the contents of an array #ZeroAry;'(^a array,^b starting index) ^a[^b]=0 ^b=(^b+1) JP#ZeroAry,(^b < ^a[-1]) EN

'Galil DMC Code Example REM Using dynamic destinations in a jump table i=1;' Counter #loop offset=#spell+i;' Calculate offset JS offset;' Jump to offset i=i+1;' Increment Counter JP#loop,i<=3;' Loop through 3 states EN ' #spell;' Subroutine containing various words MG"One";EN;' Prints "One" if this line is called (i=1) MG"Two";EN;' Prints "Two" if this line is called (i=2) MG"Three";EN;' Prints "Three" if this line is called (i=3) REM Controller responds with: REM One REM Two REM Three

JS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### K1 Proportional gain during motion

K1m= n

K1 n,n,n,n,n,n,n,n

K1m= n

K1 n,n,n,n,n,n,n,n

| Usage | K1m= n |
| --- | --- |
|  | K1 n ... |
| Operands | _K1m |

Description

K1 is the proportional gain in effect when the profiler is commanding motion (RP is changing). When no motion is commanded (RP constant), KP is in effect. Some systems will oscillate when holding position unless the gains are lowered.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 1,023.875 | 6 | 1/8 | Value of proportional term |  |

Remarks

Valid only in -NANO and -CER firmware

Examples

'Galil DMC Code Example

K1X=10;' set X axis P gain in effect during motion

K2X=1;'  set X axis I gain in effect during motion

K3X=100;'set X axis D gain in effect during motion

KPX=6;'  set X axis P gain in effect when holding position

KIX=0;'  set X axis I gain in effect when holding position

KDX=64;' set X axis D gain in effect when holding position

K1 applies to CER,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

K1

#### K2 Integratorgain during motion

K2m= n

K2 n,n,n,n,n,n,n,n

K2m= n

K2 n,n,n,n,n,n,n,n

| Usage | K2m= n |
| --- | --- |
|  | K2 n ... |
| Operands | _K2m |

Description

K2 is the integral gain in effect when the profiler is commanding motion (RP is changing). When no motion is commanded (RP constant), KI is in effect. Some systems will oscillate when holding position unless the gains are lowered.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 255.999 | 0 | 1/1,024 | Value of integrator term |  |

Remarks

Valid only in -NANO and -CER firmware

Examples

'Galil DMC Code Example

K1X=10;' set X axis P gain in effect during motion

K2X=1;'  set X axis I gain in effect during motion

K3X=100;'set X axis D gain in effect during motion

KPX=6;'  set X axis P gain in effect when holding position

KIX=0;'  set X axis I gain in effect when holding position

KDX=64;' set X axis D gain in effect when holding position

K2 applies to CER,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

K2

#### K3 Derivativegain during motion

K3m= n

K3 n,n,n,n,n,n,n,n

K3m= n

K3 n,n,n,n,n,n,n,n

| Usage | K3m= n |
| --- | --- |
|  | K3 n ... |
| Operands | _K3m |

Description

K3 is the derivative gain in effect when the profiler is commanding motion (RP is changing). When no motion is commanded (RP constant), KD is in effect. Some systems will oscillate when holding position unless the gains are lowered.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 4,095.875 | 64 | 1/8 | Value of derivative term |  |

Remarks

Valid only in -NAN and -CER firmware

Examples

'Galil DMC Code Example

K1X=10;' set X axis P gain in effect during motion

K2X=1;'  set X axis I gain in effect during motion

K3X=100;'set X axis D gain in effect during motion

KPX=6;'  set X axis P gain in effect when holding position

KIX=0;'  set X axis I gain in effect when holding position

KDX=64;' set X axis D gain in effect when holding position

K3 applies to CER,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

K3

#### KD DerivativeConstant

KDm= n

KD n,n,n,n,n,n,n,n

KDm= n

KD n,n,n,n,n,n,n,n

| Usage | KDm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | KD n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _KDm | Operand holds the value last set by the command |

Description

KD designates the derivative constant in the control filter. The derivative gain outputs a voltage based on the rate of change of the error. The filter transfer function follows:

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 4,095.875 | 64 | 1/8 | Value of derivative term |  |

Remarks

n=? will return the currently set value of KD m=* will set the KD value for all axes/channels

For further details see the section "Theory of Operation" in the controller user manual.

Examples

'Galil DMC Code Example :KD 12,14,16,20;' Implicit notation to set A,B,C,D axis derivative term :KDC= 8;' Explicit notation to set C :KD ,,8;' Implicit notation to set C :KD ?,?,?,?; ' Return A,B,C,D values 12, 14, 8, 20 :KDC= ?;' Return C value 8 :MG _KDA;' Message the operand for the A axis 12 :

'Galil DMC Code Example

REM Zeroing the PID filter allows the

REM motor command signal to be

REM used as a programmable DAC

KI*= 0;'                 Zero KI

KP*= 0;'                 Zero KP

KD*= 0;'                 Zero KD

ER -1,-1;'               Rurn off position error limit

OF 1,2;'                 Set one volt on A and two volts on B

EN

KD applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

KD

#### KI Integrator

KIm= n

KI n,n,n,n,n,n,n,n

KIm= n

KI n,n,n,n,n,n,n,n

| Usage | KIm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | KI n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _KIm | Operand holds the value last set by the command |

Description

The KI command sets the integral gain of the control loop. The integrator term will reduce the position error at rest to zero. It fits in the control equation as follows:

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 255.999 | 0 | 1/1,024 | Value of Integral term |  |

Remarks

n=? will return the currently set value of KI m=* will set the KI value for all axes/channels

For further details see the section "Theory of Operation" in the controller user manual.

Examples

'Galil DMC Code Example

:KIC= 8;'         Explicit notation to set C

:KI ,,8;'         Implicit notation to set C

:KI ?,?,?,?; '    Return A,B,C,D values

7, 14, 8, 20

:KIC= ?;'         Return C value

8

:MG _KIA;'        Message the operand for the A axis  7 :

'Galil DMC Code Example

REM Zeroing the PID filter allows the

REM motor command signal to be

REM used as a programmable DAC

KI*= 0;'               Zero KI

KP*= 0;'               Zero KP

KD*= 0;'               Zero KD

OF 1,2;'               Set one volt on A and two volts on B

EN

KI applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,EDD37010,RIO47000,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

KI

#### KP Proportional Constant

KPm= n

KP n,n,n,n,n,n,n,n

KPm= n

KP n,n,n,n,n,n,n,n

| Usage | KPm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | KP n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _KPm | Operand holds the value last set by the command |

Description

KP designates the proportional constant in the controller filter. The proportional gain outputs a control signal proportional to the amount of error. The filter transfer function follows.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 1,023.875 | 6 | 1/8 | Value of proportional term |  |

Remarks

n=? will return the currently set value of KP

For further details see the section "Theory of Operation" in the controller user manual.

Examples

'Galil DMC Code Example :KP 12,14,16,20;' Implicit notation to set a,b,c,d axis proportional term :KPC= 8;' Explicit notation to set C :KP ,,8;' Implicit notation to set C :KP ?,?,?,?; ' Return A,B,C,D values 7, 14, 8, 20 :KPC= ?;' Return C value 8 :MG _KPA;' Message the operand for the A axis 12 :

'Galil DMC Code Example

REM Zeroing the PID filter allows the

REM motor command signal to be

REM used as a programmable DAC

KI*= 0;' Zero KI

KP*= 0;' Zero KP

KD*= 0;' Zero KD

OF 1,2;' Set one volt on A and two volts on B

EN

KP applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,EDD37010,RIO47000,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

KP

STEPPER MOTOR

#### KS StepMotor Smoothing

KSm= n

KS n,n,n,n,n,n,n,n

KSm= n

KS n,n,n,n,n,n,n,n

| Usage | KSm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | KS n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _KSm | Operand holds the value last set by the command |

Description

The KS parameter sets the amount of smoothing of stepper motor pulses. Larger values of KS provide greater smoothness. KS adds a single pole low pass filter onto the output of the motion profiler.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0.5 |  | 64 | 2 | 1/32 | Value of smoothing constant |  |

Remarks

This is most useful when operating in full or half step mode.

KS effect on timing:

This parameter will increase the time to complete a motion time by 3KS sampling periods. KS will cause an overall delay in the generation of output steps.

Examples

'Galil DMC Code Example

:KSC= 8;'         Explicit notation to set C

:KS ,,8;'         Implicit notation to set C

:KS ?,?,?,?; '    Return A,B,C,D values

7, 14, 8, 20

:KSC= ?;'         Return C value

8

:MG _KSA;'        Message the operand for the A axis  7 :

KS applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

##### KS

PROGRAMMING, INTERROGATION

###### LA   List Arrays

LA

LA

| Usage | LA | Command takes no arguments |
| --- | --- | --- |

Description

The LA command returns a list of all arrays in memory. The size of each array will be included next to each array name in square brackets.

Arguments

LA is an interrogation command with no parameters

#### Remarks

The listing will be in alphabetical order.

#### Examples

'Galil DMC Code Example :DM gold[100],silver[50],plat[200];' Dimensions arrays with given name and the number of array elements in square brackets :LA;' Commands the controller to list arrays in alphabetical order gold[100] plat[200] silver[50] :DA *[];' Dialocates all arrays :LA;' List arrays now returns with no arrays :

LA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LA

STEPPER MOTOR

##### LC LowCurrent Stepper Mode

LCm= n

LC n,n,n,n,n,n,n,n

LCm= n

LC n,n,n,n,n,n,n,n

| Usage | LCm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | LC n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _LCm | Operand holds the value last set by the command |

Description

The LC command enables low current mode for stepper motors. Low current mode reduces the holding current of the stepper motors while at rest.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value | See Timing Diagram Below for behavior based on value of n |
| n | 2 |  | 32,767 | 0 | 1 | Waits for n samples after a move is completed, then provides 0% current |  |
|  | 1 |  | 1 | 0 | 0 | Provides 25% current immediately after a move is completed |  |
|  | 0 |  | 0 | 0 | 0 | Always provides 100% current |  |
|  | -1 |  | -32,767 | 0 | 1 | Waits for n samples after a move is completed, then provides 25% current |  |

###### Low Current Mode Timing

Low Current Mode Timing Diagram

Remarks

The MT command must be issued prior to the LC command.

Using LC with an internal Galil Stepper drive (SDM).

A setting of LC 0 is required to to shut off all current to the motor in the "motor off" (MO) state.

Using LC will reduce current consumption, but there will be a reduction of holding torque at rest. Consult the user manual for more details regarding your specific amplifier. Using LC with external amplifiers.

When using external amplifiers low current mode will simply disable the motors by toggling the amplifier enable line during rest.

Related Commands

AG - Amplifier Gain

BG - Begin

MT - Motor Type

###### LC

SH - Servo Here

Examples

'Galil DMC Code Example

#ex

MTA=-2;'specify stepper mode for A axis

LCA=15;'specify motor to go to low current

' 15 samples after motion has completed

EN

LC applies to DMC50000,DMC4000,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LC

ERROR CONTROL

##### LD LimitDisable

LDm= n

LD n,n,n,n,n,n,n,n

LDm= n

LD n,n,n,n,n,n,n,n

| Usage | LDm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | LD n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _LDm | Operand holds the value last set by the command |

Description

Allows user to disable forward and/or reverse limit switches.

Arguments

| Argument |  | Min |  | Max | Default | Resolution |  | Description |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis |  | Axis to assign value |  |  |
| n | 0 |  | 3 |  | 0 | 1 |  | Sets limit disable state |  | See table below for details |
| Argument | Value |  | Description |  |  |  | Notes |  |
| n | 0 |  | Both limit switches are enabled |  |  |  | Default |  |
|  | 1 |  | Forward limit switch disabled |  |  |  |  |  |
|  | 2 |  | Reverse limit switch disabled |  |  |  |  |  |
|  | 3 |  | Both limit switches disabled |  |  |  |  |  |

Remarks

n = ? will return the current setting of LD When this feature should be used:

To gain additional digital inputs if limit switches are not being utilized.

To prevent noise from causing the limit switchs conditions even though no limit switches are connected. LD does not disable software limits set by BL and FL.

Examples

'Galil DMC Code Example :LD 3,1,2; 'Implicit notation to set channel A, B, and C :MG _LDA;' 'Message the operand for the A channel 3.0000 :LDC= 3;' 'Explicit notation to set channel C-only :LD*= ?;' 'Queries the value of LD for all channels 3, 1, 3, 0 :

'Galil DMC Code Example REM use forward limit switch as an extra I/O point #io LDA=1;'disable forward limit switch io=_LFA;'set state of limit switch to variable "io" 'Use "io" in an IF statement IF io=1 MG "Input On" ELSE MG "Input Off" ENDIF EN

LD applies to DMC4000,DMC4200,DMC4103,DMC1806,DMC30010,DMC50000,DMC52000,EDD37010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LD

VECTOR/LINEAR

##### LE   Linear Interpolation End

LE

LE?

LE

LE?

| Usage | LE | Command takes no arguments |
| --- | --- | --- |
| Operands | _LEm | Operand has special meaning, see Remarks |

Description

The LE command indicates to the controller that the end of the sequence is coming up. This allows the controller to slow down through multiple segments, if required. LE is required to exit the linear interpolation mode gracefully (stop code, SC, 101).

Arguments

The LE command has no arguments. See the ? Remark below.

Remarks

_LEm will return the total move length in encoder counts for the selected coordinate system, where m is S or T.

If not spcified, the LE command will apply to the last selected coordinate system, S or T.

To select the coordinate system, use the command CA S or CA T.

The VE command is interchangeable with the LE command.

LE ? Returns the total vector move length in encoder counts for the current coordinate system

Examples

'Galil DMC Code Example

CA S;          'Specify S coordinated motion system

LM CD;         'Specify linear interpolation mode for C and D axes

LI ,,100,200;  'Specify linear distance

LE;            'Ends linear interpolation distance

BG S;          'Begin motion of the S-coodrinate system

LE applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

###### LE

VECTOR/LINEAR LI   Linear Interpolation Distance

LIm= n <o >p

LI n,n,n,n,n,n,n,n <o >p

| Usage | LIm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | LI n ... | Arguments specified with an implicit, comma-separated order |

#### Description

The LI command specifies the incremental distance of travel for each axis in the Linear Interpolation (LM) mode. LI parameters are relative distances given with respect to the current axis positions.

#### Arguments

| Argument | Min | Max | Default | Resolution | Description |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |  |  |
| n | -8,388,607 | 8,388,607 | 0 | 1 | Assigns linear interpolation point for that axis |  |  |  |
| o | 2 | 15,000,000 | N/A | 2 | Specifies the vector speed to be commanded at the beginning of the linear segment. The controller will start accelerating or decelerating at the start of the sequence to this speed. |  |  | For MT 1,1,1.5, and 1.5. |
|  | 2 | 3,000,000 | N/A | 2 | Specifies the vector speed to be commanded at the beginning of the linear segment. The controller will start accelerating or decelerating at the start of the sequence to this speed. |  |  | For MT 2,2,2.5, and 2.5. |
| p | 2 | 15,000,000 | N/A | 2 | Specifies the vector speed to be achieved at the end of the linear segment. The controller will decelerate or accelerate during the segment and will reach the specified speed at the end of the segment. |  |  | For MT 1,1,1.5, and 1.5. |
|  | 2 | 3,000,000 | N/A | 2 | Specifies the vector speed to be achieved at the end of the linear segment. The controller will decelerate or accelerate during the segment and will reach the specified speed at the end of the segment. |  |  | For MT 2,2,2.5, and 2.5. |
| Argument | Value | Description |  |  |  | Notes |
| o | -1 | Specifies vector speed to be set by Vector Speed Variable (VV command) |  |  |  | See VV command |

#### Remarks

The CA command is used to set the coordinated system (S or T) for which an LI segment is executed. The default is the S coordinate system (CAS).

The controller always uses the axis specifications from LM, not LI, to compute the speed.

For example: if LM specfies that A-, B-, and C-axis are to be used in linear interpolation mode, but LI only specifies positions for B- and C-, the Aaxis will still be used in calculating the overall vector speed.

The maximum independent speed of any axis configured as a stepper must not exceed the maximum value allowable via the SP setting. The slew speed, set by VS, 'o' or 'p' for linear interpolation mode, is the vector speed based on the axes specified in the LM mode. For example, if LM ABC designates linear interpolation for the A,B and C axes the speed of these axes (Va, Vb, and Vc respectively) will be computed from:

The Linear End (LE) command must be given after the last LI segment in a sequence. LE tells the controller to decelerate to a stop at the last LI command.

The BG S or BG T command should be issued before the total LI distance reaches 1,073,741,824 (2^30) encoder counts.

##### Linear Interpolation Mode Buffer

- Up to 511 LI segments may be given ahead of the begin sequence (BG S or BG T) command.

- Additional LI commands may be sent during motion when the controller sequence buffer frees additional space for new vector segments.

- It is the responsibility of the user to keep enough LI segments in the controller's sequence buffer to ensure continuous motion.

- _LMm (_LMS and _LMT) contains the available spaces for LI segments that can be sent to the buffer.

  - 511 returned means the buffer is empty and 511 LI segments can be sent.

  - A 0 returned means the buffer is full and no additional segments can be sent.

  - See the LM command for full details.

##### LI

#### Examples

'Galil DMC Code Example LM ABC; 'Specify linear interpolation mode between A-, B-, and C- axis LI 500,,400; 'Specifies linear interpolation point, B-axis remains stationary but is still part of the interpolation. LI 1000,2000,3000; 'Specify linear interpolation point LE; 'Last segment of sequence BG S; 'Begin sequence

LI applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LI

INTERROGATION

##### LL List Labels

LL

LL

| Usage | LL | Command takes no arguments |
| --- | --- | --- |

Description

The LL command returns a listing of all of the program labels in memory.

Arguments

LL is an interrogation command with no arguments

Remarks

The LL command label listing will be in alphabetical order.

The LL command returns all of the program labels in memory and their associated line numbers

Examples

'Galil DMC Code Example

:LL

#FIVE=5

#FOUR=4

#ONE=1

#THREE=3

#TWO=2

LL applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LL

VECTOR/LINEAR

##### LM   Linear Interpolation Mode

LM mm

LM mm

| Usage | LM mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _LMm | Operand has special meaning, see Remarks |

Description

The LM command specifies the linear interpolation mode and specifies the axes for linear interpolation.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | N/A | Multi-Axis Mask | Axes to use for linear interpolation mode |  |

Remarks

Any set of axis may be used for linear interpolation.

LI commands are used to specify the travel distances between various linear interpolation moves.

Several LI commands may be given as long as the controller sequence buffer has room for additional segments See the LI command for more information regarding the Linear Inerpolation Buffer The LE command specifies the end of the linear interpolation sequence.

Once the LM command has been given, it does not need to be given again unless the VM command has been used

###### Operand/Queries

_LMm contains the number of spaces available in the sequence buffer for the 'm' coordinate system, S or T.

The LM command will apply to the selected coordinate system, S or T. To select the coordinate system, use the command CA S or CA T.

Examples

'Galil DMC Code Example

LM ABCD;                         'Specify linear interpolation mode

VS 10000;VA 100000;VD 1000000;   'Specify vector speed, acceleration and deceleration

LI 100,200,300,400;              'Specify linear distance

LI 200,300,400,500;              'Specify linear distance

LE; BG S;                        'Last vector, then begin motion

LM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LM

INTERROGATION

##### LS List

LS n

0

,n

1

LS n

0

,n

1

| Usage | LS n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The LS command returns a listing of the programs in memory.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 |  | 3,998 | 0 | 1 | Firmware Rev 1.2a and later. Specifies the line in the program for which the listing will start |  |
| n0 | 0 |  | 1,998 | 0 | 1 | Specifies the line in the program for which the listing will start |  |
| n1 | 1 |  | 1,999 | 1,999 | 1 | Specifies the line at which the listing will end |  |

Remarks

n0 < n1 must always be true

If n0 or n1 is omitted, default values are used n0 and n1 can also specify a label, for example:

"LS #label,20" would print out program lines from #label to line 20.

Issuing this command will pause the output of the Data Record until the command is completed.

Examples

'Galil DMC Code Example :LS #a,6; ' List program starting at #A through line 6 2 #a - PR 500 - BG A - AM - WT 200 'Hint: Remember to quit the Edit Mode Q prior to giving the LS command. (DOS)

LS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LS

INTERROGATION

##### LV List Variables

LV

LV

| Usage | LV | Command takes no arguments |
| --- | --- | --- |

Description

The LV command returns a listing of all of the program variables in memory. The listing will be in alphabetical order.

Arguments

LV is an interrogation command with no parameters

Remarks

Use the _UL operand for total number of variables available for your controller. See the UL command for more details.

Issuing this command will pause the output of the Data Record until the command is completed.

Examples

'Galil DMC Code Example :LV apple = 60.0000 banana = 25.0000 zebra = 37.0000 :

LV applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LV

SYSTEM CONFIG

##### LZ Omit leading zeros

LZ n

LZ n

| Usage | LZ n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _LZ | Operand has special meaning, see Remarks |

Description

The LZ command is used for formatting the values returned from interrogation commands, variables, and arrays. By enabling the LZ function, all leading zeros of returned values will be removed.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Does not remove leading zeros from interrogated values |  |
|  | 1 | Removes leading zeros from interrogated values | Default |

Remarks

_LZ contains the state of the LZ function. '0' is disabled and '1' is enabled.

Examples

'Galil DMC Code Example :LZ 0; 'Disable the LZ function :var1= 10; 'Sets variable var1 to the value of 10. :TP A; 'Interrogate the controller for current position of A-axis 0000021645.0000 :var1=?; 'Request value of variable var1 0000000010.0000 :LZ 1; 'Enable LZ function :TP A; 'Interrogate the controller for current position of A-axis 21645.0000 :var1= ?; 'Request value of variable var1 10.0000

'Galil DMC Code Example

:LZ 0;     'Disable the LZ function

:TB;       'Tell status bits

001

:LZ 1;     'Inhibit leading zeros

:TB;       'Tell status

1

LZ applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

LZ

ETHERNET

##### MB   Modbus

MBm= n

0

, n

1

, n

2

, n

3

, str[]

MBm= n

0

, n

1

, n

2

, n

3

, str[]

| Usage | MBm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |

Description

The MB command is used to communicate with I/O devices using the Modbus TCP/IP protocol. The MB command supports the first two levels of Modbus commands. The function code -1 designates that the first level of Modbus is used (creates raw packets and receives raw data). The other codes are the 10 major function codes of the second level. The format of the command varies depending on each function code.

Galil Modbus supports one master per slave.

Arguments

Level 2 Modbus Function Codes

| Function Code, n1 | Modbus Definition | Slaved Galil Description (RIO only) |
| --- | --- | --- |
| 01 | Read Coil Status (Read Bits) | Read Digital Outputs (RIO only) |
| 02 | Read Input Status (Read Bits) | Read Digital Inputs (RIO only) |
| 03 | Read Holding Registers (Read Words) | Read Analog Inputs (RIO only) |
| 04 | Read Input Registers (Read Words) | Read Analog Outputs (RIO only) |
| 05 | Force Single Coil (Write One Bit) | Write Digital Output (RIO only) |
| 06 | Preset Single Register (Write One Word) | Write Digital Outputs (RIO only) |
| 07 | Read Exception Status (Read Error Code) | Read Digital Outputs (RIO only) |
| 15 | Force Multiple Coils (Write Multiple Bits) | Write Digital Outputs (RIO only) |
| 16 | Preset Multiple Registers (Write Words) | Write Analog Outputs (RIO only) |
| 17 | Report Slave ID |  |

###### 01: MBm= n0, 1, n2, n3, str[]

Read Coil Status (Read Bits)

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 | 255 | 1 | see Notes | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| n2 | 0 | 9,999 | N/A | 1 | Address of first coil |  |
| n3 | 0 | 99 | N/A | 1 | Quantity of coils | Or, number of IO points to read |
| str | 1 char | 8 chars | N/A | String | Name of array to store values | str[0] holds the first value. |

'Galil DMC Code Example

MBC=,1,2,8,example[];'  Read inputs 2-9 from handle C, save to example[]

'equivalent to reading Digital Outputs or registers mapped to 100000

###### 02: MBm= n0, 2, n2, n3, str[]

Read Input Status (Read Bits)

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 | 255 | 1 | see Notes | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| n2 | 0 | 9,999 | N/A | 1 | Address of first input |  |
| n3 | 0 | 99 | N/A | 1 | Quantity of inputs | Or, number of IO points to read |
| str | 1 char | 8 chars | N/A | String | Name of array to store values | str[0] holds the first value. |

'Galil DMC Code Example

MBC=,2,4,3,example[];'  Read inputs 4,5 and 6 from handle C, save to example[]

'equivalent to reading Digital Inputs or registers mapped to 000000

###### 03: MBm= n0, 3, n2, n3, str[]

Read Holding Registers (Read Words)

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 | 255 | see Notes | 1 | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| n2 | 0 | 9,999 | N/A | 1 | Address of first register |  |
| n3 | 0 | 99 | N/A | 1 | Quantity of registers to read |  |
| str | 1 char | 8 chars | N/A | String | Name of array to store values | str[0] holds the first value. 2 bytes per element. Array must be as large as the value for n3 |

'Galil DMC Code Example

MBB=,3,1,4,example[];'  Read registers 1 through 4 from handle B, save to example[]

'equivalent to reading Analog Outputs, or registers mapped to 400000

###### 04: MBm= n0, 4, n2, n3, str[]

Read Input Registers (Read Words)

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 | 255 | see Notes | 1 | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| n2 | 0 | 9,999 | N/A | 1 | Address of first register |  |
| n3 | 1 | 99 | N/A | 1 | Quantity of registers to read |  |
| str | 1 char | 8 chars | N/A | String | Name of array to store values | str[0] holds the first value. 2 bytes per element. Array must be as large as the value for n3 |

'Galil DMC Code Example

MBB=,4,1,2,example[];'  Read registers 1 through 2 from handle B, save to example[]

'equivalent to reading Analog Inputs, or registers mapped to 300000

###### 05: MBm= n0, 5, n2, n3

Force Single Coil (Write One Bit)

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 |  | 255 | see Notes | 1 | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| n2 | 0 |  | 9,999 | N/A | 1 | Address of coil |  |
| n3 | 0 |  | 1 | 0 | 1 | Set coil status | 0 = turn off coil. 1 = turn on coil |

'Galil DMC Code Example

MBB=,5,11,1;'  Set coil 11 high

'equivalent to setting a Digital Output (SB/CB)

###### 06: MBm= n0, 6, n2, n3

Preset Single Register (Write One Word)

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 |  | 255 | see Notes | 1 | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| n2 | 0 |  | 9,999 | N/A | 1 | Address of holding register |  |
| n3 | 0 |  | 65,535 | 0 | 1 | Set register value |  |

'Galil DMC Code Example

MBC=,6,10,128;'        Write 128 to holding register 10 on handle C

'equivalent to setting digital outputs on the RIO, or setting registers addressed 400000

###### 07: MBm= n0, 7, str[]

Read Exception Status (Read Error Code)

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 | 255 | see Notes | 1 | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| str | 1 char | 8 chars | N/A | String | Name of array to store value | str[0] holds the received value, one byte only. |

When using function code 7 with a Galil slave, array element zero will be set to the byte value of the combined first 8 digital outputs. Only one byte in the array will be populated, element zero of array str[].

'Galil DMC Code Example

MBE=,7,example[];'   Read register and store in example[0]

###### 15: MBm= n0, 15, n2, n3, str[]

Force Multiple Coils (Write Multiple Bits)

| Argument | Min | Max | Default | Resolution | Description | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |  |
| n0 | 0 | 255 | see Notes | 1 | Unit ID | Default to Handle number (A=1, B=2, etc.) |  |
| n2 | 0 | 9,999 | N/A | 1 | Address of first coil |  |  |
| n3 | 1 | 16 | N/A | 1 | Quantity of coils |  |  |
| str | 1 char | 8 chars | N/A | String | Array to set values for coils | str[0] holds the first value. 16 bits per element |  |
| 'Galil DMC Code Example example[0]=255;' MBC=,15,0,16,example[];' Set 1st byte of coils high and 2nd byte of coils low 'equivalent to setting digital outputs on RIO, or setting coils addressed 000000 |  |  |  |  |  |  |  |

###### 16: MBm= n0, 16, n2, n3, str[]

Preset Multiple Registers (Write Words)

| Argument | Min | Max | Default | Resolution | Description | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |  |
| n0 | 0 | 255 | see Notes | 1 | Unit ID | Default to Handle number (A=1, B=2, etc.) |  |
| n2 | 0 | 9,999 | N/A | 1 | Address of first register |  |  |
| n3 | 0 | 99 | N/A | 1 | Quantity of registers |  |  |
| str | 1 char | 8 chars | N/A | String | Array containing modbus data | str[0] holds the first value. 2 bytes per element. Array size must be > n3 |  |
| 'Galil DMC Code Example example[0]=$AEAE MBD=,16,2,1,example[];' Set $AEAE to holding register 2 on handle D 'equivalent to setting analog outputs, or writing to holding registers addressed 400000 |  |  |  |  |  |  |  |

###### 17: MBm= n0,17,str[]

Report Slave ID

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 | 255 | see Notes | 1 | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| str | 1 char | 8 chars | N/A | String | Name of array to receive data | str[0] holds the value. |

'Galil DMC Code Example

MBB=,17,example[];'   store slave ID of device on handle B to example[]

###### Raw Modbus Packet Send MBm= n0,-1,n2,str[]

Raw Modbus Send

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 | 255 | 1 | see Notes | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| n2 | 0 | 999 | N/A | 1 | Number of array bytes to send |  |
| str | 1 char | 8 chars | N/A | String | Name of array containing outgoing data | Array size >= n2. See Remarks |

###### Raw Modbus Packet Send/Receive MBm= n0,-1,n2,str0[],n3,n4,str1[]

Raw Modbus Send/Receive

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Handle | Handle to send Modbus command |  |
| n0 | 0 | 255 | 1 | see Notes | Unit ID | Default to Handle number (A=1, B=2, etc.) |
| n2 | 1 | 999 | N/A | 1 | Number of array bytes to send |  |
| str0 | 1 char | 8 chars | N/A | String | Name of array containing outgoing data | Array size >= n2. See Remarks |
| n3 | 1 | 999 | N/A | 1 | Number of bytes of incoming data to discard |  |
| n4 | 1 | 999 | N/A | 1 | Number of bytes of incoming data to store in str1[] |  |
| str1 | 1 char | 8 chars | N/A | String | Name of array storing incoming data. | Array size >= n4. See Remarks |

MB applies to DMC4000,DMC4103,DMC4200,DMC30010,RIO47000,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TRIPPOINT

##### MC   Motion Complete

MC mm

MC mm

| Usage | MC mm | Argument is an axis mask |
| --- | --- | --- |

Description

The MC command is a trippoint command that holds up execution until motion is complete on the specified axes. The MC command, unlike the AM (after motion command) requires that both the motion profiler has completed motion AND that the motor encoder has reached the specified position before continuing execution.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axis to assign value | Any combination of the axis is valid. If no axis is specified, command applies to all axis. |

Remarks

Motion must be actively profiling on an axis for the MC command to take affect. If the MC command is issued for an axis which is not profiling motion, the trippoint will immediately clear.

###### Using MC with Stepper Motors

With stepper motors, the MC trippoint will clear once the controller has generated the number of step pulses required to complete the move. The MC command is recommended when operating with stepper motors in lieu of AM since the generation of step pulses can be delayed due to the stepper motor smoothing function, KS. In this case, the MC command would only be satisfied after all steps are generated.

###### Using MC as part of the #MCTIME error routine

The command TW can be used to set an acceptable amount of time between when the motion profiler has completed and the encoder is in position; if this condition is not satisfied, a timeout error occurs.

- When a timeout occurs, the trippoint will clear and the stop code will be set to 99.

- Thread 0 of the DMC program will also jump to the special label #MCTIME, if present.

See the #MCTIME automatic subroutine, TW and SC commands for more information

Examples

'Galil DMC Code Example #move; 'Label #move TW 1000,1000; 'Set motion complete timeout to 1000 milliseconds per axis PR 2000,4000; 'Position relative Move on A- and B-axis BG AB; 'Start the motion on A- and B-axis MC AB; 'After the move is complete on A and B axes MG "DONE"; 'Print message EN; 'End of Program ' ' #MCTIME; 'Motion Complete timeout Subroutine MG "Motion Timeout"; 'Print failure message SC; 'Print stop codes EN; 'End subroutine

MC applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

###### MC

ETHERNET

ME   Modbus array write enable

ME n

ME n

| Usage | ME n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

##### Description

The ME command enables the ability for Modbus masters to write to array locations in the hardware's array table. When enabled, array locations can be written to as 16 bit integers or as 32 bit floating point by a modbus master by specifying different address ranges. ME is not required to read array locations, reads are always supported.

##### Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Disables the ability for Modbus masters to write to the array table | Default |
|  | 1 | Enables ability for Modbus masters to write to the array table |  |

##### Remarks

Array writes when enabled by ME are done using function code 16 Galil Modbus supports one master per slave.

DMC-4103 requires firmware revision 1.2c or newer for use as Modbus slave.

Modbus Register Map

Each element is accessible as a 16 bit unsigned integer (Modbus registers 10xx) -OR- as a 32 bit floating point number (Modbus registers 2000). The table below shows the mapping for a Modbus master writing to the controller with ME 1 set.

1000 (0-999) elements are available for read/write on the DMC-4103. Other array elements are not exposed to Modbus. Modbus Register Map to Galil Array A[]

| Modbus Registers: | 1000-1999 | 2000-2999 |
| --- | --- | --- |
| Available Modbus function codes | 3 (read) and 16 (write) | 3 (read) and 16 (write) |
| Number Type | 16 bit unsigned integer | 32 bit floating point |
| References in A[] array | A[0]-A[999] | A[0]-A[999] |
| Number written to A[] | Integer only, fraction not changed | Galil 4.2 format (internal from float conversion) |
| Number read from A[] | Integer only, fraction not read | 32 bit float (internal to float conversion) |
| Example Modbus Master Write | MBH=0,16,1000,1,write[] | MBH=0,16,2001,2,write[] |
| Example Modbus Master Read | MBH=0,3,1000,1,read[] | MBH=0,3,2001,2,read[] |

Embedded Array Mapping

Once enabled, the entire array table can be written remotely. These writes can span across dimensioned user arrays. It is the user's responsibility to partition the array table and to read/write remotely to the correct location.

When using multiple array names, the array table is partitioned alphabetically (all captital letters first).

For example, a partioned array of Grape[600] and Orange[200] would place the first 600 registers in Grape[], and the next 200 registers in Orange[]. The last 200 elements would be inaccesible from embedded code. If the user then dimensioned the array Apple[200], the register mapping would change. The first 200 registers would read/write from Apple[], the next 600 from Grape[], and finally the last 200 from Orange[].

Additionally, all captial letters come before lowercase letters. For example, a partioned array of Banana[100] and apple[200] would place the first 100 registers in Banana[] and the next 200 registers in apple[].

For simplicity, Galil recommends that a single array be dimensioned with the array name "A".

##### Examples

'Galil DMC Code Example :DA *[];' Deallocates all arrays :DM A[400];' Allocates array for Modbus Read/Write :ME0;' Disables write access :ME1;' Enables write access :ME?;' Interrogate current value 1 :

'Galil DMC Code Example 'This example is written for a Galil modbus master to a DMC-4000, DMC-4103, DMC-4200, DMC-30010, or RIO-47000 (with expanded memory) 'Master is E.G. DMC-2103, RIO, DMC-4000 'This code runs on the master. 'Assumes a Modbus handle is available at H, ' and that ME1 has been set on the remote device MW1;' Turn on modbus wait DM write[2];' Dimension an array for holding data to transmit write[0]=1234;' Assign an integer to element 0 MBH=0,16,1000,1,write[];' Send the integer to register 1000 on the remote write[0]=$42F6;' Set the 32 bit float in two steps, the value is 123.456 write[1]=$E978 MBH=0,16,2001,2,write[];' Send the float to register 2001 on the remote 'note that register 2000 would have stepped on the integer memory written at 1000 DM read[2];' Dimension an array for holding read data MBH=0,3,1000,1,read[];' Read the integer at register 1000 MG"Integer=",read[0];' Print the read integer MBH=0,3,2001,2,read[];' Read the float at register 1000

###### ME

float=(read[0]*$10000) + read[1];'    Construct the float. Shifting necesary for high bytes

MG"Float=", float{$8.0};'             Print the float in hex

ME applies to DMC4000,DMC4200,DMC4103,DMC30010,RIO47000

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

###### ME

TRIPPOINT MF   Forward Motion to Position

### MFm= n MF n,n,n,n,n,n,n,n

| Usage | MFm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | MF n ... | Arguments specified with an implicit, comma-separated order |

Description

This command will hold up the execution of the following command until the specified motor moves forward and crosses the position specified.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | - 2,147,483,648 | 2,147,483,647 | N/A | 1 | Position required to be crossed before subsequent commands will be executed. |  |

Remarks

Although multiple positions can be specified, only one of the MF conditions must be satisfied for subsequent code execution.

MF command references absolute position.

The MF command only requires an encoder and does not require that the axis be under servo control.

The accuracy of the MF command is the number of counts that occur in 2*TM sec. Multiply the speed by 2*TM sec to obtain the maximum error. Example with speed of 20,000 counts/second and TM of 1000 (1000 us).

Maximum error = 2 * 1000 E-6 seconds * 20,000 counts/second = 40 counts When using a stepper motor:

This condition is satisfied when the stepper position (as determined by the output buffer - TD) has crossed the specified Forward Motion Position.

Examples

'Galil DMC Code Example

#test;                   'Program Test

DP 0;                    'Define zero

JG 1000;                 'Jog mode (speed of 1000 counts/sec)

BG A;                    'Begin move

MF 2000;                 'After passing the position 2000 v1= _TPA;                'Assign V1 A position

MG "Position is",v1;     'Print Message

ST A;                    'Stop

EN;                      'End of Program

MF applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### MF

INTERROGATION

##### MG   Message

MG "str", {^n

0

}

, n

1

MG "str", {^n

0

}

, n

1

| Usage | MG n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The MG command is used to send strings, operands, variables, and array values to a specified destination.

##### Arguments

| Argument | Value |  | Description |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | String |  | A string including alphanumeric characters to be displayed |  |  | Limited to 76 characters |
| n0 | ASCII character in decimal |  | Allows users to print ASCII characters |  |  | Range of 0-255 |
| n1 | Numeric value |  | Prints the numeric value specified |  |  | See Examples for valid uses of n1. |
|  | Variable name |  | Prints the numeric value stored by the variable |  |  |  |
|  | Operand |  | Prints the numeric value stored by the operand |  |  |  |
|  | Array element |  | Prints the numeric value stored by the array element |  |  |  |
|  | Mathematical expression |  | Prints the numeric value of the solved equation |  |  |  |
| Argument | Value | Description |  | Notes |
| n | Operand | Prints the numeric value stored by the operand |  |  |

##### Remarks

Multiple strings, variables, and ASCII characters may be used; each must be separated by a comma.

Solicited Messages

From a host terminal, application code, or device, sending the MG command will return with the requested information. This is known as a solicited command, because the host sends the command and expects a response.

Formatting

Formatting

Unsolicited Messages

From embedded DMC code, the MG command will send an unsolicited, asynchronous message from the controller to the host. This can be used to alert an operator, send instructions, or return a variable value. This is known as an unsolicited command because the host is not explicitly requesting it.

The CW command controls the ASCII format of all unsolicited messages.

Unsolicited messages can go to any of the Ethernet handles or serial ports.

The CF command sets the default communication port for routing unsolicited messages.

Formatters can be placed after each argument in to modify how it is printed.

{Fm.n} Display variable in decimal format with m digits to left of decimal and n to the right.

{Zm.n} Same as {Fm.n} but suppresses leading zeros.

{$m.n} Display variable in hexadecimal format with m digits to left of decimal and n to the right.

{Sn} Display variable as a string of length n, where n is 1 through 6. If n is greater than the length of the string stored in the variable, null chars (0000) will be inserted at the end of the string.

{N} Suppress carriage return at the end of the message.

Message Routing

MG can override the default CF setting by using the following modifiers at the beginning of the message, right after MG. {Pn} Sends the message out the Serial port n, where n is 1 or 2 denoting Main or Auxilary (where equipped).

{Ex} Sends the message out the Ethernet handle x, where x is A,B,C,D,E,F,G, or H

##### Examples

Valid uses of n1 argument

'Galil DMC Code Example :'Values :MG 1234.5678 1234.5678 :' :'Variables :var= 12345678.9101 :MG var 12345678.9101 :' :'Operands :MG @AN[1] 0.0121 :' :'Array Elements :DM arr[3] :arr[0]=0 :arr[1]=1 :arr[2]=2 :MG arr[0],arr[1],arr[2] 0.0000 1.0000 2.0000 :' :'Mathematical Expressions :MG 1+2 3.0000 :MG arr[2]+var 12345680.9101 :

General Use

'Galil DMC Code Example

:MG "Good Morning";                  'Message command displays ASCII string Good Morning

:total= 1234.5322;                   'Assigns variable total with the value 1234.5322

:MG "The answer is...",total{F4.2};  'Will print the message and the value of variable total formatted with 4 integer digits and 2 fractional digits The answer is... 1234.53

:MG {^13}, {^10}, {^48}, {^055};     'Specifies carriage return, line feed, and the characters 0 and 7 in ASCII decimal values

07

:MG TIME;                            'Messages the operand TIME  261928200.0000

:variable= 10;                       'Sets the variable equal to 10

#### MG

:MG variable+5;                      'Messages out variable + 5

15.0000

:MG _TI0;                            'Messages the value stored in the operand _TI0  255.0000

| 'Galil DMC Code Example CF A; 'Messages configured to go out Ethernet handle A MG {EB}var; 'Override CF and send the value of variable var to B handle |  |
| --- | --- |

MG applies to

DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

MG

SYSTEM CONFIG

#### MO Motor Off

MO mm

MO mm

| Usage | MO mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _MOm | Operand has special meaning, see Remarks |

Description

The MO command turns off the motor command line and toggles the amplifier enable signal.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Specifies axis to turn off |  |

Remarks

The controller will continue to monitor the motor position See the TP command for more details

To turn the motor back on use the SH (Servo Here) command.

The MO command is useful for positioning the motors by hand.

_MOm contains 1.000 if the axis is in the motor off state or 0.000 if the axes is in the servo here state.

Examples

'Galil DMC Code Example MO; 'Turns off all motors MO A; 'Turns off the A motor. MO B; 'Turns off the B motor. MO CA; 'Turns off the C and A motors. SH; 'Turns all motors on axis= _MOA; 'Sets variable axis equal to the A-axis servo status

MO applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

##### MO

TRIPPOINT MR Reverse Motion to Position

### MRm= n MR n,n,n,n,n,n,n,n

| Usage | MRm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | MR n ... | Arguments specified with an implicit, comma-separated order |

Description

This command will hold up the execution of subsequent DMC code until the specified axis moves backwards and crosses the position specified.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | - 2,147,483,648 | 2,147,483,647 | N/A | 1 | Value of position that must be crossed in the reverse direction |  |

Remarks

MR command references absolute position.

Although multiple positions can be specified, only one of the MR conditions must be satisfied for subsequent code execution.

The MR command only requires an encoder and does not require that the axis be under servo control.

The accuracy of the MR command is the number of counts that occur in 2*TM usec. Multiply the speed by 2*TM usec to obtain the maximum error. Example with speed of 20,000 counts/second and TM of 1000 (1000 us).

Maximum error = 2 * 1000 E-6 seconds * 20,000 counts/second = 40 counts

When using a stepper motor, this condition is satisfied when the stepper position (as determined by the output buffer - TD) has crossed the specified reverse motion position.

Examples

'Galil DMC Code Example

#TEST;'                 Program Test

DP0;'                  Define zero

JG -1000;'             Jog mode (speed of 1000 counts/sec)

BG A;'                 Begin move

MR -3000;'             After passing the position -3000

V1=_TPA;'              Assign V1 A position

MG "Position is", V1;' Print Message

ST;'                   Stop

EN;'                    End of Program

MR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

MR

SYSTEM CONFIG

#### MT Motor Type

MTm= n

MT n,n,n,n,n,n,n,n

MTm= n

MT n,n,n,n,n,n,n,n

| Usage | MTm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | MT n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _MTm | Operand holds the value last set by the command |

Description

The MT command selects the type of the motor and the polarity of the drive signal. Motor types include standard servomotors, which require a voltage in the range of +/- 10 Volts, and stepper motors, which require step and direction signals. The polarity reversal inverts the analog signals for servomotors, or inverts logic level of the pulse train for step motors.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |  |
| Argument | Value |  | Description |  |  |  |  |  | Notes |  |
| n | 1 |  | Servo motor (3-phased brushless) |  |  |  |  |  | Default |  |
|  | -1 |  | Servo motor (3-phased brushless), reversed direction |  |  |  |  |  |  |  |
|  | 2 |  | Stepper motor with active low step pulses |  |  |  |  |  |  |  |
|  | 2.5 |  | Stepper motor with active low step pulses, reversed direction |  |  |  |  |  |  |  |
|  | -2 |  | Stepper motor with active high step pulses |  |  |  |  |  |  |  |
|  | -2.5 |  | Stepper motor with active high step pulses, reversed direction |  |  |  |  |  |  |  |
|  | 4 |  | Servo motor (2PB) |  |  |  |  |  | Only valid for D3547 |  |
|  | -4 |  | Servo motor (2PB), reversed direction |  |  |  |  |  | Only valid for D3547 |  |

Remarks

The axis must be in the motor off state (MO) before setting MT n = ? will return the value of the motor type for the specified axis.

For stepper motor configuration (n=2,-2,2.5,-2.5), the auxiliary encoder input for the axis is no longer available.

Related Commands

#AMPERR - Amplifier error automatic subroutine

AG - Amplifier Gain

AU - Set amplifier current loop

AZ - Clear Amplifier Errors

TA - Tell Amplifier Error

TK - Peak Torque Limit

TL - Torque Limit

Examples

'Galil DMC Code Example

MO

MT 1,-1,2,2;   'Configure A as servo, B as reverse servo, C and D as steppers

MT ?,?;        'Interrogate motor type for A- and B-axis

MT applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

MT

ETHERNET

#### MU Multicast Address

MU n

0

,n

1

,n

2

,n

3

MU n

0

,n

1

,n

2

,n

3

| Usage | MU n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _MU | Operand has special meaning, see Remarks |

Description

MU sets the controller's multicast address. This address is used by Galil software to detect an available Ethernet controller on the network.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 |  | 255 | 239 | 1 | First field of the multicast address |  |
| n1 | 0 |  | 255 | 255 | 1 | Second field of the multicast address |  |
| n2 | 0 |  | 255 | 19 | 1 | Third field of the multicast address |  |
| n3 | 0 |  | 255 | 56 | 1 | Last field of the multicast address |  |

Remarks

Supported on DMC-4103 firmware rev 1.1b and above.

MU ? returns the current multicast address setting in 4 byte format _MU contains the 32-bit multicast address number in two's complement.

Examples

'Galil DMC Code Example :MU 239,255,19,57 :MU? 239, 255, 019, 057 :MG_MU -268496071.0000 :MG_MU{$8.0} $EFFF1339 :

MU applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

MU

ETHERNET

#### MW   Modbus Wait

MW n

MW n

| Usage | MW n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _MW0 _MW1 | Operand has special meaning, see Remarks |

Description

Enabling the MW command causes the controller to hold up execution of the program after sending a Modbus command until a response from the Modbus device has been received. The MW command ensures that the command that was sent to the Modbus device was successfully received before continuing program execution.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Disables Modbus wait |  |
|  | 1 | Enables Modbus wait | Default |

Remarks

n = ? returns the state of the Modbus wait, either 1 or 0

If a Modbus response is never received, then thread 0 will jump to the #TCPERR subroutine, if it exists, and TC will report an error code of 123. MW prevents the controller from sending multiple commands to the same Modbus device before it has a chance to execute them. Operands

_MW0 returns last function code received

_MW1 returns Modbus error code

##### _MWn operands

'Galil DMC Code Example

:MG_MW0{$8.0};' $ is the hex formatter

$00000001

:'above is an expected response to function code 1

###### :MG_MW1{$8.0}

$00000000

:'no error

_MW0 Responses

| Function Code Sent | Normal _MW0 Response | _MW0 Exception Response |
| --- | --- | --- |
| 1 | $01 | $81 |
| 2 | $02 | $82 |
| 3 | $03 | $83 |
| 4 | $04 | $84 |
| 5 | $05 | $85 |
| 6 | $06 | $86 |
| 7 | $07 | $87 |
| 15 | $0F | $8F |
| 16 | $10 | $90 |

_MW1 Responses

| _MW1 returns | Exception description |
| --- | --- |
| $00 | Normal response |
| $01 | The request referenced an illegal function code |
| $02 | The request referenced an illegal data address |

Examples

'Galil DMC Code Example

MW1;      'Enables Modbus Wait

SB1001;   'Set Bit 1 on Modbus Handle A

CB1001;   'Clear Bit 1 on Modbus Handle A

'Galil DMC Code Example

REM Example on Modbus master, DMC-4000

REM Using _MW operands

:IHH=192,168,42,43<502>2;' connect to RIO

:MW1

:SB 8001;' set bit one on RIO

::MBH=,5,1,0;' clear it with MB

::'CB 8001 would also work

:MG_MW0

5.0000

:'funct code 5 confirmed

:MG_MW1

##### MW

0.0000

:'no errors

:MBH=,5,100,1;' invalid output point

::TC1

0

:MG_MW0{$8.0}

$00000085

:'Exception on funct code 5

:MG_MW1{$8.0}

$00000002

:'illegal data address

MW applies to DMC4000,DMC4103,DMC4200,DMC30010,RIO47000,DMC2103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

##### MW

FILTER/CONTROL NB Notch Bandwidth

### NBm= n NB n,n,n,n,n,n,n,n

| Usage | NBm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | NB n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _NBm | Operand holds the value last set by the command |

Description

The NB command sets real part of the notch poles. In other words, the NB controls the range of frequencies that will be attenuated.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 62.5 | 0.5 | 1/2 | Value of the notch bandwidth in Hz | Max value dependent upon TM setting, see Remarks |

Remarks

_NBm contains the value of the notch bandwidth for the specified axis.

NB also determines the ratio of NB/NZ which controls the attenuation, or depth, of the notch. See NZ for more details. See the NF command for recommendations on choosing NZ, NB, and NF values.

See Application note #2431 for additional information on setting the NF, NB and NZ commands http://www.galil.com/download/application-note/note2431.pdf

#### Maximum Range

The maximum n argument is specified in Hz and is calculated by the equation below:

where TM is specified in microseconds.

The default TM is 1000, therefore default maximum NB value = 1/(16x1000E-6) = 62.5 Hz

Examples

'Galil DMC Code Example

NBA= 10;          'Sets the real part of the notch pole to 10/2 Hz notch = _NBA;     'Sets the variable "notch" equal to the notch bandwidth value for the A axis

NB applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### NB

FILTER/CONTROL NF NotchFrequency

### NFm= n NF n,n,n,n,n,n,n,n

| Usage | NFm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | NF n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _NFm | Operand holds the value last set by the command |

Description

The NF command sets the frequency of the notch filter, which is placed in series with the PID compensation.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -250 | 250 | 0 | 1 | Sets the frequency of the notch filter | See Remarks |

Remarks

n = 0 disables the notch.

n > 0 applies the notch filter in series with the PID compensation. The value of n is the frequency of the notch filter.

n < 0 applies the notch filter to the profiler. The absolute value of n is the frequency of the notch filter.

n < 0 only applies to firmware rev 1.3b and later.

n = 0 disables the notch.

_NFm contains the value of notch filter for the specified axis. n = ? Returns the value of the Notch filter for the specified axis.

See Application note #2431 for additonal information on setting the NF, NB and NZ commands http://www.galil.com/download/application-note/note2431.pdf

#### Chosing NF, NB, and NZ

- A simple way for attaining NF, NB, and NZ parameters is to follow these simple rules:

  - Estimate the resonance frequency

  - Set NF equal to the resonance frequency

  - Set NB = 1/2NF

  - Set NZ between 0 and 5

- The ratio of NB/NF is extremly important. See the NB command for more details.

#### Maximum Range

The maximum n argument is specified in Hz and is calculated by the equation below:

Where TM is in microseconds.

Default TM is 1000, therefore default maximum value = 1E6/(4*1000) = 250 Hz

Examples

'Galil DMC Code Example

NF, 20;' Sets the notch frequency of B axis to 20 Hz

NF applies to EDD37010,DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

NF

PROGRAMMING

#### NO No Operation

NO str

NO str

| Operands | _NO | Operand has special meaning, see Remarks |
| --- | --- | --- |

Description

The NO command performs no action in a sequence and can be used as a comment in a program.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| str | String | A no action sequence used to document a program | Comments are limited to the maximum row size in a program. This will vary by controller. |

Remarks

_NO returns a bit mask indicating which threads are running.

For example:

- means no threads are running

- means only thread 0 is running

3 means threads 0 and 1 are running

Examples

'Galil DMC Code Example

#a;                    'Program A

NO;                    'No Operation

NO This Program ;      'No Operation

NO Does Absolutely;    'No Operation

NO Nothing;            'No Operation

EN;                    'End of Program

NO applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

NO

FILTER/CONTROL

NZ NotchZero

### NZm= n NZ n,n,n,n,n,n,n,n

| Usage | NZm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | NZ n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _NZm | Operand holds the value last set by the command |

Description

The NZ command sets the real part of the notch zero. In other words, the NB/NZ ratio controls the amount of attenuation, or depth, of the notch filter.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0.5 |  | 62.5 | 0 | 0.5 | Value of Notch Frequency in Hz | Max value dependent upon TM setting, see Remarks |

Remarks

See the NF command for recommendations on chosing NZ, NB, and NF values.

The maximum n argument is determiend by the following equation

Where TM is in microseconds, the default TM is 1000.

See Application note #2431 for additonal information on setting the NF, NB and NZ commands http://www.galil.com/download/application-note/note2431.pdf

#### The NB/NZ Ratio

The ratio, NB/NZ controls the amount of attenuation, or depth of the notch.

The larger the ratio of NB/NZ, the larger the attenuation, and vice versa.

If NB/NZ > 1 the signal will amplifiy the output signal causing a resonance. NB = NZ essentially eliminates the notch

Examples

'Galil DMC Code Example

NZA = 10;' Sets the real part of the notch pole to 10/2 Hz

NZ applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

NZ

ERROR CONTROL

#### OA Off onencoder failure

OAm= n

OA n,n,n,n,n,n,n,n

OAm= n

OA n,n,n,n,n,n,n,n

| Usage | OAm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | OA n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _OAm | Operand holds the value last set by the command |

Description

The OA command turns on or off encoder failure detection. The controller can detect a failure on either or both channels of the encoder. This is accomplished by checking on whether motion of less than 4 counts is detected whenever the torque exceeds a preset level (OV) for a specified time (OT).

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 1 |  | 0 | 1 | Status of encoder failure detection | 1 = enabled, 0 = disabled |

Remarks

The OA command works like the OE command: if OA is set to 1 and an encoder failure occurs, the axis goes into the motor off (MO) state and the stop code (SC) is set to 12 if detected during motion.

The encoder failure detection will shut the motor off regardless of profiling status, but the stop code is not updated unless the axis is executing a profiled move at the time of the detection of the encoder failure.

If included in the application program and OA is set to 1, #POSERR will run when an encoder failure is detected for the axis. Note that for this function to work properly it is recommended to have a non-zero value for KI.

Examples

'Galil DMC Code Example

OAA= 1;' enable A axis encoder error detection

MG_OAA;'query OA value for A axis

'Galil DMC Code Example

OA ,1;'  enable B axis encoder error detection

'Galil DMC Code Example

#setup

'setup the encoder error detection

OTA=10;'  Set time to 10 milliseconds

OVA=5;'   Set voltage to 5

OAA=1;'   Enable encoder detection feature

EN

'Galil DMC Code Example REM #POSERR example for checking to see if encoder failure occured REM This procedure is needed because the stop code will only update if REM the profilier is running at the time the encoder failure is detected. #POSERR ~a=0 #loop IF _MO~a=1 IF ((_TE~a<_ER~a)&(_OE~a)&(_OA~a)) MG "possible encoder failure on ",~a{Z1.0}," axis" ENDIF ENDIF ~a=~a+1 JP#loop,~a<_BV AI1;' wait for input 1 to go high SH;' enable all axes RE

OA applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806,EDD37010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

OA

IO

#### OB OutputBit

OB n,(ex)

OB n,(ex)

| Usage | OB n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The OB command allows variable control of an output bit based on logical expressions. The OB command defines output bit n as either 0 or 1 depending on the result from the logical expression.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 1 | 16 | 0 | 1 | Output bit specified | Outputs 9-16 only valid on 5-8 axis controller |
| n | 1,000 | 8,999 | N/A | 1 | Modbus output bit specified | See Remarks |
| ex | N/A | N/A | N/A | Expression | Expression that defines status of output | If ex is true/non-zero, set output to 1. If ex is false/zero, set output to 0 |

Remarks

An expression is any valid logical expression, variable or array element.

Any non-zero value of the expression results in a one set to the output bit.

Using OB with a Modbus Slave n = (SlaveAddress*10000) + (HandleNum*1000) + ((Module-1)*4) + (Bitnum-1)

Slave Address is used when the ModBus device has slave devices connected to it and specified as Addresses 0 to 255. Please note that the use of slave devices for modbus are very rare and this number will usually be 0.

HandleNum is the handle specifier where A is 1, B is 2 and so on.

Module is the position of the module in the rack from 1 to 16. BitNum is the I/O point in the module from 1 to 4

Examples

'Galil DMC Code Example

OB 1, pos;'          If pos<>0, Bit 1 is high.

'                    If pos=0, Bit 1 is low

OB 2, @IN[1]&@IN[2];' If Input 1 and Input 2 are both high, then

'                    Output 2 is set high

OB 3, count[1];' If the element 1 in the array is zero, clear bit 3 OB n, count[1];' If element 1 in the array is zero, clear bit n

OB applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,RIO47000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

OB

IO

#### OC OutputCompare

OCm = n

0

, n

1

OCm = n

0

, n

1

| Usage | OCm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
| Operands | _OC | Operand has special meaning, see Remarks |

Description

The OC command sets up the Output Compare feature, also known as Pulse on Position. The controller has a special digital output which can be configured to pulse on a specified absolute encoder position, and optionally on a delta encoder change after that. These operations are known as oneshot and circular compare, respectively.

Each set of 4 axes, ABCD and EFGH, has one digital output which can be configured to this mode of operation

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to enable output compare | Axes A-D share one output compare, axes E-H share a second output compare output |
| n0 | - 2,147,483,648 | 2,147,483,647 | N/A | 1 | Absolute encoder position of first pulse | n0 must be within 65535 counts of current position |
| n1 | -65,536 | 65,535 | N/A | 1 | Incremental encoder distance between pulses | 0 indicates single-shot pulse in positive direction, 65536 indicates single shot when moving in the negative direction |

Remarks

For controllers with 5-8 axes, two output compares are available. One for the A-D axes, the other for the E-H axes This command is only valid when both n0 and n1 are specified.

One shot Compare Mode:

The output compare signal will go low, and stay low at a specified absolute encoder position. This is done by specifying n1 as 0 for positive motion, and -65536 for negative motion.

Circular Compare Mode:

After the absolute position of the first pulse (n0), the circular compare can be configured to pulse low at a relative distance thereafter (n1). This is done by specifying n1 to a non-zero delta position (range of -65535 to 65535).

OCA = 0 will disable the Circular Compare function on axes A-D. OCE = 0 will disable the Circular Compare function on axes E-H.

The circular compare output is a low-going pulse with a duration of approximately 510 nanoseconds.

##### Limitations

The Output Compare function is only valid with incremental encoders.

The Output Compare function is not valid with SIN/COS (AF settings of 5-12), standard analog (AF setting of 1), BiSS or SSI feedback (SS or SI commands).

The OC function cannot work for an axis configured as a stepper.

The auxiliary encoder of the corresponding axis cannot be used when in this mode.

Dual loop mode (which uses the aux encoder input) will not operate when the OC command is enabled.

The OC function requires that the main encoder and auxiliary encoders be configured exactly the same (see the command, CE). For example: CE 0, CE 5, CE 10, CE 15.

OC only requires an encoder, and is independent of axis tuning, and motion profiling.

##### Operand Usage

_OC contains the state of the OC function.

_OC = 0 : OC function has been enabled but not generated any pulses.

_OC = 1: OC function not enabled or has generated the first output pulse.

On a 5-8 axis controller, _OC is a logical AND of axes A-D and E-H.

Examples

'Galil DMC Code Example

OCA=300,100;' Select A encoder as position sensor.

REM First pulse at 300. Following pulses at 400, 500, 600 ...

'Galil DMC Code Example

REM Output compare can be used to create raster scans.

REM By using circular compare on one axis, followed by an index move on a perpindicular axis

REM raster patterns are easily made.

REM The following image shows a rastered "dot matrix" type image easily created REM with output compare and a laser on a two dimensional stage.

##### OC

OC applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

OC

ERROR CONTROL

#### OE Off-on-Error

OEm= n

OE n,n,n,n,n,n,n,n

OEm= n

OE n,n,n,n,n,n,n,n

| Usage | OEm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | OE n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _OEm | Operand holds the value last set by the command |

Description

The OE command sets the Off On Error function for the controller. The OE command causes the controller to shut off the motor command if a position error exceeds the limit specified by the ER command, an abort occurs from either the abort input or on AB command, or an amplifier error occurs based on the description of the TA command.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 0 |  | 0 | 0 | Disables the Off On Error Function | Default |
|  | 1 |  | 1 |  | 0 | 0 | Motor shut off by position error, amplifier error or abort input |  |
|  | 2 |  | 2 |  | 0 | 0 | Motor shut off by hardware limit switch |  |
|  | 3 |  | 3 |  | 0 | 0 | Motor shut off by position error, amplifier error, abort input or by hardware limit switch |  |

Remarks

For any value of OE <> 0, the axis will be shut off due to amplifier faults on any amplifier axis. See the TA command for conditions of an amplifier fault. BR1 must be enabled when internal brushless servo amplifiers are installed but the axis is driven with an external amplifier. BR1 disables hall error checking when OE <> 0

Examples of brushless servo amps that require this consideration include the AMP-43040 (-D3040) or the AMP-20540

Motion Behavior:

If an error or axis-specific abort is detected, and the motion was executing an independent move, only that axis will be shut off. If the motion is a part of coordinated mode of the types GM, VM, LM or CM, all participating axes will be stopped.

Examples

'Galil DMC Code Example :OE 1,1,1,1;' Enable OE on all axes :OE 0;' Disable OE on A-axis, other axes remain unchanged :OE ,,1,1;' Enable OE on C-axis and D-axis, other axes remain unchanged :OE 1,0,1,0;' Enable OE on A and C-axis, Disable OE on B and D axis :MG _OEA;' Query A axis OE setting 1.0000

'Galil DMC Code Example #main 'code to enable the OE command for all error conditions 'and setup the corresponding automatic subroutines 'to display relevent data 'no loop for abort input, as that stops code operation OE 3,3,3,3 SHABCD JG*=1000;' all jog at 1000 BGABCD #loop 'endless loop WT1000 JP#loop EN #AMPERR MG "amplifier fault" MG _TA0,_TA1,_TA2,_TA3 EN #POSERR MG "position error fault" MG _TEA,_TEB,_TEC,_TED EN #LIMSWI MG "limit switch fault" MG _TSA,_TSB,_TSC,_TSD EN

OE applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

OE

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

OE

FILTER/CONTROL

#### OF Offset

OFm= n

OF n,n,n,n,n,n,n,n

OFm= n

OF n,n,n,n,n,n,n,n

| Usage | OFm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | OF n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _OFm | Operand holds the value last set by the command |

Description

The OF command sets a bias voltage in the command output or returns a previously set value.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -9.9982 | 9.9982 | 0 | 20/65,536 | Offset voltage applied to MCMD |  |

Remarks

This can be used to counteract gravity or an offset in an amplifier.

Examples

'Galil DMC Code Example

:OF 1,-2,3,5;' Set A-axis offset to 1, the B-axis offset to -2, the C-axis to 3, and the D-axis to 5

:OF -3;'  Set A-axis offset to -3  Leave other axes unchanged

:OF ,0;'  Set B-axis offset to 0  Leave other axes unchanged

:OF ?,?,?,?;' Return offsets

-3.0000,0.0000,3.0000,5.0000

:OF ?;'  Return A offset

-3.0000

:OF ,?;' Return B offset 0.0000

OF applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

OF

IO

##### OP   Output Port

OP n

OP n

| Usage | OP n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _OP0 | Operand holds the value last set by the command |

Description

The OP command sets the output ports of the controller in a bank using bitmasks. Arguments to the OP command are bit patterns (decimal or hex) to set entire banks (bytes) of digital outputs. Use SB, CB or OB to set bits individually.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  | 65,535 | 0 | 1 | Decimal representation: General Outputs 1-16 | On a 1-4 axis controller, max is 255 ($FF) for outputs 1-8 only. |

Remarks

###### Output Mapping Examples

| Example | Command Issued (Hex version) | Bits Set | Bits Cleared |
| --- | --- | --- | --- |
| 1-4 axis Set all outputs | OP255 (OP$FF) | 1-8 | - |
| 5-8 axis Set all outputs | OP65535 (OP$FFFF) | 1-16 | - |
| Clear all outputs | OP0 (OP$0000) | - | 1-16 |
| Alternating on/off | OP43690 (OP$AAAA) | 2,4,6,8,10,12,14,16 | 1,3,5,7,9,11,13,15 |
| Set High Byte | OP65280 (OP$FF00) | 9-16 | 1-8 |
| Set Low Byte | OP255 (OP$00FF) | 1-8 | 9-16 |

Examples

| 'Galil DMC Code Example OP 0;' Clear Output Port -- all bits OP $85;' Set outputs 1,3,8 and clear the others MG _OP0;' Returns the parameter "n0" |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Argument | Min | Max | Default | Resolution | Description | Notes |
| n | 0 | 65,535 | 0 | 1 | Decimal representation: General Outputs 1-16 | On a 1-4 axis controller, max is 255 ($FF) for outputs 1-8 only. |

Remarks

###### Output Mapping Examples

| Example | Command Issued (Hex version) | Bits Set | Bits Cleared |
| --- | --- | --- | --- |
| 1-4 axis Set all outputs | OP255 (OP$FF) | 1-8 | - |
| 5-8 axis Set all outputs | OP65535 (OP$FFFF) | 1-16 | - |
| Clear all outputs | OP0 (OP$0000) | - | 1-16 |
| Alternating on/off | OP43690 (OP$AAAA) | 2,4,6,8,10,12,14,16 | 1,3,5,7,9,11,13,15 |
| Set High Byte | OP65280 (OP$FF00) | 9-16 | 1-8 |
| Set Low Byte | OP255 (OP$00FF) | 1-8 | 9-16 |

Examples

'Galil DMC Code Example

OP 0;'   Clear Output Port -- all bits

OP $85;' Set outputs 1,3,8 and clear the others

MG _OP0;' Returns the parameter "n0"

OP applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

OP

ERROR CONTROL

#### OT Off on encoderfailure time

OTm= n

OT n,n,n,n,n,n,n,n

OTm= n

OT n,n,n,n,n,n,n,n

| Usage | OTm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | OT n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _OTm | Operand holds the value last set by the command |

Description

The OT command sets the timeout time for the encoder failure routine. The command sets the time in samples that the encoder failure will wait for motion after the OV threshold has been exceeded. The controller can detect a failure on either or both channels of the encoder.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 1 |  | 32,000 | 30 | 1 | Number of samples for error detection |  |

Remarks

Encoder error detection is based on whether motion of at least 4 counts is detected whenever the torque exceeds a preset level (OV) for a specified time (OT).

Note that for this function to work properly it is necessary to have a non-zero value for KI. See the OA command for more details on this error detection mode

Examples

'Galil DMC Code Example

OTD= 400;'   Set D axis encoder error timeout to 400 samples

OT 100,200;' Set A axis to 100 and B axis to 200 sample timeouts

'Galil DMC Code Example

#setup

OTA= 10;'  Set time to 10 milliseconds

OVA= 5;'   Set voltage to 5

OAA= 1;'   Enable encoder detection feature

EN

'Galil DMC Code Example REM #POSERR example for checking to see if encoder failure occured REM This procedure is needed because the stop code will only update if REM the profilier is running at the time the encoder failure is detected. #POSERR ~a=0 #loop IF _MO~a=1 IF ((_TE~a<_ER~a)&(_OE~a)&(_OA~a)) MG "possible encoder failure on ",~a{Z1.0}," axis" ENDIF ENDIF ~a=~a+1 JP#loop,~a<_BV AI1;' wait for input 1 to go high SH;' enable all axes RE

OT applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806,EDD37010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

OT

ERROR CONTROL

#### OV   Off on encoderfailure voltage

OVm= n

OV n,n,n,n,n,n,n,n

OVm= n

OV n,n,n,n,n,n,n,n

| Usage | OVm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | OV n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _OVm | Operand holds the value last set by the command |

Description

The OV command sets the threshold voltage for detecting an encoder failure. The controller can detect a failure on either or both channels of the encoder.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 9.9982 | 0.9438 | 20/65,536 | Torque voltage to trigger encoder error detection |  |

Remarks

Encoder error detection is accomplished by checking on whether motion of at least 4 counts is detected whenever the torque exceeds a preset level (OV) for a specified time (OT).

Note that for this function to work properly it is recommended to have a non-zero value for KI.

The value of OV should be high enough to guarantee that the motor would overcome any static friction in the system. If it is too low, there will be false triggering of the error condition.

The OV value may not be higher than the TL value.

See the OA command for more details on this error detection mode

Examples

'Galil DMC Code Example

OVB= 1.2;' Set B axis encoder detection torque value to 1.2V

OV 0.54;'  Set A axis encoder detection torque value to 0.54V

'Galil DMC Code Example

#setup

'setup the encoder error detection

OTA= 10;'  Set time to 10 milliseconds

OVA= 5;'   Set voltage to 5

OAA= 1;'   Enable encoder detection feature

EN

'Galil DMC Code Example REM #POSERR example for checking to see if encoder failure occured REM This procedure is needed because the stop code will only update if REM the profilier is running at the time the encoder failure is detected. #POSERR ~a=0 #loop IF _MO~a=1 IF ((_TE~a<_ER~a)&(_OE~a)&(_OA~a)) MG "possible encoder failure on ",~a{Z1.0}," axis" ENDIF ENDIF ~a=~a+1 JP#loop,~a<_BV AI1;' wait for input 1 to go high SH;' enable all axes RE

OV applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806,EDD37010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

OV

#### P2CD	code

### P2CD

| Usage | variable= P2CD | Holds a value |
| --- | --- | --- |
| Operands | P2CD | Operand has special meaning, see Remarks |

Description

P2CD returns the status of the auxiliary serial port (port 2). The value of P2CD returns zero after the corresponding string or number is read.

Arguments

P2CD is an operand that holds a value cooresponding to status. See Examples for use in code.

Remarks

P2CD contains the following status codes

P2CD Status Codes

| Status Code | Meaning |
| --- | --- |
| -1 | Mode disabled |
| 0 | Nothing received |
| 1 | Received character, but not carriage return |
| 2 | received a string, not a number |
| 3 | received a number |

Examples

'Galil DMC Code Example

:^R^V

DMC2240 Rev 1.0o

:^R^S

:CC 9600,0,1,0

:MG "TEST" {P2};'    send a message to the hand terminal

:MG P2CD;'           no characters entered on hand terminal

0.0000

:MG P2CD;'           the number 6 was pushed on the hand terminal

1.0000

:MG P2CD;'           enter key pushed on hand terminal

3.0000

:MG P2CD;'           the character B was pushed (shift f2) then enter 2.0000

P2CD applies to DMC50000,DMC4000,DMC4200,DMC4103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

P2CD

#### P2CH	character

### P2CH

| Usage | variable= P2CH | Holds a value |
| --- | --- | --- |
| Operands | P2CH | Operand has special meaning, see Remarks |

Description

P2CH returns the last character received by the controller's auxiliary serial port (port 2)

Arguments

P2CH is an operand that holds a value cooresponding to ASCII characters received by the serial port. See Examples for use in code.

Remarks

None

Examples

'Galil DMC Code Example :^R^V DMC2240 Rev 1.0o :^R^S :CC 9600,0,1,0 :MG "TEST" {P2} ;'send a message to the hand terminal :MG P2CH {S1} ;'the 6 button was pushed on the hand terminal 6 :

P2CH applies to DMC50000,DMC4000,DMC4200,DMC4103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

P2CH

#### P2NM	number

### P2NM

| Usage | variable= P2NM | Holds a value |
| --- | --- | --- |
| Operands | P2NM | Operand has special meaning, see Remarks |

Description

P2NM returns the last number (followed by carriage return) sent to auxiliary serial port (port 2).

Arguments

P2NM is an operand that holds a numerical value received by the controller's serial port. See Examples for use in code.

Remarks

Converts from ASCII (e.g. "1234") to binary so that a number can be stored into a variable and math can be performed on it. Numbers from -2147483648 to 2147483647 can be processed.

Examples

'Galil DMC Code Example :^R^V DMC2240 Rev 1.0o :^R^S :CC 9600,0,1,0 :MG "TEST" {P2} ;'send a message to the hand terminal :x = P2NM ;'the 1, 2, 3, buttons were pushed :MG x 123.0000 :

P2NM applies to DMC50000,DMC4000,DMC4200,DMC4103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

P2NM

#### P2ST	string

### P2ST

| Usage | variable= P2ST | Holds a value |
| --- | --- | --- |
| Operands | P2ST | Operand has special meaning, see Remarks |

Description

P2ST returns the last string (followed by carriage return) sent to auxiliary serial port (port 2)

Arguments

P2ST is an operand that contains a string. See Examples for usage.

Remarks

No more than 6 characters can be assessed by this operand

Examples

'Galil DMC Code Example :CC 9600,0,1,0 :MG "TEST" {P2} ;'send a message to the hand terminal :MG P2ST {S3} ;'the characters ABC were entered ABC

P2ST applies to DMC50000,DMC4000,DMC4200,DMC4103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

P2ST

INDEPENDENT MOTION

#### PA Position Absolute

PAm= n

PA n,n,n,n,n,n,n,n

PAm= n

PA n,n,n,n,n,n,n,n

| Usage | PAm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | PA n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _PAm | Operand has special meaning, see Remarks |

Description

The PA command sets the end target of the Position Absolute Mode of Motion.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
|  | M | N | N/A | Axis | Virtual axis to assign value |  |
| n | - 2,147,483,648 | 2,147,483,647 | 0 | 1 | Absolute position target for independant move | n=? returns the commanded position at which motion last stopped |

Remarks

The position is referenced to the absolute zero position, defined as position 0.

By default a new PA command may not be issued before the previous PA command has finished executing. This operation may be changed by running in Position Tracking Mode - See the PT command for more information.

Operand Usage  _PAm contains the last commanded position at which motion stopped.

Examples

'Galil DMC Code Example

:PA 400,-600,500,200;'     A-axis will go to 400 counts B-axis will go to -600 counts

:'                          C-axis will go to 500 counts D-axis will go to 200 counts :BG;'                  Execute Motion

:PA ?,?,?,?;'          Returns the current commanded position after motion has completed 400, -600, 500, 200

:PA 700;'                 A-axis will go to 700 on the next move while the

:BG;'                  B,C and D-axis will travel the previously set relative distance

:'                         if the preceding move was a PR move, or will not move if the :'                         preceding move was a PA move.

'Galil DMC Code Example

DP10000;'   set current position to 10000

PA3000;'    move to absolute position 3000, which is a -7000 count move BGA;'       begin -7000 count move

EN

PA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

PA

SYSTEM CONFIG

#### PF Position Format

PF n

0

.n

1

PF n

0

.n

1

| Usage | PF n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _PF | Operand holds the value last set by the command |

Description

The PF command allows the user to format the position numbers such as those returned by TP. The number of digits of integers and the number of digits of decimal can be selected with this command. An extra digit for sign and a digit for decimal point will be added to the total number of digits.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | -8 |  | 10 | 10 | 1 | Number of places displayed preceding the decimal point | Negative numbers force data to display in hexadecimal format |
| n1 | 0 |  | 4 | 0 | 1 | Number of places displayed after the decimal point |  |

Remarks

If PF is minus, the format will be hexadecimal and a dollar sign will precede the characters. Hex numbers are displayed as 2's complement with the first bit used to signify the sign.

If a number exceeds the format, the number will be displayed as the maximum possible positive or negative number (i.e. 999.99, -999, $8000 or $7FF).

The PF command formats the values returned from the following commands:

| BL? | IP ? | TD |
| --- | --- | --- |
| DE ? | LE ? | TE |
| DP ? | PA ? | TN |
| EM ? | PR ? | TP |
| FL ? | RL | VE |
| GP | RP |  |

Examples

'Galil DMC Code Example

:DP 2l;'  Set position of A axis for example

:TP A;'   Tell position of A in default format

21

:PF 5.2;' Change format to 5 digits of integers and 2 of decimal

:TP A

21.00

:PF-5.2;' Change format to hexadecimal

:TP A

$00015.00

PF applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

PF

FILTER/CONTROL

#### PL Pole

PLm= n

PL n,n,n,n,n,n,n,n

PLm= n

PL n,n,n,n,n,n,n,n

| Usage | PLm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | PL n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _PLm | Operand holds the value last set by the command |

Description

The PL command adds a low-pass filter in series with the PID compensation.

The crossover frequency is entered directly as an argument to PL. To maintain compatibility with earlier versions, a value less than 1 may also be specified.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 250 | 0 | 1 | Crossover frequency created by the PL command | 'Max' is a function of TM. See Remarks. n = 0 disables the Pole filter. |
|  | 0 |  | 0.9999 | 0 | 2/65,536 | Value used to generate pole filter crossover frequency | See Remarks for the equation used. n = 0 disables the Pole filter |

Remarks

At lower TM settings, the maximum pole frequency is increased. The maximum value of the PL command is determined by the value of TM according to the following equation

Max = (1/4 * 10^6) * (1/TM)

The digital transfer function of the filter is (1 - n) / (Z - n) and the equivalent continuous filter is A/(S+A) where A is the filter cutoff frequency: A=(1/T) ln (1 / n) rad/sec and T is the sample time.

##### Calculated Pole

To convert from the desired crossover (-3 dB) frequency in Hertz to the value given to PL, use the following formula

where

n is the argument given to PL (less than 1)

T is the controller's servo loop sample time in seconds (TM divided by 1,000,000) Fc is the crossover frequency in Hertz

Example: Fc=36Hz TM=1000 n=e^(-0.001*36*2*pi) =0.8

The following shows several example crossover frequencies achieved with various values of PL

| n | Fc (Hz) |
| --- | --- |
| 0 | Infinite (off) |
| 0.2 | 256 |
| 0.4 | 145 |
| 0.6 | 81 |
| 0.8 | 36 |
| 0.999 | 0 |

Examples

'Galil DMC Code Example 'Set A-axis Pole to 0.95, B-axis to 0.9, C-axis to 0.8, D-axis pole to 0.822 :PL .95,.9,.8,.822 Query all Pole values :PL ?,?,?,? 0.9527,0.8997,0.7994,0.8244 Return A Pole only :PL? 0.9527

PL applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

PL

INDEPENDENT MOTION

#### PR Position Relative

PRm= n

PR n,n,n,n,n,n,n,n

PRm= n

PR n,n,n,n,n,n,n,n

| Usage | PRm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | PR n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _PRm | Operand holds the value last set by the command |

Description

The PR command sets the incremental distance and direction of the next move. The move is referenced with respect to the current position. .

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
|  | M | N | N/A | Axis | Virtual axis to assign value |  |
| n | - 2,147,483,648 | 2,147,483,647 | N/A | 1 | Incremental distance for independent move | n = ? returns the current incremental distance specified |

Remarks

_PRm contains the current incremental distance for the specified axis.

Examples

'Galil DMC Code Example

:PR 100,200,300,400;' On the next move the A-axis will go 100 counts,

:BG;' the B-axis will go to 200 counts forward, C-axis will go 300 counts and the D-axis will go 400 counts.

:PR ?,?,?;' Return relative distances

100,200,300

:PR 500;' Set the relative distance for the A axis to 500

:BG;' The A-axis will go 500 counts on the next move while the B-axis will go its previously set relative distance.

'Galil DMC Code Example 'using PA/PR, you can query PR for the incremental distance :DP 10000 :PA 8000 :PR ? -2000

PR applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

PR

INDEPENDENT MOTION

#### PT Position Tracking

PTm= n

PT n,n,n,n,n,n,n,n

PTm= n

PT n,n,n,n,n,n,n,n

| Usage | PTm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | PT n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _PTm | Operand holds the value last set by the command |

Description

The PT command will place the controller in the position tracking mode. In this mode, the controller will allow the user to issue absolute position commands that begin motion immediately without requiring a BG command. The absolute position may be specified such that the axis will begin motion, continue in the same direction, reverse directions, or decelerate to a stop

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 1 |  | 0 | 1 | Setting for position tracking mode of motion | n = 1 enables PT mode, n = 0 disables PT mode |

Remarks

The PA command is used to give the controller an absolute position target. Motion commands other than PA are not supported in this mode. The motion profile is trapezoidal with the parameters controlled by acceleration, deceleration, and speed (AD, DC, SP).

When in the PT mode the ST command will exit the mode.

The AM and MC trip points are not valid in this mode.

MF and MR are recommended with this mode as they allow the user to specify both the absolute position, and the direction. The AP trip point may also be used.

Position Tracking is not valid on virtual axes

Examples

'Galil DMC Code Example DPA= 0;' Start position at absolute zero PTA= 1;' Start PT mode on A axis PA 1000;' Move to position 1000, motion starts right away MF 500;' Wait till position 500 reached PA -1000;' Reverse direction to move to position -1000 EN

'Galil DMC Code Example #A PT 1,1,1,1;' Enable the position tracking mode for axes A, B, C, and D ' NOTE: The BG command is not used to start the PT mode. #LOOP;' Create label #LOOP in a program. This small program will ' update the absolute position at 100 Hz. Note that the ' user must update the variables v1, v2, v3 and v4 from the ' host PC, or another thread operating on the controller. ' PA v1,v2,v3,v4;' Command ABCD axes to move to absolute positions. Motion ' begins when the command is processed. BG is not used ' to begin motion in this mode. In this example, it is ' assumed that the user is updating the variable at a ' specified rate. The controller will update the new ' target position every 10 milliseconds (WT10). WT10;' Wait 10 milliseconds JP#LOOP;' Repeat by jumping back to label LOOP

PT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

PT

PVT MODE

#### PV PVT Data

PVm= n

0

,n

1

,n

2

PVm= n

0

,n

1

,n

2

| Usage | PVm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
| Operands | _PVm | Operand has special meaning, see Remarks |

Description

The PV command is used to enter PVT data into the PVT buffer. Data is entered by specifying the target delta position, target velocity, and delta time for the segment duration.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n0 | - 30,000,000 | 30,000,000 | 0 | 1 | Position target for PVT segment |  |
| n1 | - 15,000,000 | 15,000,000 | 0 | 2 | Velocity target for PVT segment |  |
| n2 | 0 | 2,048 | 0 | 2 | Number of samples for PVT segment | n2 = -1 clears the PVT buffer, n2 = 0 exits PVT mode. See Remarks |

Remarks

n2 is in samples and sample time is defined by TM

With TM 1000 set, n2 = 1024 is equal to 1 second

If t is omitted from the PVT command, the previous n2 value is used For more details on PVT mode of motion see the user manual.

##### Operand Usage

_PVm contains the number of spaces available in the PV buffer for the specified axis. Each axis has a 255 segment PVT buffer

Examples

'Galil DMC Code Example

PVA= 100,2000,256;'   Move 100 counts over 256 samples, end at 2000 cnts per sec

PVA= 500,1000,128;'   Move 500 counts over 128 samples, end at 1000 cnts per sec

PVA= 1000,2500;'      Move 1000 counts over 128 samples, end at 2500 cnts per sec

PVA= 0,0,0;'          End PVT mode

Desired X/Y Trajectory

| X Position (relative/absolute) | X Speed at end of time period (c/s) | Time (ms at TM1000) (relative/time from start) | Y Position (relative/absolute) | Y Speed at end of time period (c/s) | Time (ms at TM1000) (relative/time from start) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0/0 | 0 | 0/0 | 0/0 | 0 | 0/0 |  |
| 100/100 | 200 | 256/256 | -50/-50 | 500 | 100/100 |  |
| 200/300 | 200 | 50/306 | -100/-150 | -100 | 510/610 |  |
| 300/600 | 0 | 50/356 | 300/150 | 0 | 50/660 |  |
| 'Galil DMC Code Example DP0,0;' Define zero position PVA=100,200,256;' Command X axis to move 100 counts reaching an ending speed of 200c/s in 256 samples PVB=-50,500,100;' Command Y axis to move -50 counts reaching an ending speed of 500c/s in 100 samples PVB=-100,-100,510;' Command Y axis to move -100 counts reaching an ending speed of -100c/s in 510 samples PVA=200,200,50;' Command X axis to move 200 counts reaching an ending speed of 200c/s in 50 samples PVA=300,0,50;' Command X axis to move 300 counts reaching an ending speed of 0c/s in 50 samples PVB=300,0,50;' Command Y axis to move 300 counts reaching an ending speed of 0c/s in 50 samples PVB=,,0;' Exit PVT mode on Y axis PVA=,,0;' Exit PVT mode on X axis ' When the PVT mode is exited, the axis will be in the "SH" state ' (assuming position error is not exceeded, etc) BTAB;' Begin PVT on X and Y axis AMAB;' Trip point will block until PVT motion on X AND Y is complete EN;' End program |  |  |  |  |  |  |

PV applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

PV

SYSTEM CONFIG

#### PW   Password

PW str,str

PW str,str

| Usage | PW n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The PW command sets the password used to lock the controller. Locking the controller prevents interrogation of the controller program space.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 0 chars | 8 chars | "" | String | String to be used for password | Both parameters must match for the PW command to succeed. |

Remarks

The password can only be changed when the controller is in the unlocked state. See the ^L^K for more details.

The password is burnable but cannot be interrogated. If you forget the password and the controller is locked you must master reset the controller to gain access.

Quotes are not used to frame the password string. If quotes are used, they are part of the password.

Examples

'Galil DMC Code Example :PWapple,orange ? :TC1 138 Passwords not identical :PWapple,apple :^L^K apple,1

'Galil DMC Code Example

:PWtest,test;'            Set password to "test"

:^L^K test,1;'            Lock the program :ED;'                     Attempt to edit program ?

:TC1

106 Privilege violation

PW applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,RIO47000,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

PW

SYSTEM CONFIG

#### QD Download Array

QD str[],n

0

,n

1

QD str[],n

0

,n

1

| Usage | QD n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The QD command transfers array data from the host computer to the controller. QD array[], start, end requires that the array name be specified along with the index of the first element of the array and the index of the last element of the array.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 7 chars | N/A | String | Name of array to receive data via download. |  |
| n0 | 0 | see Notes | 0 | 1 | Index of the first array element. | Value cannot exceed size of array - 2 |
| n1 | 1 | see Notes | see Notes | 1 | Index of the last array element. | Value cannot exceed size of array - 1. Defaults to size of array - 1. |

Remarks

Array name must be a valid, dimensioned array name followed by empty [] brackets.

The array elements may be separated by a comma ( , ), a carriage return (\r), or a carriage return and line feed (\r\n). Do not use spaces. The downloaded array is terminated by a \ character.

QD is not supported in the terminal of Galil software packages.

It is recommended to use the array download functions available through the Galil software and drivers rather than directly using the QD command.

Issuing this command will pause the output of the Data Record until the command is completed.

Examples

'Galil DMC Code Example :'From a character-buffered terminal such as Telnet or Hyperterm :DM array[3] :QD array[] 1,2,3\:LA array[3] :array[0]=? 1.0000 :array[1]=? 2.0000 :array[2]=? 3.0000 :

QD applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

QD

#### QH Query Hall State

QH mm

QH mm

| Usage | QH mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _QHm | Operand has special meaning, see Remarks |

Description

The QH command transmits the state of the Hall sensor inputs. The value is decimal and represented by a 3 bit value (see Remarks).

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to return Hall status |  |

Remarks

The 3 bit value returned by QH is defined in the table below:

| Bit | Status |
| --- | --- |
| 07 | Undefined (set to 0) |
| 06 | Undefined (set to 0) |
| 05 | Undefined (set to 0) |
| 04 | Undefined (set to 0) |
| 03 | Undefined (set to 0) |
| 02 | Hall C State |
| 01 | Hall B State |
| 00 | Hall A State |

QH should return a value from 1 through 6 as valid Hall combinations. A value of 0 or 7 is invalid when using Hall sensors and will generate a Hall error with OE set.

The valid sequence for Hall inputs is a greycode output (only one bit changes at a time): 1,3,2,6,4,5 (or 5,4,6,2,3,1)

To disable Hall error checking, set the axis to brushed with a BR 1 command.

When using an internal sine amplifier, the BA command must be issued before QH will report the Hall state status.

Operand Usage _QHm Contains the state of the Hall sensor inputs for the specified axis

Examples

'Galil DMC Code Example

QHA;'       Query the A axis Hall state

var=_QHB;'  Set a variable var equal to the B axis Hall state

'Galil DMC Code Example

:QHA;'   Query A axis Hall status

7

:TA1;'   Check for Hall errors in the amp

1

:'A 1 indicates Hall error on axis A

QH applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

QH

OPERAND ONLY

#### QQ Clear Sample Time Overflow

QQ

QQ

| Usage | QQ | Command takes no arguments |
| --- | --- | --- |
| Operands | _QQ | Operand has special meaning, see Remarks |

Description

QQ is used along with #FWERR to detect a sample overflow. Many features require that the controller perform an action every sample. Running many of these features simultaneously can lead to there not being enough time to complete every action. See remarks for a list of these features. In the event that a sample overflow occurs, the QQ command is used to report and clear this error. For the majority of applications, this will never occur. This behavior should only occur when running a low sample time (TM) and using absolute encoders with a high number of bits at a low clock frequency.

Arguments

The QQ command has no arguments.

Remarks

If the servo sample has not yet overflowed, the value of _QQ will be 0.

If the servo sample has overflowed and not yet been cleared, _QQ will be 1.

The following are features, and their associated DMC comands, which add a non-negligible amount of time to the servo sample calculations.

Serial Encoders (SI/SS)

Analog Encoders (AF)

Notch Filter (NF,NZ,NB)

Low Sample Time setting (TM)

Examples

'Galil DMC Code Example 'Code detects a checksum error and notifies the user #FWERR MG "Interrupt overflow event occurred:" ,_QQ; 'printing the _QQ operand showing that the interrupt has overflown QQ; 'returns the _QQ operand to 0 MG "Shutdown for diagnostics" EN

QQ applies to DMC4000,DMC4103,DMC30010

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

QQ

#### QR I O Data Record

QR mm

QR

QR mm

QR

| Usage | QR mm | Argument is an axis mask |
| --- | --- | --- |

Description

The QR command causes the controller to return a record of information regarding controller status.

This status information includes 4 bytes of header information and specific blocks of information as specified by the command arguments. The details of the status information is described in Chapter 4 of the user's manual.

Arguments

| Argument |  | Min | Max | Default | Resolution |  | Description |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGHSTI | ABCDEFGHSTI | Multi-Axis Mask |  | Axes/Coordinated/IO data specified to display in the data record |  | If no argument entered, mm = "ABCDEFGHSTI" |
| Argument | Value |  | Description |  |  | Notes |  |
| mm | A-H |  | Output axes A-H data record block |  |  |  |  |
|  | S |  | Output coordinated axis S data block |  |  |  |  |
|  | T |  | Output coordinated axis T data block |  |  |  |  |
|  | I |  | Output General IO data block |  |  |  |  |

Remarks

The data returned by the QR command is in binary format

Galil API has specialized commands to parse the data record packet. See the API documentation for more details.

Examples

'Galil DMC Code Example

QR A;'   Return the data record with A axis block only

QR BI;'  Return the data record with B axis block and IO block

QR ST;'  Return the data record with S and T coordinated axis blocks

QR;'    Return the data record for all axes, including IO and S and T axis blocks

QR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

QR

INTERROGATION, STEPPER MOTOR

#### QS Error Magnitude

QS mm

QS mm

| Usage | QS mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _QSm | Operand has special meaning, see Remarks |

Description

The QS command reports the magnitude of error, in drive step counts, for axes in Stepper Position Maintenance mode. A step count is directly proportional to the micro-stepping resolution of the stepper drive.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to query for step motor error magnitude | Default value used if mm is undefined. |
| m | A |  | H | N/A | Axis | Single Axis to query for error magnitude |  |

Remarks

The result of QS is modularized so that result is never greater than 1/2 the revolution of the stepper motor. Largest possible QS result = 0.5*YA*YB

If present in embedded code, command execution will jump to #POSERR when QS is equal to 3 full motor steps (_YAm * 3) QSm=? will return the current error for axis m

Operand Usage _QSm contains the error magnitude in drive step counts for the specified axis.

Examples

'Galil DMC Code Example

'For an SDM-20620 microstepping drive, query the error of B axis:

:QSB

253

:' Above shows 253 step counts of error.

:' The SDM-20620 resolution is 64 microsteps per full motor step

:' nearly four full motor steps of error.

Query the value of all axes:

:QS

0,253,0,0,0,0,0,0

:' Response shows all axes error values

QS applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

QS

#### QU UploadArray

QU str[],n

0

,n

1

,n

2

QU str[],n

0

,n

1

,n

2

| Usage | QU n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The QU command transfers array data from the controller to a host computer. The QU requires that the array name be specified along with the first element of the array and last element of the array.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 7 chars | N/A | String | Name of array to be uploaded |  |
| n0 | 0 | see Notes | 0 | 1 | Index of first array element | Value cannot exceed size of array - 2 |
| n1 | 1 | see Notes | see Notes | 1 | Index of last array element | Defaults to last element of array. Value cannot exceed size of array - 1 |
| n2 | 0 | 1 | 0 | 1 | Selects character delimiter between array elements | n2 = 0 selects CR delimiting. n2 = 1 select comma delimiting. |

Remarks

Array name must be a valid, dimensioned array name followed by empty [] brackets.

The uploaded array will be followed by a <control>Z as an end of text marker.

Issuing this command will pause the output of the Data Record until the command is completed.

Examples

'Galil DMC Code Example

DM test[10];'     Dimension a 10 element sized array

QU test[],0,1,1;' Upload first 2 elements

QU test[],8,9,1;' Upload last 2 elements (size-2 and size-1 used for n1,n2) EN

'Galil DMC Code Example

:DM array[5];'                            Dimension Array

:QU array[],0,4,1;'                       Upload Array

0.0000, 0.0000, 0.0000, 0.0000, 0.0000

:array[0]=9;'                             Set value

:array[1]=1

:QU array[],0,4,1

9.0000, 1.0000, 0.0000, 0.0000, 0.0000

:array[0]=?;'                             Alternative method to return just one array value

9.0000

QU applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

QU

INTERROGATION

#### QZ ReturnData Record information

QZ

QZ

| Usage | QZ | Command takes no arguments |
| --- | --- | --- |

Description

The QZ command is an interrogation command that returns information regarding the data record. The controller's response to this command will be the return of 4 integers separated by commas.

Arguments

QZ is an interrogation command with no parameters.

Remarks

The four fields returned by QZ represent the following:

- First field returns the number of axes.

- Second field returns the number of bytes to be transferred for general status

- Third field returns the number of bytes to be transferred for coordinated move status

- Fourth field returns the number of bytes to be transferred for axis specific information _QZ operands

| Operand | Description |
| --- | --- |
| _QZ0 | Holds the number of axes |
| _QZ1 | Holds the number of bytes to be transferred for general status |
| _QZ2 | Holds the number of bytes to be transferred for coordinated move status |
| _QZ3 | Hold the number of bytes to be transferred for axis specific information |

Examples

'Galil DMC Code Example

:QZ;'             standard DMC-4143 example response

4, 52, 26, 36

QZ applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

QZ

RA	Array

### RA str[ ],str[ ],str[ ],str[ ],str[ ],str[ ],str[ ],str[ ]

| Usage | RA n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The RA command selects the user arrays to be populated by the Record Array function. The data to be captured is specified by the RD command and time interval by the RC command.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 7 chars | N/A | String | Valid array name to use in record array function | The arrays listed correspond to the source list defined by the RD command. See Remarks |

Remarks

The array name str must be followed by the [] brackets. Those brackets must be empty.

The array name str must be a valid array defined by the DM command and reported by LA.

Examples

'Galil DMC Code Example

'  try to start record array without defining array[] :RA array[] ?

:TC1

82 Undefined array

:DM array[100]

:RA array[]

'Galil DMC Code Example

#record;'     Label

DM pos[100];' Define array

RA pos[];'    Specify Record Mode

RD _TPA;'     Specify data type for record

RC 1;'        Begin recording at 2 msec intervals

PR 1000;BG;'  Start motion

EN;'          End

'The record array mode is useful for recording the real-time motor position during motion.

'The data is automatically captured in the background and does not interrupt the program sequencer.

'The record mode can also be used for a teach or learn of a motion path.

RA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

RA

## RC

RC n

0

,n

1

RC n

0

,n

1

| Usage | RC n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _RC | Operand has special meaning, see Remarks |

Description

The RC command begins recording for the Automatic Record Array Mode. RC 0 stops recording. The record array mode loads source data specified by the RD command into the arrays defined by the RA command. The address for the array element for the next recording can be interrogated with _RD.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 | 8 | 0 | 1 | Specify the record array time interval as 2^n samples. | n0 = 0 stops recording. |
| n1 | see Notes | see Notes | 0 | 1 | Specify the number of records to perform | n1 has special rules for the maximum setting. See Remarks. |

Remarks

Do not allocate or deallocate arrays (DM,DA) while the Automatic Record Array Mode is running.

Do not attempt to download arrays from a host application while automatic record array mode is running.

n0 = non zero number automatically starts record mode. n0 = ? returns status of recording. '1' if recording, '0' if not recording.

### Second Parameter Rules

n1 specifies the last array element to use for record mode.

If arrays specified by RA have different sizes, the smallest array size is the maximum value for n1 If n1 = 0 or not specified, the maximum value is used.

A negative value for n1 specifies circular (continuous) record over array addresses 0 to (n1-1). The absolute value of the minimum n1 allowed = maximum n1 allowed

Operand Usage  _RC contains status of recording. '1' if recording, '0' if not recording.

### Setting up the record array mode

- Dimension an array/arrays for storing data. Make sure you dimension the array with the number of elements required to capture data for your application.

- Set the RA command with the arrays to be used for recording.

- Set the RD command with the data sources to be applied to the arrays. The order of your arrays entered into RA will match the order of data sources set by RD.

- Set the RC command to get the desired time between records and enable the recording.

- Monitor the _RC operand for a 0 to indicate recording is done.

Examples

'Galil DMC Code Example

#record;'             Record label

DM torque[1000];'     Define Array

RA torque[];'         Specify Array to record data

RD _TTA;'             Specify Data Type

RC 2;'                Begin recording and set 4 msec between records

JG 1000;BG;'          Begin motion

#A;JP #A,_RC=1;'      Loop until done

MG "DONE RECORDING";' Print message

EN;'                  End program

RC applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

RC

RD	 Data

### RD arg, arg, arg, arg, arg, arg, arg, arg

| Usage | variable= RD | Holds a value |
| --- | --- | --- |
| Operands | _RD | Operand has special meaning, see Remarks |

Description

The RD command specifies the data type to be captured for the Record Array (RA) mode. The data defined in this command is stored in arrays defined by the RA command at the time interval specified with the RC command.

Arguments

Valid arguments for RD command

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| arg | TIME | Time in servo samples | Value as read by the TIME command |
|  | _AFm | Analog input digital value | Data range is -32768 to 32767. The analog inputs are limited to those which correspond to an axis on the controller. Syntax Note: Unlike the operand _AFm, the symbol _AFm in the context of RD records the ADC value, not the AF setting. |
|  | _DEm | Auxiliary encoder position | _DEm and _TDm capture the same data |
|  | _OP | Output status |  |
|  | _RLm | Latched position |  |
|  | _RPm | Commanded position | _RPm and _SHm capture the same data |
|  | _SCm | Stop code |  |
|  | _SHm | Commanded position | _RPm and _SHm capture the same data |
|  | _TDm | Auxiliary encoder position | _DEm and _TDm capture the same data |
|  | _TEm | Position error |  |
|  | _TI | Input status |  |
|  | _TPm | Encoder position |  |
|  | _TSm | Switches | Only bits 0-4 valid |
|  | _TTm | Torque command | The values recorded for torque are in the range of +/- 32767 where 0 is 0 torque, -32767 is -10 volt command output, and +32767 is +10 volt. |
|  | _TVm | Filtered velocity | This value will be 64 times greater than TV command |

Remarks

Arguments listed as _XXm are valid when m is a valid axis mask

The order of args specified in RD corresponds with the array order specified in the RA command. the operand _RD contains the address for the next array element for recording.

When recording _AFm, the returned value is signed. This means that when AQ is used to set unipolar inputs, values on the upper half of the voltage range are sign extended. Anding the value with $0000FFFF will return the expected unsigned value.

Examples

'Galil DMC Code Example

DM errora[50],errorb[50];'      Define arrays

RA errora[],errorb[];'          Specify arrays to be recorded

RD _TEA,_TEB;'                  Specify data source

RC 1;'                          Begin recording, period is once every other servo sample JG 1000;BG;'                    Begin motion

RD applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

RD

#### RE Return from Error Routine

RE n

RE n

| Usage | RE n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The RE command is used to end subroutines in application code. An RE at the end of these routines causes a return to the main program. Specific automatic error subroutines require the use of the RE command to end the code correctly.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  | 1 |  | 0 | 1 | Determines state of interrupted trippoint when returning from an automatic subroutine. | n = 1 restores the interrupted trippoint. n = 0 clears the trippoint |

Remarks

The RE command is used to end the following error automatic subroutines.

| Automatic Subroutines Used | Notes |
| --- | --- |
| #AMPERR | Only when using internal amps |
| #LIMSWI |  |
| #POSERR |  |
| #SERERR | Only when equipped with serial encoder firmware support |
| #TCPERR |  |

Care should be taken to ensure the error conditions are cleared when finishing the subroutine to avoid immediate re-entering of the error routine. To avoid returning to the main program on an interrupt, use the ZS command to zero the subroutine stack, then use JP to return to the desired location in code.

RE 1 restores the trippoint that was interrupted by an automatic subroutine (like WT)

A motion trippoint like MF or MR requires the axis to be actively profiling in order to be restored with the RE 1 command.

Examples

'Galil DMC Code Example REM dummy loop #A JP #A EN #POSERR;' Begin Error Handling Subroutine MG "ERROR";' Print message SB1;' Set output bit 1 RE;' Return to main program and clear trippoint

RE applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

RE

REM   Remark

REM str

Description

REM is used for comment lines. The REM statement is NOT a controller command. Rather, it is recognized by Galil PC software, which strips away the REM lines before downloading the DMC file to the controller.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| str | String | Comment to be removed from code prior to download | This comment is not limited by the character limit of the controller, as it is never downloaded |

Remarks

REM differs from NO (or ') in the following ways:

NO (or ') comments are downloaded to the controller and REM comments are not.

NO (or ') comments take up execution time and REM comments don't; therefore, REM should be used for code that needs to run fast. REM comments cannot be recovered when uploading a program but NO (or ') comments are recovered. Thus the uploaded program is less readable with REM.

NO (or ') comments take up program line space and REM lines do not.

REM comments must be the first and only thing on a line, whereas NO (or ') can be used to place comments to the right of code (after a semicolon) on the same line.

Examples

'Galil DMC Code Example

REM This comment will be stripped when downloaded to the controller

'This comment will be downloaded and takes some execution time PRX=1000 ;'this comment is to the right of the code

REM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

REM

#### RI Return from Interrupt Routine

RI n

RI n

| Usage | RI n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The RI command is used to end the input interrupt subroutine.

The input interrupt subroutine begins with the label #ININT. An RI at the end of this routine causes a return to the main program.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  | 1 |  | 0 | 1 | Determines state of interrupted trippoint when returning from an automatic subroutine. | n = 0 clears the trippoint. n = 1 restores the interrupted trippoint. |

Remarks

To avoid returning to the main program on an interrupt, use the command ZS to zero the subroutine stack. This turns the jump subroutine into a jump only. Refer to Application Note 2418.

http://www.galil.com/download/application-note/note2418.pdf

If the program sequencer was interrupted while waiting for a trippoint, such as WT, RI 1 restores the trippoint on the return to the program. RI 0 clears the trippoint.

A motion trippoint like MF or MR requires the axis to be actively profiling in order to be restored with the RI1 command. The RI command re-enables input interrupts.

Examples

'Galil DMC Code Example

#A;II1;JP #A;EN;' Program label

#ININT;' Begin interrupt subroutine

MG "INPUT INTERRUPT";' Print Message

SB 1;' Set output line 1

RI 1;' Return to the main program and restore trippoint

RI applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

RI

INTERROGATION

#### RL Report Latched Position

RL mm

RL mm

| Usage | RL mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _RLm | Operand has special meaning, see Remarks |

Description

The RL command will return the last position captured by the latch. The latch must first be armed by the AL command and then the appropriate input must be activated. Each axis uses a specific general input for the latch input; see the AL command for information on latch inputs.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to query for latched position |  |

Remarks

The armed state of the latch can be configured using the CN command. The Latch Function works with the main or auxiliary encoder.

##### Capturing Stepper Position using the Latch

When working with a stepper motor without an encoder, the latch can be used to capture the stepper position. Follow the steps below to achieve this.

- Place a wire from the controller Step (PWM) output into the main encoder input, channel A+.

- Connect the Direction (sign) output into the channel B+ input.

- Configure the main encoder for Step/Direction using the CE command.

- The latch will now capture the stepper position based on the pulses generated by the controller.

Operand Usage  _RLm contains the latched position of the specified axis.

Examples

'Galil DMC Code Example

:JG ,5000;' Set up to jog the B-axis

:BG B;'    Begin jog

:AL B;'    Arm the B latch, assume that after about 2 seconds, input goes low

:RL B;'    Report the latch

10000

RL applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

RL

INTERROGATION

#### RP Reference Position

RP mm

RP mm

| Usage | RP mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _RPm | Operand has special meaning, see Remarks |

Description

The RP command returns the commanded reference position of the motor(s). RP command is useful when operating step motors since it provides the commanded position in steps when operating in stepper mode.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to report commanded position |  |
|  | M |  | N | N/A | Multi-Axis Mask | Virtual axes to report commanded position |  |

Remarks

The relationship between RP, TP and TE: TE equals the difference between the reference position, RP, and the actual position, TP. TE = RP - TP

_RPm contains the commanded reference position for the specified axis.

Examples

'Galil DMC Code Example

'Assume that A axis is commanded to be at the position 200

'The returned units are in quadrature counts.

:PF 7;' Position format of 7

:RP

200

:RPA

200 Return the A motor reference position

:PF-6.0;' Change to hex format

:RP

$0000C8

:position =_RPA;' Assign the variable, position, the value of RPA

'Galil DMC Code Example

'Assume that ABC and D axes are commanded to be at the positions 200, -10, 0, -110

'respectively.  The returned units are in quadrature counts.

:PF 7;' Position format of 7

:RP;'   Return A,B,C,D reference positions

200,-10,0,-110

:RPA

200 'Return the A motor reference position

:RPB

-10 'Return the B motor reference position

:PF-6.0;' Change to hex format

:RP

$0000C8,$FFFFF6,$000000,$FFFF93 'Return A,B,C,D in hex

:Position =_RPA;' Assign the variable, position, the value of RPA

'Galil DMC Code Example :GAN;' make A axis slave to N imaginary axis :GR-1;' 1:-1 gearing :SPN=10000 :PRN=10000 :BGN;' Begin motion :RPN;' Get master position 10000 :RPA;' Get slave commanded position -10000

RP applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

RP

SYSTEM CONFIG

##### RS   Reset

RS n

RS n

| Usage | RS n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _RS | Operand has special meaning, see Remarks |

Description

The RS command resets the state of the processor to its power-on condition. The previously saved state of the hardware, along with parameter values and saved program, are restored.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | -1 |  | 0 |  | 0 | 1 | Set behavior of RS command | n = 0 peforms normal reset. n = -1 performs soft master reset. See Remarks. |

RS has no arguments.

Remarks

A soft master reset performed by issuing RS -1 restores factory default settings without erasing the EEPROM. To restore saved EEPROM settings use RS with no arguments, or RS 0.

###### Operand Usage

below.

below.

_RS returns the state of the processor on its last power-up condition. The value returned is the decimal equivalent of the 4 bit binary value shown

Bit 3 For master reset error

Bit 2 For program checksum error

Bit 1 For parameter checksum error Bit 0 For variable checksum error

At startup the controller operating system verifies the firmware sector. If there is a checksum error shown by _RS in firmware, it is not loaded and the controller will boot to monitor mode.

The #AUTOERR automatic subroutine will run if this error occurs and the subroutine is located in the program space.

Examples

'Galil DMC Code Example

:RS;'     Reset the hardware

:RS-1;'   Perform a soft master reset :

RS applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

RS

IO

##### SB   Set Bit

SB n

SB n

| Usage | SB n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The SB command sets a particular digital output. The SB and CB (Clear Bit) instructions can be used to control the state of output lines.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 1 | 16 | N/A | 1 | General output bit to be set | Max value is 8 for 1-4 axis controllers |
| n | 1,000 | 8,999 | N/A | 1 | Set Modbus slave bit | See "SB via Modbus Slave" in Remarks |

Remarks

The state of the output can be read with the @OUT[] command.

Using SB with a Modbus Slave n = (SlaveAddress*10000) + (HandleNum*1000) + ((Module-1)*4) + (Bitnum-1)

Slave Address is used when the ModBus device has slave devices connected to it and specified as Addresses 0 to 255. Please note that the use of slave devices for modbus are very rare and this number will usually be 0.

HandleNum is the handle specifier where A is 1, B is 2 and so on.

Module is the position of the module in the rack from 1 to 16. BitNum is the I/O point in the module from 1 to 4

Examples

'Galil DMC Code Example

SB 5;'     Set digital output 5

SB 1;'     Set digital output 1

CB 5;'     Clear digital output 5

CB 1;'     Clear digital output 1

EN

'Galil DMC Code Example #modbus REM connect to modbus slave at IP address 192.168.42.50 IHH=192,168,42,50<502>2 WT100 SB 8001;'set bit 1 on modbus slave WT 10 CB 8003;'set bit 3 on modbus slave EN

For detailed information on connecting to a Modbus slave, see: http://www.galil.com/news/dmc-programming-io-control/setting-rio-pocket-plc-or-generic-modbus-slave-extended-io

SB applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

###### SB

INTERROGATION

SC Stop Code

SC mm

SC mm

| Usage | SC mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _SCm | Operand has special meaning, see Remarks |

Description

The Stop Code command returns a number indicating why a motor has stopped.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | N/A | Multi-Axis Mask | Axis to query stop code | Omitting argument shows stop code for all axes |

Remarks

When SC is issued, the controller reponds with a number for the axis queried. The number is interpreted as follows:

Stop Code Table

| Stop Code Number | Meaning |
| --- | --- |
| 0 | Motors are running, independent mode |
| 1 | Motors decelerating or stopped at commanded independent position |
| 2 | Decelerating or stopped by FWD limit switch or soft limit FL |
| 3 | Decelerating or stopped by REV limit switch or soft limit BL |
| 4 | Decelerating or stopped by Stop Command (ST) |
| 6 | Stopped by Abort input |
| 7 | Stopped by Abort command (AB) |
| 8 | Decelerating or stopped by Off on Error (OE1) |
| 9 | Stopped after finding edge (FE) |
| 10 | Stopped after homing (HM) or Find Index (FI) |
| 11 | Stopped by selective abort input |
| 12 | Decelerating or stopped by encoder failure (OA1) (For controllers supporting OA/OV/OT) |
| 15 | Amplifier Fault (For controllers with internal drives) |
| 16 | Stepper position maintenance error |
| 30 | Running in PVT mode |
| 31 | PVT mode completed normally |
| 32 | PVT mode exited because buffer is empty |
| 50 | Contour Running |
| 51 | Contour Stopped |
| 60 | ECAM Running |
| 61 | ECAM Stopped |
| 70 | Stopped due to EtherCAT communication failure |
| 71 | Stopped due to EtherCAT drive fault |
| 99 | MC timeout |
| 100 | Vector Sequence running |
| 101 | Vector Sequence stopped |

_SCm contains the value of the stop code for the specified axis.

Examples

'Galil DMC Code Example tom =_SCA;'      Assign the Stop Code of A axis to variable tom

'Galil DMC Code Example :JG10000 :BGA :SCA 0 //Axis is running in independent mode :STA :SCA 4 //Axis is stopped by ST command :

SC applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SC

INDEPENDENT MOTION SD Limit Switch Deceleration

### SDm= n SD n,n,n,n,n,n,n,n

| Usage | SDm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | SD n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _SDm | Operand holds the value last set by the command |

Description

The Limit Switch Deceleration command (SD) sets the linear deceleration rate of the motors when a limit switch has been reached.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | 1,024 | 1,073,740,800 | 256,000 | 1,024 | Value of switch deceleration | Resolution changes with TM, see Remarks |

Remarks

The resolution of the SD command is dependent upon the update rate setting (TM). With the default rate of TM 1000 the resolution is 1024 cnts/second^2. The equation to calculate the resolution of the AC command is: Resolution = 1024*(1000/TM)^2 Example:

With TM 500 the minimum AC setting and resolution is 4096 cnts/second^2 resolution = 1024*(1000/500)^2 = 4096

The SD command may be changed during the move in JG move, but not in PR or PA move.

Examples

'Galil DMC Code Example #main PR 10000;' Specify position AC 2000000;' Specify acceleration rate DC 1000000;' Specify deceleration rate SD 5000000;' Specify Limit Switch Deceleration Rate SP 5000;' Specify slew speed EN

SD applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SD

FILTER/CONTROL

#### SH Servo Here

SH mm

SH mm

| Usage | SH mm | Argument is an axis mask |
| --- | --- | --- |

Description

The SH commands tells the controller to use the current motor position as the command position and to enable servo control at the current position.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to enable |  |

Remarks

Examples

'Galil DMC Code Example

SH;' Servo A,B,C,D motors

SHA;' Only servo the A motor, the B,C and D motors remain in its previous state.

SHB;' Servo the B motor, leave the A,C and D motors unchanged

SHC;' Servo the C motor, leave the A,B and D motors unchanged

SHD;' Servo the D motor, leave the A,B and C motors unchanged

'Galil DMC Code Example 'show how issuing SH clears position error 'by resetting the coordinate system :MOA;' disable the A axis :TEA;' check error on A axis -12435 :TPA;' Check position 12435 :SHA;' enable A axis, doing so clears the error :TEA;' check error again 0 :TPA;' confirm position hasn't changed 12435

SH applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SH

SYSTEM CONFIG

#### SI Configure the special Galil SSI feature

SIm = n

0

, n

1

, n

2

, n

3

<o >p

SIm = n

0

, n

1

, n

2

, n

3

<o >p

| Usage | SIm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
| Operands | _SIm | Operand has special meaning, see Remarks |

Description

The SI command enables and configures the controller to read SSI encoder data. Synchronous Serial Interface (SSI) allows for serial transmission of absolute position data (either binary or Gray code) from the encoder based on a timed clock pulse train from the controller. Connection between the controller and encoder is based on two signal lines, clock and data, which are usually differential for increased noise immunity. For each sequential clock pulse of the controller, the encoder transmits one data bit from shift registers on the encoder. SSI Timing

Arguments

| Argument | Min | Max | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- |
| m | A | H | Axis | Axis to configure for SSI | Each axis has a dedicated SSI port (1) |
| n0 | 0 | 2 | 1 | Position register to use | 1 = TPA, 2 = TDA, 0 = Off |
| n1 | 12 | 31 | 1 | Number of Data and Status Bits | Sign of n1 parameter sets the position mode (2) |
| n2 | 0 | 0 | N/A | Reserved |  |
| n3 | 0 | 8 | 1 | Number of status bits | Sign of n3 parameter sets location of status bits (see timing diagram above) |
| o | 4 | 26 | 1 | Clock divider | Defines SSI Clock Frequency (3) |
| p | 1 | 2 | 1 | Data Encoding | 1 = Binary, 2 = Gray Code |

- SSI encoder support requires the SER Option on the DMC and -SER Firmware

- The firmware will use the position data in one of two modes:

Absolute Mode, n1 > 0. The controller will use the absolute position transmitted by the encoder. If the encoder exceeds its position limits, the encoder data will roll over from the maximum value to the minimum value. This discontinuity will be perceived by the controller as a large change in servo error. Absolute mode is typically used with linear encoders that have a limited range of travel.

Continuous Mode, n1< 0. In this mode if the absolute position rolls over, the firmware will compensate by accounting for the roll over and counting past it. This allows for a smooth transition, avoiding the large error that occurs in Absolute Mode. Continuous mode is typically used with rotary absolute encoders. When the SS command is issued, the data first loaded into firmware will the same as in Absolute mode.

- Galil recommends using the lowest clock divider (highest clock frequency) possible. This will mainly be dictated by the encoder specifications and thelength of the clock and data transmission lines. ** Clock frequencies (f) in MHz can be calculated from the clock divider ( o) as follows: f=10/(o+1) o = 9 is a good starting value for most applications.

Remarks

Axis must be in MO state prior to issuing the SI command.

SIm=? will return the configuration parameters for the specified axis.

Clocking in SSI data has a timing overhead which may be non-negligible. In the event that clocking in data will result in the controller being unable to complete all required operations, (e.g. using multiple encoders with a lowered TM sample rate) the controller will respond by turning of all serial encoders. See #FWERR and QQ for more information. This error mode is very rare, and is expected to occur only during development.

##### Operand Usage

_SIm returns the value of the SSI encoder's status bits as an 8 bit bitmask. SSI status bit placement and interpretation is manufacturer specific, consult the encoder documentation for further information.

Related Commands

DF - Dual Feedback

DV - Dual Velocity (Dual Loop)

TD - Tell Dual Encoder

TP - Tell Position

Examples

Related Commands

DF - Dual Feedback

DV - Dual Velocity (Dual Loop)

TD - Tell Dual Encoder

TP - Tell Position

Examples

##### SI

'Galil DMC Code Example

'SSI encoder data on the A axis sent to the _TPA register

'25 total bits, 22 position data, 3 status, prepended

'1 MHz clock frequency, binary encoding

SIA= 1,25,0,-3<9>1

'message status bits to the host

MG _SIA

EN

SI applies to SER

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SI

PROGRAMMING

#### SL Single Step

SL n

SL n

| Usage | SL n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The SL command is used to single-step through a program for debugging purposes. SL can be used after execution has paused at a breakpoint (BK). The argument n allows user to specify the number of lines to execute before pausing again.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n | 1 |  | 255 | 1 | 1 | Number of lines to execute before pausing | If n is omitted, default value used. |

Remarks

The BK command resumes normal program execution.

Examples

'Galil DMC Code Example :BK 3;' Pause at line 3 (the 4th line) in thread 0 :BK 5;' Continue to line 5 :SL;' Execute the next line :SL 3;' Execute the next 3 lines :BK;' Resume normal execution

SL applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SL

ETHERNET

#### SM Subnet Mask

SM n

0

,n

1

,n

2

,n

3

SM n

SM n

0

,n

1

,n

2

,n

3

SM n

| Usage | SM n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _SM0 | Operand has special meaning, see Remarks |

Description

The SM command assigns a subnet mask to the controller. All packets sent to the controller whose source IP address is not on the subnet will be ignored by the controller. For example, for SM 255,255,0,0 and IA 10,0,51,1, only packets from IP addresses of the form 10.0.000.000 will be accepted.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 | 255 | 0 | 1 | Byte 3 of the Subnet mask |  |
| n1 | 0 | 255 | 0 | 1 | Byte 2 of the Subnet mask |  |
| n2 | 0 | 255 | 0 | 1 | Byte 1 of the Subnet mask |  |
| n3 | 0 | 255 | 0 | 1 | Byte 0 of the Subnet mask |  |
| n | - 2,147,483,648 | 2,147,483,647 | 0 | 1 | The full subnet mask specified as a signed 32 bit two's complement integer |  |

Remarks

n = ? will return the subnet mask of the controller as n0,n1,n2,n3

_SM0 contains the subnet mask representing a 32 bit signed number (Two's complement)

Use the following equation to change the 4 byte subnet (n0,n1,n2,n3) to a single 32 bit number, n n = (n0*2^24) + (n1*2^16) + (n2*2^8) + n3

For more information, see http://www.galil.com/news/dmc-programming-software/blocking-unwanted-ethernet-devices-connecting

Examples

'Galil DMC Code Example

SM 255,255,255,255;' Ignore all incoming Ethernet packets

SM 0,0,0,0;' Process all incoming Ethernet packets

SM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,RIO47000

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SM

INDEPENDENT MOTION

SP Speed

### SPm= n SP n,n,n,n,n,n,n,n

| Usage | SPm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | SP n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _SPm | Operand holds the value last set by the command |

Description

The SP command sets the slew speed of any or all axes for independent moves.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
|  | M |  | N | N/A | Axis | Virtual axis to assign value |  |
| n | 0 |  | 15,000,000 | 25,000 | 2 | Value of jog speed in cnts/second | For MT settings of 1,-1,1.5 and -1.5 (Servos) - See Remarks for Resolution details |
|  | 0 |  | 3,000,000 | 25,000 | 2 | Value of jog speed in cnts/second | For MT settings of 2,-2,2.5 and -2.5 (Steppers) - See Remarks for Resolution details |

Remarks

Negative values will be interpreted as the absolute value

#### Resolution

The resolution of the SP command is dependent upon the update rate setting (TM).

With the default rate of TM 1000 the resolution is 2 cnts/second.

The equation to calculate the resolution of the SP command is:

resolution = 2*(1000/TM)

example:

With TM 250 the resolution of the SP command is 8 cnts/second resolution = 2*(1000/250) = 8

Examples

'Galil DMC Code Example

PR 2000,3000,4000,5000;'          Specify a,b,c,d parameter

SP 5000,6000,7000,8000;'   Specify a,b,c,d speeds

BG;'                              Begin motion of all axes

AM C;'                            After C motion is complete

'

'

'For vector moves, use the vector speed command (VS) to change the speed.

'SP is not a "mode" of motion like JOG (JG). 'Note:  2 is the minimum non-zero speed.

SP applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SP

SYSTEM CONFIG

#### SS Configure the special Galil BiSS feature

SSm = n

0

,n

1

,n

2

,n

3

< o

SSm = n

0

,n

1

,n

2

,n

3

< o

| Usage | SSm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
| Operands | _SSm | Operand has special meaning, see Remarks |

Description

The SS command enables and configures the controller to read BiSS encoder data. BiSS is an open-source, digital interface for encoders. BiSS is hardware compatible to the industrial standard SSI (Serial Synchronous Interface) but offers additional features and options.

##### BiSS Timing

During the controller's servo interrupt, a clock will be transmitted for encoders configured for BiSS. The BiSS encoder responds by transmitting a serial data stream synchronized to the controllers's clock. The data stream has four important features as described below.

- Data. Encoder position data and zero padding bits, set via n1 and n3

- Error Bit. Shown above as E, BiSS defines an Error bit that is set by the encoder. Depending on the manufacturer, this bit can be active high or low. Use the SY command to set the correct polarity for this bit. Once BiSS is enabled, the current state of the Error Bit can be checked via the _SSm0 operand.

- Warning Bit. Shown above as W, BiSS also defines a Warning bit that is set by the encoder. Depending on the manufacturer, this bit can be active high or low. Use the SY command to set the correct polarity for this bit. Once BiSS is enabled, the current state of the Error Bit can be checked via the _SSm0 operand.

- CRC. The Data, Error and Warning bits are all included in the 6 bit CRC calculation. This CRC allows end to end transmission validity. In the event that the CRC is not valid, the Galil will not update the encoder position information, and _SSm0 will show an invalid CRC. See the table in the Remarks section for more details.

Arguments

| Argument |  | Min | Max | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | Axis | Axis to configure for BiSS | Each axis has a dedicated BiSS port (1) |
| n0 | 0 |  | 2 | 1 | Position register to use | 1 = TPA, 2 = TDA, 0 = Off |
| n1 | 12 |  | 38 | 1 | Number of Data bits, including zero padding bits | Sign of n1 parameter sets the position mode (2) |
| n2 | 0 |  | 0 | 0 | Reserved |  |
| n3 | 0 |  | 7 | 1 | Number of zero padding bits | Manufacturer specific |
| o | 4 |  | 26 | 1 | Clock divider | Defines BiSS Clock Frequency (3) |

- BiSS encoder support requires the SER option on the DMC and -SER Firmware

- The firmware will interpret the encoder position data in one of two ways:

Absolute Mode, n1 > 0. The controller will use the absolute position transmitted by the encoder. If the encoder exceeds its position limits, the encoder data will roll over from the maximum value to the minimum value. This discontinuity will be perceived by the controller as a large change in servo error. Absolute mode is typically used with linear encoders that have a limited range of travel.

Continuous Mode, n1 < 0. In this mode if the absolute position rolls over, the firmware will compensate by accounting for the roll over and counting past it. This allows for a smooth transition, avoiding the large error that occurs in Absolute Mode. Continuous mode is typically used with rotary absolute encoders. When the SS command is issued, the data first loaded into firmware will be the same as in Absolute mode.

- Galil recommends using the lowest clock divider (highest clock frequency) possible. This will mainly be dictated by the encoder specifications and the length of the clock and data transmission lines.

Clock frequencies (f) in MHz can be calculated from the clock divider ( o) as follows: f=10/(o+1) o = 9 is a good starting value for most applications.

Remarks

Axis must be in MO state prior to issuing the SS command.

SSm=? Returns the configuration parameters.

Clocking in BiSS data has a timing overhead which may be non-negligible. In the event that clocking in data will result in the controller being unable to complete all required operations, (e.g. using multiple encoders with a lowered TM sample rate) the controller will respond by turning of all serial encoders. See #FWERR and QQ for more information. This error mode is very rare, and is expected to occur only during development. Axis must be in MO state prior to issuing the SS command.

##### Operand Usage

_SSm0: _SSm0 returns a 4 bit bitmask containing the status of the following _SSm Bit Map

##### SS

| Bit | Bit Meaning | 0 | 1 | Description |
| --- | --- | --- | --- | --- |
| 0 | Timeout Status | No timeout | Timeout occurred | Timeout bit will be set if the encoder doesn't set the start bit within 30us of receiving the first clock pulse. |
| 1 | CRC Status | CRC valid | CRC invalid | CRC is calculated in hardware, only the validity of the CRC is sent to firmware. |
| 2 | Warning bit | No Warning | Warning | Set active level using the SY command. |
| 3 | Error bit | No Error | Error | Set active level using the SY command. |

An _SSm0 value of 0 means that the encoder is functioning properly.

_SSm1 saves the initial position data returned from the encoder upon issuing the SS command. Used for initial configuration steps, helpful when configuring the encoder if getting an error

###### Error Handling

SERERR is an automatic subroutine which runs when any of the bits in _SSm0 are active i.e.

Encoder Timeout

Invalid CRC

Error Bit Active Warning Bit Active

Related Commands

#SERERR - Serial Encoder Error Automatic Subroutine

DF - Dual Feedback

DV - Dual Velocity (Dual Loop)

SY - Serial Encoder BiSS Active Level

TD - Tell Dual Encoder

TP - Tell Position

Examples

'Galil DMC Code Example 'BiSS encoder data on the A axis sent to the TPA register '24 total bits, 22 position data, 2 zero padding bits '1 MHz clock frequency SSA= 1,24,0,2<9 EN #SERERR 'Serial Error routine messages out to the host the type of error. sercode=_SSA0 IF (sercode & 1) MG "BiSS Timeout" ENDIF IF (sercode & 2) MG "Invalid CRC" ENDIF IF (sercode & 4) MG "Warning Bit Set" ENDIF IF (sercode & 8) MG "Error Bit Set" ENDIF RE

SS applies to SER

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SS

INDEPENDENT MOTION, VECTOR/LINEAR

#### ST Stop

ST mm

ST mm

| Usage | ST mm | Argument is an axis mask |
| --- | --- | --- |

Description

The ST command stops motion on the specified axis. Motors will come to a decelerated stop.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGHMNST | ABCDEFGH | Multi-Axis Mask | Axes to command to stop motion |  |

Remarks

If ST is sent from the host without an axis specification, program execution will stop in addition to motion.

Examples

'Galil DMC Code Example

ST A;' Stop A-axis motion

ST S;' Stop coordinate plane S

ST ABCD;' Stop A,B,C,D motion

ST SCD;' Stop coordinate plane S, as well as axes C and D

ST;' Stop motion on all axes including any virtual axes and coordinate planes 'Use the after motion complete command, AM, to wait for motion to be stopped.

'Galil DMC Code Example

:ST A;'       Stop motion on the A axis

:SC A;'       Query A axis status

4            Indicates stopped by ST command

:MG _NO;'     Check if code is running

1            Thread 0 running

:ST ;'        General stop

:MG _NO;'     check code again

0            Thread 0 stopped

ST applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ST

SYSTEM CONFIG

#### SY Serial encoder BiSS active level

SYm= n

SY n,n,n,n,n,n,n,n

SYm= n

SY n,n,n,n,n,n,n,n

| Usage | SYm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | SY n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _SYm | Operand holds the value last set by the command |

Description

This command is used to designate the active level of the Error and Warning bits when using the Galil BiSS feature. The BiSS protocol defines two bits, Error and Warning, which can be used by the encoder to signal various events. For more information, see the SS command.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |  |  |
| Argument | Value |  | Description |  |  |  |  | Notes |  |
| n | 0 |  | Warning bit = Active Low; Error bit = Active Low |  |  |  |  |  |  |
|  | 1 |  | Warning bit = Active High; Error bit = Active Low |  |  |  |  |  |  |
|  | 2 |  | Warning bit = Active Low; Error bit = Active High |  |  |  |  |  |  |
|  | 3 |  | Warning bit = Active High; Error bit = Active High |  |  |  |  | Default |  |

Remarks

The SY command should be appropriately configured to ensure that the #SERERR automatic subroutine will run when the error or warning bits are active, and that the _SSm0 operand reports the fault state of the encoder correctly. Refer to the encoder's data sheet for information regarding the active level of these bits.

Examples

'Galil DMC Code Example SYA=1;' Waring bit is active low, Error is active high SSA=1,32,0,0<9;' Set up a BiSS encoder on axis A, 32 bits of data, 0 padding, 1 MHz clock frequency. EN #SERERR 'Serial Error routine messages out to the host the type of error. sercode=_SSA0 IF (sercode & 1) MG "BiSS Timeout" ENDIF IF (sercode & 2) MG "Invalid CRC" ENDIF IF (sercode & 4) MG "Warning Bit Set" ENDIF IF (sercode & 8) MG "Error Bit Set" ENDIF RE

SY applies to SER

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

SY

INTERROGATION, ERROR CONTROL

#### TA Tell amplifier error status

TA n

TA n

| Usage | TA n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _TA0 _TA1 _TA2 _TA3 | Operand has special meaning, see Remarks |

Description

The command returns the amplifier error status. The value is decimal and represents an 8 bit value. Bit 7 is the most significant bit. Bit 0 is the least significant bit.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  | 3 |  | N/A | 1 | Selects amp status byte to return |  |

##### D3040, D3240

Tell Amplifier Error Status Bit Definition

| Bit # | Decimal | TA0 | TA1 | TA2 | TA3 |
| --- | --- | --- | --- | --- | --- |
| 7 | 128 | Under-Voltage (E-H Axes) | Hall Error H Axis | Peak Current H Axis |  |
| 6 | 64 | Over-Temperature (E-H Axes) | Hall Error G Axis | Peak Current G Axis |  |
| 5 | 32 | Over-Voltage (E-H Axes) | Hall Error F Axis | Peak Current F Axis |  |
| 4 | 16 | Over-Current (E-H Axes) | Hall Error E Axis | Peak Current E Axis |  |
| 3 | 8 | Under-Voltage (A-D Axes) | Hall Error D Axis | Peak Current D Axis |  |
| 2 | 4 | Over-Temperature (A-D Axes) | Hall Error C Axis | Peak Current C Axis |  |
| 1 | 2 | Over-Voltage (A-D Axes) | Hall Error B Axis | Peak Current B Axis | ELO Active (E-H Axes) |
| 0 | 1 | Over-Current (A-D Axes) | Hall Error A Axis | Peak Current A Axis | ELO Active (A-D Axes) |

##### D3140

Tell Amplifier Error Status Bit Definition

| Bit # | Decimal | TA0 | TA1 | TA2 | TA3 |
| --- | --- | --- | --- | --- | --- |
| 7 | 128 |  |  | Peak Current H Axis |  |
| 6 | 64 |  |  | Peak Current G Axis |  |
| 5 | 32 |  |  | Peak Current F Axis |  |
| 4 | 16 |  |  | Peak Current E Axis |  |
| 3 | 8 |  |  | Peak Current D Axis |  |
| 2 | 4 |  |  | Peak Current C Axis |  |
| 1 | 2 |  |  | Peak Current B Axis | ELO Active (E-H Axes) |
| 0 | 1 |  |  | Peak Current A Axis | ELO Active (A-D Axes) |

##### D3540

Tell Amplifier Error Status Bit Definition

| Bit # | Decimal | TA0 | TA1 | TA2 | TA3 |
| --- | --- | --- | --- | --- | --- |
| 7 | 128 | Under-Voltage (E-H Axes) | Hall Error H Axis | Peak Current H Axis | Error latched (E-H Axes)1 |
| 6 | 64 | Over-Temperature (E-H Axes) | Hall Error G Axis | Peak Current G Axis | Error latched (A-D Axes)1 |
| 5 | 32 | Over-Voltage (E-H Axes) | Hall Error F Axis | Peak Current F Axis |  |
| 4 | 16 | Over-Current (E-H Axes) | Hall Error E Axis | Peak Current E Axis |  |
| 3 | 8 | Under-Voltage (A-D Axes) | Hall Error D Axis | Peak Current D Axis |  |
| 2 | 4 | Over-Temperature (A-D Axes) | Hall Error C Axis | Peak Current C Axis |  |
| 1 | 2 | Over-Voltage (A-D Axes) | Hall Error B Axis | Peak Current B Axis | ELO Active (E-H Axes) |
| 0 | 1 | Over-Current (A-D Axes) | Hall Error A Axis | Peak Current A Axis | ELO Active (A-D Axes) |

- Only available after AZ2 is issued to the controller.

- Amplifier errors for a bank of axes will begin reporting after the BA command is issued for an axis.

##### D3547

Tell Amplifier Error Status Bit Definition

| Bit # | Decimal | TA0 | TA1 | TA2 | TA3 |
| --- | --- | --- | --- | --- | --- |
| 7 | 128 | Under-Voltage (E-H Axes) | Hall Error H Axis | Peak Current H Axis | Error latched (E-H Axes) |
| 6 | 64 | Over-Temperature (E-H Axes) | Hall Error G Axis | Peak Current G Axis | Error latched (A-D Axes) |
| 5 | 32 | Over-Voltage (E-H Axes) | Hall Error F Axis | Peak Current F Axis |  |
| 4 | 16 | Over-Current (E-H Axes) | Hall Error E Axis | Peak Current E Axis |  |
| 3 | 8 | Under-Voltage (A-D Axes) | Hall Error D Axis | Peak Current D Axis |  |
| 2 | 4 | Over-Temperature (A-D Axes) | Hall Error C Axis | Peak Current C Axis |  |
| 1 | 2 | Over-Voltage (A-D Axes) | Hall Error B Axis | Peak Current B Axis | ELO Active (E-H Axes) |
| 0 | 1 | Over-Current (A-D Axes) | Hall Error A Axis | Peak Current A Axis | ELO Active (A-D Axes) |

##### D3640

Tell Amplifier Error Status Bit Definition

| Bit # | Decimal | TA0 | TA1 | TA2 | TA3 |
| --- | --- | --- | --- | --- | --- |
| 7 | 128 |  | Hall Error H Axis | Peak Current H Axis |  |
| 6 | 64 | Over-Temperature (E-H Axes) | Hall Error G Axis | Peak Current G Axis |  |
| 5 | 32 |  | Hall Error F Axis | Peak Current F Axis |  |
| 4 | 16 |  | Hall Error E Axis | Peak Current E Axis |  |
| 3 | 8 |  | Hall Error D Axis | Peak Current D Axis |  |
| 2 | 4 | Over-Temperature (A-D Axes) | Hall Error C Axis | Peak Current C Axis |  |
| 1 | 2 |  | Hall Error B Axis | Peak Current B Axis | ELO Active (E-H Axes) |
| 0 | 1 |  | Hall Error A Axis | Peak Current A Axis | ELO Active (A-D Axes) |

##### D4040

Tell Amplifier Error Status Bit Definition

| Bit # | Decimal | TA0 | TA1 | TA2 | TA3 |
| --- | --- | --- | --- | --- | --- |
| 7 | 128 |  |  |  |  |
| 6 | 64 |  |  |  |  |
| 5 | 32 |  |  |  |  |
| 4 | 16 | Over-Current (E-H Axes)1 |  |  |  |
| 3 | 8 |  |  |  |  |
| 2 | 4 |  |  |  |  |
| 1 | 2 |  |  |  | ELO Active (E-H Axes) |
| 0 | 1 | Over-Current (A-D Axes)1 |  |  | ELO Active (A-D Axes) |

1. An Over-Current error will report if the D4040 is not powered

##### D4140

Tell Amplifier Error Status Bit Definition

| Bit # | Decimal | TA0 | TA1 | TA2 | TA3 |
| --- | --- | --- | --- | --- | --- |
| 7 | 128 | Under-Voltage (E-H Axes) |  |  |  |
| 6 | 64 |  |  |  |  |
| 5 | 32 |  |  |  |  |
| 4 | 16 | Over-Current (E-H Axes) |  |  |  |
| 3 | 8 | Under-Voltage (A-D Axes) |  |  |  |
| 2 | 4 |  |  |  |  |
| 1 | 2 |  |  |  | ELO Active (E-H Axes) |
| 0 | 1 | Over-Current (A-D Axes) |  |  | ELO Active (A-D Axes) |

1. This will also be reported in the case of Under-Voltage

Remarks

Refer to the controller User Manual for more details on clearing amplifier errors.

_TAn Contains the amplifier error status. n = 0,1,2, or 3.

If a brushed-type servo motor is disabling and TA1 shows a Hall error, use the BR command to set the axis as a brushed axis. This causes the controller to ignore invalid Hall states.

Related Commands

#AMPERR - Amplifier error automatic subroutine

AG - Amplifier Gain

AU - Set amplifier current loop

AZ - Clear Amplifier Errors

MT - Motor Type

TK - Peak Torque Limit

TL - Torque Limit

Examples

'Galil DMC Code Example

#AMPERR

ST;' stop motion on all axes

AM;' wait until motion is halted

MO;WT2;' disable all axes

IF((_TA0&1)|(_TA0&16));' check if an Over-Current error occured

MG "Over-Current amplifier error" ENDIF

IF((_TA0&2)|(_TA0&32));' check if an Over-Voltage error occured

IF((_TA0&2)|(_TA0&32));' check if an Over-Voltage error occured

MG "Over-Voltage amplifier error" ENDIF IF((_TA0&4)¦(_TA0&64));' check if an Over-Temperature error occured MG "Over-Temperature amplifier error" ENDIF IF((_TA0&8)¦(_TA0&128));' check if an Under-Voltage error occured MG "Under-Voltage amplifier error" ENDIF IF((_TA3&1)¦(_TA3&2));' check if the ELO input was asserted MG "ELO input asserted" ENDIF IF((_TA3&64)¦(_TA3&128));' check if an amplifier error has latched MG "Amplifier error has latched" AZ1;' clear latched amplifier errors ENDIF RE

'Galil DMC Code Example

:TA1

1   'bit 0 means Hall error for A axis

:TA0

8   'bit 3 means under voltage error for amp

TA applies to DMC50000,DMC4000,DMC4103,DMC30010,DMC2103,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

INTERROGATION

#### TB   Tell Status Byte

TB

TB

| Usage | TB | Command takes no arguments |
| --- | --- | --- |
| Operands | _TB | Operand has special meaning, see Remarks |

Description

The TB command returns status information from the controller as a decimal number. Each bit of the status byte denotes an active condition when the bit is set (high):

Arguments

The following table describes the specific conditions reported with each bit of the TB report.

Tell Status Byte Response Bit Description

| Bit # | Status |
| --- | --- |
| Bit 7 | Executing application program |
| Bit 6 | N/A |
| Bit 5 | Contouring |
| Bit 4 | Executing error or limit switch routine |
| Bit 3 | Input Interrupt enabled |
| Bit 2 | Executing input interrupt routine |
| Bit 1 | N/A |
| Bit 0 | Echo on |

Remarks

_TB Contains the status byte reported by the TB command

Examples

'Galil DMC Code Example

:TB

33'      Contouring on and Echo is on (2^5 + 2^0 = 32 + 1 = 33)

'Galil DMC Code Example

:TB;'      Tell status information

129'     Executing program and echo on (2^7 + 2^0 = 128 + 1 = 129)

TB applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,RIO47000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TB

INTERROGATION, ERROR CONTROL

##### TC   Tell Error Code

TC n

TC n

| Usage | TC n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _TC | Operand has special meaning, see Remarks |

Description

The TC command reports programming or command errors detected by the controller. The TC command returns a number between 1 and 255. This number is a code that reflects why a command was not accepted by the controller. This command is useful when the controller halts execution of a program or when the response to a command is a question mark.

Arguments

| Argument |  | Value | Description | Notes |
| --- | --- | --- | --- | --- |
| n | 0 |  | Return the numerical code only | Default |
|  | 1 |  | Return the numerical code and human-readable message |  |

TC Error Code List

| Tell Code Number | Description |  | Notes |
| --- | --- | --- | --- |
| 1 | Unrecognized command |  |  |
| 2 | Command only valid from program |  |  |
| 3 | Command not valid in program |  |  |
| 4 | Operand error |  |  |
| 5 | Input buffer full |  |  |
| 6 | Number out of range |  |  |
| 7 | Command not valid while running |  |  |
| 8 | Command not valid while not running |  |  |
| 9 | Variable error |  |  |
| 10 | Empty program line or undefined label |  |  |
| 11 | Invalid label or line number |  |  |
| 12 | Subroutine more than 16 deep |  |  |
| 13 | JG only valid when running in jog mode |  |  |
| 14 | EEPROM check sum error |  |  |
| 15 | EEPROM write error |  |  |
| 16 | IP incorrect sign during position move or IP given during forced deceleration |  |  |
| 17 | ED, BN and DL not valid while program running |  |  |
| 18 | Command not valid when contouring |  |  |
| 19 | Application strand already executing |  |  |
| 20 | Begin not valid with motor off |  |  |
| 21 | Begin not valid while running |  |  |
| 22 | Begin not possible due to Limit Switch |  |  |
| 24 | Begin not valid because no sequence defined |  |  |
| 28 | S operand not valid |  |  |
| 29 | Not valid during coordinated move |  |  |
| 30 | Sequence Segment Too Short |  |  |
| 31 | Total move distance in a sequence > 2 billion |  |  |
| 32 | Segment buffer full |  |  |
| 33 | VP or CR commands cannot be mixed with LI commands |  |  |
| 39 | No time specified |  |  |
| 41 | Contouring record range error |  |  |
| 42 | Contour data being sent too slowly |  |  |
| 46 | Gear axis both master and follower |  |  |
| 50 | Not enough fields |  |  |
| 51 | Question mark not valid |  |  |
| 52 | Missing " or string too long |  |  |
| 53 | Error in {} |  |  |
| 54 | Question mark part of string |  |  |
| 55 | Missing [ or [] |  |  |
| 56 | Array index invalid or out of range |  |  |
| 57 | Bad function or array |  |  |
| 58 | Bad command response | i.e._GNX |  |
| 59 | Mismatched parentheses |  |  |
| 60 | Download error - line too long or too many lines |  |  |
| 61 | Duplicate or bad label |  |  |
| 62 | Too many labels |  |  |
| 63 | IF statement without ENDIF |  |  |
| 66 | Array space full |  |  |
| 67 | Too many arrays or variables |  |  |
| 80 | Record mode already running |  |  |
| 81 | No array or source specified |  |  |
| 82 | Undefined Array |  |  |
| 83 | Not a valid number |  |  |
| 84 | Too many elements |  |  |
| 90 | Only A B C D valid operand |  |  |
| 97 | Bad Binary Command Format |  |  |
| 98 | Binary Commands not valid in application program |  |  |
| 99 | Bad binary command number |  |  |
| 100 | Not valid when running ECAM |  |  |
| 101 | Improper index into ET |  |  |
| 102 | No master axis defined for ECAM |  |  |
| 103 | Master axis modulus greater than 256 EP value |  |  |
| 104 | Not valid when axis performing ECAM |  |  |
| 105 | EB1 command must be given first |  |  |
| 106 | Privilege Violation |  |  |
| 110 | No hall effect sensors detected |  |  |
| 111 | Must be made brushless by BA command |  |  |
| 112 | BZ command timeout |  |  |
| 113 | No movement in BZ command |  |  |
| 114 | BZ command runaway |  |  |
| 118 | Controller has GL1600 not GL1800 |  |  |
| 119 | Not valid for axis configured as stepper |  |  |
| 120 | Bad Ethernet transmit | not valid for PCI |  |
| 121 | Bad Ethernet packet received | not valid for PCI |  |
| 123 | TCP lost sync | not valid for PCI |  |
| 124 | Ethernet handle already in use | not valid for PCI |  |
| 125 | No ARP response from IP address | not valid for PCI |  |
| 126 | Closed Ethernet handle | not valid for PCI |  |
| 127 | Illegal Modbus function code | not valid for PCI |  |
| 128 | IP address not valid | not valid for PCI |  |
| 130 | Remote IO command error | not valid for PCI |  |
| 131 | Serial Port Timeout | not valid for PCI, See Remarks |  |
| 132 | Analog inputs not present |  |  |
| 133 | Command not valid when locked / Handle must be UDP | not valid for PCI |  |
| 134 | All motors must be in MO for this command |  |  |
| 135 | Motor must be in MO |  |  |
| 136 | Invalid Password |  |  |
| 137 | Invalid lock setting |  |  |
| 138 | Passwords not identical |  |  |
| 140 | Serial encoder error | Valid for BiSS support |  |
| 141 | Feature not supported |  |  |
| 143 | TM timed out | Valid on SER firmware (SSI and BiSS) |  |
| 144 | Incompatible with encoder type |  |  |
| 160 | BX failure |  |  |
| 161 | Sine amp axis not initialized |  |  |
| 163 | IA command not valid when DHCP mode enabled |  |  |
| 164 | Exceeded maximum sequence length, BGS or BGT is required |  |  |
| 165 | Cannot have both SINE and SSI feedback enabled at once | DMC-30000 only |  |
| 166 | Unable to set analog output | 30000 Hardware, see AO |  |

Remarks

TC command accepts ? as a query. This is equivalent to TC or TC 0 After TC has been read, the error code is set to zero.

_TC contains the value of the error code. Use of the operand does not clear the error code.

Note: Error code 131 means that an RS232/USB timeout is being generated while trying to transmit data to the serial port.

This is usually caused by MG. Numerous timeouts on serial communication can cause a slowdown in DMC code execution and should be avoided.

Examples

'Galil DMC Code Example

:GF32;'   Bad command

?

:TC1;'    Tell error code 1       Unrecognized command :

TC applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC52000,RIO57400,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### TD	Dual Encoder

TD mm

TD mm

| Usage | TD mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _TDm | Operand has special meaning, see Remarks |

Description

The TD command returns the current position of the dual (auxiliary) encoder input. When operating with stepper motors, the TD command returns the number of counts that have been output by the controller.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to report dual (auxiliary) encoder position. |  |

Remarks

Auxiliary encoders are not available for a stepper axis or for the axis where output compare is used.

Operand Usage _TDm reports the dual encoder position for the specified axis.

Examples

'Galil DMC Code Example

:TD;' Return A,B,C,D Dual encoders

200, -10, 0, -110

:TDA;' Return the A motor Dual encoder

200

:DUAL=_TDA;' Assign the variable, DUAL, the value of TDA

TD applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TD

#### TE	Error

TE mm

TE mm

| Usage | TE mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _TEm | Operand has special meaning, see Remarks |

Description

The TE command returns the current error in the control loop.

The command returns the position error of the motor(s), which is the difference between commanded (RP) and actual (TP) position.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to report position error |  |

Remarks

Under normal operating conditions with servo control, the position error should be small. The position error is typically largest during acceleration and deceleration.

The Tell Error command is not valid for step motors since they operate open-loop.

Operand Usage _TEm contains the current position error value for the specified axis.

Examples

'Galil DMC Code Example

:TE;' Return all position errors

5, -2, 0, 6

:TEA;' Return the A motor position error

5

:TEB;' Return the B motor position error

-2

:Error =_TEA;' Sets the variable, Error, with the A-axis position error

TE applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,RIO47000,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TE

#### TH	Ethernet Handle

TH

TH

| Usage | TH | Command takes no arguments |
| --- | --- | --- |

Description

The TH command returns a list of data pertaining to the Galil's Ethernet connection. This list begins with the IP address and Ethernet address (physical address), followed by the status of each handle indicating connection type and IP address.

Arguments

TH is an interrogation command with no parameters

Remarks

If no handles are shown as AVAILABLE, the controller will be unable to create or accept more Ethernet connections with TCP or UDP. Ping will still function when all handles are taken.

Related Commands

DH - DHCP Client Enable

IA - IP Address

IH - Open IP Handle

WH - Which Handle

Examples

'Galil DMC Code Example

:TH

CONTROLLER IP ADDRESS 10,51,0,87 ETHERNET ADDRESS 00-50-4C-08-01-1F

IHA TCP PORT 1050 TO IP ADDRESS 10,51,0,89 PORT 1000

IHB TCP PORT 1061 TO IP ADDRESS 10,51,0,89 PORT 1001

IHC TCP PORT 1012 TO IP ADDRESS 10,51,0,93 PORT 1002

IHD TCP PORT 1023 TO IP ADDRESS 10,51,0,93 PORT 1003

IHE TCP PORT 1034 TO IP ADDRESS 10,51,0,101 PORT 1004

IHF TCP PORT 1045 TO IP ADDRESS 10,51,0,101 PORT 1005

IHG AVAILABLE

IHH AVAILABLE

TH applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TH

INTERROGATION, IO

##### TI   Tell Inputs

TI n

TI n

| Usage | TI n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _TI0 _TI1 _TI10 _TI11 | Operand has special meaning, see Remarks |

Description

The TI command returns the state of the inputs in banks of 8 bits, or 1 byte. The value returned by this command is decimal and represents an 8 bit value (decimal value ranges from 0 to 255). Each bit represents one input where the LSB is the lowest input number and the MSB is the highest input bit.

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Report status of Inputs 1-8 | Default |
|  | 1 | Report status of Inputs 9-16 | Only valid for 5-8 axis controllers |
|  | 10 | Report status of Inputs 81-88 | Auxiliary encoder inputs. See Remarks |
|  | 11 | Report status of Inputs 89-96 | Auxiliary encoder inputs. Only valid for 5-8 axis controllers. See Remarks |

Remarks

For n = 10 and n = 11, the auxiliary encoder channels A and B can be used as additional IO. Only 2 * the number of axes worth of inputs are available.

See the User manual for more details.

###### Operand Usage

_TIn contains the status byte of the input block specified by 'n'.

Note that the operand can be masked to return only specified bit information - see section on Bit-wise operations.

Examples

'Galil DMC Code Example :TI1;' Tell input state on bank 1 8 Bit 3 is high, others low :TI0 0 All inputs on bank 0 low :Input=_TI1;' Sets the variable, Input, with the TI1 value :Input=? 8.0000

TI applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,RIO57400,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TI

OPERAND ONLY

#### TIME   Time Operand

TIME

TIME

| Usage | variable= TIME | Holds a value |
| --- | --- | --- |
| Operands | TIME | Operand has special meaning, see Remarks |

Description

The TIME operand returns the value of the internal free running, real time clock.

The returned value represents the number of servo loop updates and is based on the TM command. The default value for the TM command is 1000. With this update rate, the operand TIME will increase by 1 count every update of approximately 1000usec. The clock is reset to 0 with a standard reset or a master reset.

Arguments

TIME is an operand and has no parameters

Remarks

The keyword, TIME, does not require an underscore (_) as with the other operands.

TIME will increment up to +2,147,483,647 before rolling over to -2,147,483,648 and continuing to count up.

TIME rollover occurs after ~24-25 days of on-time at TM 1000 with no reset.

TM 1000 will actually set an update rate of 976 microseconds. Thus the value returned by the TIME operand will be off by 2.4% of the actual time.

Examples

'Galil DMC Code Example

MG TIME;'  Display the value of the internal clock t1=TIME;'  Sets the variable t1 to the TIME value

TIME applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,RIO47000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TIME

#### TK Peak Torque Limit

TKm= n

TK n,n,n,n,n,n,n,n

TKm= n

TK n,n,n,n,n,n,n,n

| Usage | TKm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | TK n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _TKm | Operand holds the value last set by the command |

Description

The TK command sets the peak torque limit on the motor command output. This command works with the TL command which sets the continuous torque limit. When the average torque is below TL, the motor command signal can go up to the TK (Peak Torque) limit for a short amount of time.

To convert motor command output (V) to actual motor current (A), use the following equation.

For Galil controllers with internal drives, refer to AG command for amplifier gain setting. For external drive control, consult drive documentation.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 9.9982 | 0 | 20/65,536 | Value of peak torque limit | n = 0 disables the peak torque limit |

Remarks

TK provides the absolute value of the peak torque limit for -/+ torque outputs

Peak torque can be achieved for approximately 1000 samples upon initial command from 0V torque

If TK is set lower than TL, then TL is the maximum command output under all circumstances TK should be set after the amplifier gain is selected

Related Commands

#AMPERR - Amplifier error automatic subroutine

AG - Amplifier Gain

AU - Set amplifier current loop

AZ - Clear Amplifier Errors

MT - Motor Type

TA - Tell Amplifier Error

TL - Torque Limit

Examples

'Galil DMC Code Example

TLA= 7;'   Limit A-axis to a 7 volt average torque output

TKA= 9.99;'  Limit A-axis to a 9.99 volt peak torque output

TK applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TK

#### TL Torque Limit

TLm= n

TL n,n,n,n,n,n,n,n

TLm= n

TL n,n,n,n,n,n,n,n

| Usage | TLm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | TL n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _TLm | Operand holds the value last set by the command |

Description

The TL command sets the limit on the motor command output. This limit is designed to prevent over current to motors with lower current rating than the drive.

To convert motor command output (V) to actual motor current (A), use the following equation.

For Galil controllers with internal drives, refer to AG command for amplifier gain setting.

TL works along with the TK (Peak torque) command to control output current to the motor.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | 0 | 9.9982 | 9.9982 | 20/65,536 | Value of torque limit |  |

##### TL With Internal Drives

When using the maximum AG setting, the maximum torque limit will automatically be lowered to ensure the amplifier is limited to its rated continuous current.

The maximum torque limit is different for certain amplifiers when configured for its maximum AG setting.

| Amplifier | AG setting | TL limit |
| --- | --- | --- |
| D3040 | 2 | 7 |
| D3240 | 2 | 5 |
| D3540 | 2 | 5 |
| D3547 | 2 | 5 |
| D3640 | N/A | 5 |

Remarks

TL sets the absolute torque maximum for negative and positive torque.

For example, TL of 5 limits the motor command output to 5 volts maximum and -5 volts minimum. TL should be set after the amplifier gain is selected.

Related Commands

#AMPERR - Amplifier error automatic subroutine

AG - Amplifier Gain

AU - Set amplifier current loop

AZ - Clear Amplifier Errors

MT - Motor Type

TA - Tell Amplifier Error

TK - Peak Torque Limit

Examples

'Galil DMC Code Example

:TL 1,5,9,7.5;' Limit A-axis to 1 volt. Limit B-axis to 5 volts. Limit C-axis to 9 volts. Limit D-axis to 7.5 volts. :TL ?,?,?,?;' Return limits

1.0000,5.0000,9.0000,7.5000

:TL ?;'  Return A-axis limit 1.0000

TL applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TL

#### TM Update Time

TM n

TM n

| Usage | TM n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _TM | Operand holds the value last set by the command |

Description

The TM command sets the sampling period of the control loop. The units of this command are microseconds. A negative number turns off the servo loop.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n | 125 | 5,000 | 1,000 | 31.25 | Set the sample time in usecs | The minimum value varies based on axis count and firmware usage. See Remarks |

Remarks

TM 1000 will actually set an update rate of 976 microseconds. Thus the value returned by the TIME operand will be off by 2.4% of the actual time. If a higher sampling frequency is required, please contact Galil.

The minimum allowed TM setting for the controller is listed in the tables below.

The following commands are automatically scaled to adjust for changes in sample time.

AC

AS

AT

DC

FA FV

HV

JG

KP

NB

NF

NZ

PL

SD

SP

VA

VD

VS

WT

The following commands are NOT automatically scaled to adjust for changes in sample time

BW

DR

DT

IT

KD

KI

TIME

TK TV

TW

For more infomation see:

[http://www.galil.com/news/dmc-programming-motion-controllers/time-based-commands-accelera-motion-controllers]

| Axis Count | Minimum TM |
| --- | --- |
| 1-2 | 125 |
| 3-4 | 250 |
| 5-6 | 375 |
| 7-8 | 500 |

Examples

'Galil DMC Code Example

:TM 2000;' Set sample rate to 2000 usec :TM 1000;' Return to default sample rate :

TM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TM

VECTOR/LINEAR

#### TN Vector Tangent

TN n

0

,n

1

TN n

0

,n

1

| Usage | TN n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _TNm | Operand has special meaning, see Remarks |

Description

The TN command describes the tangent axis to the coordinated motion path. n0 is the scale factor in counts/degree of the tangent axis. n1 is the absolute position of the tangent axis where the tangent axis is aligned with zero degrees in the coordinated motion plane. The tangent function is useful for cutting applications where a cutting tool must remain tangent to the part.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| n0 | -127 | 127 | 0 | 0.004 | Scale factor in counts/degree of the tangent axis |  |
| n1 | -8,388,608 | 8,388,607 | 0 | 1 | Absolute position of tangent axis where the tangent angle is 0 |  |

Remarks

When operating with stepper motors, n0 is the scale factor in steps / degree

The tangent axis is specified with the VMm0m1m2 command where m2 is the tangent axis. For example, VMABD specifies the D axis as the tangent axis

##### Operand Usage

_TNm (where m = S or T) contains the first position value for the tangent axis in the specified vector plane. This allows the user to correctly position the tangent axis before the motion begins.

_TNm will change based upon the vector path described in the VM declaration. See the example below. n0 = ? also reports this value

Examples

Use a 2D table with a tangent cutting blade to cut a half circle. Ensure that the blade is oriented before turning on the saw. The saw is activated with output 1.

'Galil DMC Code Example #EXAMPLE VM XYZ;' Z axis is tangent VSS=500;' Set vector speed m=1000/360;' Z axis encoder is 1000 counts per full revolution n=0;' When TPZ=0, blade is oriented to cut along X axis TN m,n;' Set these tangent characteristics CR 1000,0,180;' Profile a circle with radius 1000 counts, ' starting at 0 degrees ' and spanning 180 degrees VE;' End the vector path MG_TNS;' Print the calculated initial tangent entry point (250) PAZ=_TNS;' Profile a move to orient the Z axis to begin BGZ;' Move the blade into place AMZ;' Wait until the blade motion is done SB1;' Turn on the saw WT1000;' Wait for saw to spin up BGS;' Begin vector motion, saw will stay tangent AMS;' Wait for the cut to complete CB0;' Turn off the saw MG "ALL DONE";' Print a message EN

TN applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC50000,DMC52000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TN

INTERROGATION

##### TP   Tell Position

TP mm

TP mm

| Usage | TP mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _TPm | Operand has special meaning, see Remarks |

Description

The TP command returns the current position of the motor.

#### Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to report motor position |  |

#### Remarks

_TPm contains the current position value for the specified axis. Omitting mm returns the position of all axes.

#### Examples

'Galil DMC Code Example

'Assume the A-axis is at the position 200 (decimal), the B-axis is at the position -10 (decimal)

'the C-axis is at position 0, and the D-axis is at -110 (decimal).  The returned parameter units are in quadrature counts.

:PF 7;' Position format of 7

:TP;' Return A,B,C,D positions

200, -10, 0, -110

:TPA;' Return the A motor position

200

:TPB;' Return the B motor position

-10

:PF-6.0;' Change to hex format

:TP;' Return A,B,C,D in hex

$0000C8,$FFFFF6,$000000,$FFFF93

:Position =_TPA;' Assign the variable, Position, the value of TPA

TP applies to DMC4000,DMC4103,DMC4200,DMC30010,DMC50000,DMC52000,EDD37010,DMC1802,DMC1806,DMC2103,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TP

##### TR Trace

TR n

0

, n

1

TR n

0

, n

1

| Usage | TR n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The TR command causes each instruction in a program to be sent out the communications port prior to execution. The trace command is useful in debugging programs.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 |  | 1 | 0 | 1 | Set status of trace function | n0 = 0 or null disables Trace. n0 = 1 enables trace. |
| n1 | 0 |  | 255 | 255 | 1 | Set threads to trace by bitmask | See Remarks |

Remarks

n1 sets a 1-byte bitmask which determines which threads will run. Bit n set corresponds to thread n traced.

For example, setting bit 2 and 3 sets TR to trace threads 2 and 3. (2^2 + 2^3 = 4 + 8 = 12. TR 1,12 is issued) Omitting n1 sets it to the default maximum value to enable trace on all threads.

Examples

'Galil DMC Code Example :'Turn on trace during a program execution :LS - MGTIME - WT1000 - JP0 3 :XQ : 18003461.0000 18004461.0000 18005461.0000 :TR1 : 2 JP0 - MGTIME 18006461.0000 - WT1000 - JP0 - MGTIME 18007461.0000 - WT1000 :TR0 : 18008461.0000 18009461.0000 :ST :

TR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TR

##### TS	Switches

TS mm

TS mm

| Usage | TS mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _TSm | Operand has special meaning, see Remarks |

Description

The TS command returns information including axis-specific IO status, error conditions, motor condition and state. The value returned by this command is decimal and represents an 8 bit value (decimal value ranges from 0 to 255).

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to report axis switches |  |

Remarks

Each bit of the TS response represents the following status information when the bit is set (1).

| Bit # | Status |
| --- | --- |
| Bit 7 | Axis in motion |
| Bit 6 | Position error exceeds error limit |
| Bit 5 | Motor off |
| Bit 4 | Reserved (0) |
| Bit 3 | Forward Limit switch inactive |
| Bit 2 | Reverse Limit switch inactive |
| Bit 1 | Home switch status |
| Bit 0 | Position Latch has occurred |

For active high or active low configuration (CN command), the limit switch bits are '1' when the switch is inactive and '0' when active.

Operand Usage _TSm contains the current status of the switches for the specified axis.

Examples

'Galil DMC Code Example

:V1=_TSB;' Assigns value of TSB to the variable V1

:V1=?;' Interrogate value of variable V1

15 (returned value) Decimal value corresponding to bit pattern 00001111

Y axis not in motion (bit 7 - has a value of 0)

Y axis error limit not exceeded (bit 6 has a value of 0)

Y axis motor is on (bit 5 has a value of 0)

Y axis forward limit is inactive (bit 3 has a value of 1)

Y axis reverse limit is inactive (bit 2 has a value of 1)

Y axis home switch is high (bit 1 has a value of 1)

Y axis latch is not armed (bit 0 has a value of 1)

TS applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TS

##### TT	Torque

TT mm

TT mm

| Usage | TT mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _TTm | Operand has special meaning, see Remarks |

Description

The TT command reports the value of the analog output signal, which is a number between -9.9982 and 9.9982 volts.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to report output torque command |  |

Remarks

Torque output is limited by the value set for the TL command. _TTm contains the value of the torque for the specified axis.

Examples

'Galil DMC Code Example

:v1=_TTA;' Assigns value of TTA to variable, v1

:TTA;'   Report torque on A

-0.2843

TT applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TT

##### TV	Velocity

TV mm

TV mm

| Usage | TV mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _TVm | Operand has special meaning, see Remarks |

Description

The TV command returns the actual velocity of the axes in units of encoder count/s. The value returned includes the sign bit for direction.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mm | A |  | ABCDEFGH | ABCDEFGH | Multi-Axis Mask | Axes to report velocity |  |

Remarks

The TV command is computed using a special averaging filter (over approximately 0.25 sec for TM1000). Therefore, TV will return average velocity, not instantaneous velocity.

_TVm contains the value of the velocity for the specified axis.

Examples

'Galil DMC Code Example :vela=_TVA;' Assigns value of A-axis velocity to the variable VELA :TVA;' Returns the A-axis velocity 3420

TV applies to DMC4000,DMC4200,DMC4103,DMC2103,DMC1806,DMC1802,DMC30010,DMC50000,DMC52000,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TV

ERROR CONTROL

##### TW   Timeout for MC trippoint

TWm= n

TW n,n,n,n,n,n,n,n

TWm= n

TW n,n,n,n,n,n,n,n

| Usage | TWm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | TW n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _TWm | Operand holds the value last set by the command |

Description

The TW command sets the timeout time for the MC trippoint. The TW command sets the timeout to declare an error if the MC command is active and the motor is not at or beyond the actual position within the specified time after the completion of the motion profile. If a timeout occurs, then the MC trippoint will clear and the stopcode will be set to 99. A running program will jump to the special label #MCTIME, if located in the application code.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | -1 |  | 32,767 | 32,766 | 1 | Set the timeout in msec for the MC command | n = -1 disables the timeout |

Remarks

The EN command should be used to return from the #MCTIME subroutine.

Examples

'Galil DMC Code Example

TWA= 1000;' set timeout time for MC to 1000 for A axis var= _TWA;' set value of TW for A axis to variable, var

'Galil DMC Code Example TWA= 5000;' set MC timeout to 5 seconds PRA= 10000;' set move length BGA MCA MG"Move done";' message when move completes EN ' #MCTIME 'code when motor doesn't reach final pos in time MG"Move didn't finish" MG"Longer than ",_TWA," msecs" STA AMA MOA;' shut off axis EN

TW applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TW

INTERROGATION

##### TZ Tell I O Configuration

TZ

TZ

| Usage | TZ | Command takes no arguments |
| --- | --- | --- |

Description

The TZ command is used to request the I/O status of the controller. This is returned to the user as a human-readable text string.

Arguments

TZ is an interrogation command with no parameters

Remarks

The data reported by TZ is also accessible through the TI (inputs) and OP (outputs) command

Examples

'Galil DMC Code Example

:TZ

Block 0 (8-1) dedicated as input - value 255 (1111_1111)

Block 0 (8-1) dedicated as output - value 0 (0000_0000)

Block 1 (16-9) dedicated as input - value 255 (1111_1111)

Block 1 (16-9) dedicated as output - value 0 (0000_0000)

Block 10 (88-81) dedicated as input - value 255 (1111_1111)

Block 11 (96-89) dedicated as input - value 191 (1011_1111) :

TZ applies to DMC4000,DMC4200,DMC4103,DMC2103,RIO47000,DMC50000,DMC52000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

TZ

SYSTEM CONFIG

##### UI   User Interrupt

UI n

UI n

| Usage | UI n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The UI command allows user-defined interrupts to be created. UI can generate 16 different status bytes, $F0 to $FF (240-255), corresponding to UI0 to UI15.

UI pushes a user-defined status byte into the EI queue. When the UI command (e.g. UI5) is executed, the status byte value (e.g. $F5 or 245) is queued up for transmission to the host, along with any other interrupts.

Arguments

| Argument |  | Min |  | Max |  | Default |  | Resolution | Description |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  |  | 15 |  | 0 |  | 1 | Set the status byte for the interrupt |  |  |  |
| Status Byte (dec) |  |  | Condition |  |  |  | Status Byte (dec) |  |  | Condition |
| $F0 (240) |  |  | UI0 was executed |  |  |  | $F8 (248) |  |  | UI8 was executed |
| $F1 (241) |  |  | UI1 was executed |  |  |  | $F9 (249) |  |  | UI9 was executed |
| $F2 (242) |  |  | UI2 was executed |  |  |  | $FA (250) |  |  | UI10 was executed |
| $F3 (243) |  |  | UI3 was executed |  |  |  | $FB (251) |  |  | UI11 was executed |
| $F4 (244) |  |  | UI4 was executed |  |  |  | $FC (252) |  |  | UI12 was executed |
| $F5 (245) |  |  | UI5 was executed |  |  |  | $FD (253) |  |  | UI13 was executed |
| $F6 (246) |  |  | UI6 was executed |  |  |  | $FE (254) |  |  | UI14 was executed |
| $F7 (247) |  |  | UI7 was executed |  |  |  | $FF (255) |  |  | UI15 was executed |

Remarks

The UDP interrupt packet dispatch may be delayed depending on the number of interrupts in the queue

If immediate packet dispatch is required, use the message command (MG) to send a unique message to the host software.

EI,,h must be set to a valid UDP port (set by the host, not the DMC code, is recommended) before any interrupt packet will be dispatched.

Examples

'Galil DMC Code Example

JG 5000;' Jog at 5000 counts/s

BGA;' Begin motion

ASA;' Wait for at speed

UI 1;' Cause an interrupt with status byte $F1 (241)

'The program above interrupts the host PC with status byte $F1 (241)

'when the motor has reach its target speed of 5000 counts/s

UI applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806,DMC1802

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

UI

PROGRAMMING

##### UL Upload

UL

UL

| Usage | UL | Command takes no arguments |
| --- | --- | --- |
| Operands | _UL | Operand has special meaning, see Remarks |

Description

The UL command transfers the program from the controller to a host computer. Programs are sent without line numbers. The uploaded program will be followed by a <control>Z or a '\' as an end of text marker.

Arguments

UL is a command with no parameters

Remarks

In a Galil software, the UL command is not necessary because the UL command is handled by the graphical interface (Upload Program). In a terminal utility such as HyperTerminal or Telnet, the UL command will bring the uploaded program to screen.

From there, the user can copy it and save it to a file.

Issuing this command will pause the output of the Data Record until the command is completed.

###### Operand Usage

When used as an operand, _UL responds with the number of spaces left in memory to assign new variables.

Examples

'Galil DMC Code Example :UL;' Begin upload #A Line 0 NO This is an Example Line 1 NO Program Line 2 EN Line 3 {cntrl}Z Terminator :

UL applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

UL

##### US   USB port configuration

US n

US n

| Usage | US n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _US | Operand holds the value last set by the command |

Description

Configures the operation of the USB port on the DMC-4103 series controller.

Arguments

| Argument |  | Value | Description | Notes |
| --- | --- | --- | --- | --- |
| n | 0 |  | USB is main, interpreted port | Default |
|  | 1 |  | Swap USB and Aux DB9 port. USB is not interpreted. DB9 is the main, interpreted port | Null modem cable required for DB9 |

Remarks

To take effect, US state must be burned with BN and the controller must be power cycled, or a push button reset performed. RS is not sufficient.

See CC, CI, and #COMINT for using the USB port in swap mode, US 1.

A null modem cable is required for DB9 port.

To load firmware, use Ethernet or USB. Do not attempt to load firmware over the DB9 If firmware loading fails, perform an upgrade with the following

- install UG jumper and MR jumper

- power cycle the controller

- connect via USB with baud rate set by jumpers

- load firmware over USB connection

- remove UG jumper and MR jumper

- power cycle

- connect via USB or Ethernet. Assigning the IP address will be required. Issue US as needed.

After issuing the US1 command, the CC command is used to set the speed of the USB while the baud rate jumper is used to set the speed of the DB9. If desired, Galil can provide a more permanent USB and DB9 swap; contact the factory for more information n=? returns the currently set value

Examples

'Galil DMC Code Example

:MG _US

0.0000

:US 1;' swap the ports

:BN

:'power cycle to swap ports

US applies to DMC4103

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

US

##### VA	Acceleration

VAm= n

VA n,n

VAm= n

VA n,n

| Usage | VAm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | VA n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _VAm | Operand holds the value last set by the command |

Description

The VA command sets the acceleration of the vector in a coordinated motion sequence.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | S | T | S | Axis | Coordinate plane to be specified |  |
| n | 1,024 | 1,073,740,800 | 256,000 | 1,024 | Vector acceleration for the coordinate system |  |

Remarks

_VAm contains the value of the vector acceleration for the specified coordinate system

When issuing VA implicitly, the first argument refers to the "S" plane and the second refers to the "T" plane.

Examples

'Galil DMC Code Example

:VA 1024;' Set vector acceleration to 1024 counts/sec2

:VA ?;'   Return vector acceleration

1024

:VA 20000;' Set vector acceleration

:VA ?;'         Return vector acceleration

19456

:accel=_VAS;' Assign variable, accel, the value of VA

VA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VA

VECTOR/LINEAR

##### VD Vector Deceleration

VDm= n

VD n,n

VDm= n

VD n,n

| Usage | VDm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | VD n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _VDm | Operand has special meaning, see Remarks |

Description

The VD command sets the deceleration of the vector in a coordinated motion sequence.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | S | T | S | Axis | Coordinate plane to be specified |  |
| n | 1,024 | 1,073,740,800 | 256,000 | 1,024 | Vector deceleration for the coordinate system |  |

Remarks

_VDm contains the value of the vector deceleration for the specified coordinate system.

When issuing VD implicitly, the first argument refers to the "S" plane and the second refers to the "T" plane.

Examples

'Galil DMC Code Example

#vector;'      Vector Program Label

VMAB;'         Specify plane of motion

VA 1000000;'   Vector Acceleration

VD 5000000;'   Vector Deceleration

VS 2000;'      Vector Speed

VP 1000,2000;' Vector Position

VE;'           End Vector

BG S;'         Begin Sequence

AM S;'         Wait for Vector sequence to complete

EN;'           End Program

VD applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VD

##### VE	Sequence End

VE n

VE n

| Usage | VE n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _VEm | Operand has special meaning, see Remarks |

Description

The VE command indicates to the controller that the end of the vector is coming up. This allows the controller to slow down through multiple segments, if required. VE is required to exit the vector mode gracefully (stop code, SC, 101).

Arguments

| Argument | Value | Description | Notes |
| --- | --- | --- | --- |
| n | 0 | Specify the end of a vector segment | Also occurs when n = 'null' |
|  | ? | Returns the length of the vector in counts |  |

Remarks

The VE command will apply to the selected coordinate system, S or T. To select the coordinate system, use the command CAS or CAT. _VEm contains the length of the vector in counts for the specified coordinate system, S or T

Examples

'Galil DMC Code Example

#vector;'      Vector Program Label

VM AB;'        Specify plane of motion

VA 1000000;'   Vector Acceleration

VD 5000000;'   Vector Deceleration

VS 2000;'      Vector Speed

VP 1000,2000;' Vector Position

VE;'           End Vector

BG S;'         Begin Sequence

AM S;'         Wait for Vector sequence to complete

EN;'           End Program

VE applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VE

SYSTEM CONFIG

##### VF Variable Format

VF n

0

.n

1

VF n

0

.n

1

| Usage | VF n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _VF | Operand has special meaning, see Remarks |

Description

The VF command formats the number of digits to be displayed when interrogating the controller. If a number exceeds the format, the number will be displayed as the maximum possible positive or negative number (i.e. 999.99, -999, $8000 or $7FF).

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | -8 |  | 10 | 10 | 1 | Specify the number of digits displayed before the decimal point | A negative value specifies hexadecimal format, see Remarks |
| n1 | 0 |  | 4 | 4 | 1 | Specify the number of digits displayed after the decimal point |  |

Remarks

A negative n0 specifies hexadecimal format. When in hexadecimal, the string will be preceded by a $ and Hex numbers are displayed as 2's complement with the first bit used to signify the sign. A positive n0 specifies standard decimal format.

A ? is only valid for querying n0. When queried, the value reported will be the value of the format for variables and arrrays specified by n0 and n1 eg. VF 10,4 would respond to VF ? with 10.4

_VF contains the value of the format for variables and arrays

If the number of digits set by n0 is insufficient for representing the integer portion of a variable, the returned value will be the greatest number representable by n0.n1. For example, if var=123, and VF is 2.4, var=? will return 99.9999.

Examples

'Galil DMC Code Example

VF 5.3;' Sets 5 digits of integers and 3 digits after the decimal point

VF 8.0;' Sets 8 digits of integers and no fractions

VF -4.0;' Specify hexadecimal format with 4 bytes to the left of the decimal

'Galil DMC Code Example

:VF8,4;'  set vf to 8 digits of integers and 4 digits of fraction

:VF?;'    query the value of VF

8.4

:MG_VF;'  query again

8.4

VF applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VF

##### VM	Mode

VM m0m1m2

VM m0m1m2

| Usage | VM mm | Argument is an axis mask |
| --- | --- | --- |
| Operands | _VMm | Operand has special meaning, see Remarks |

Description

The VM command enables the coordinated motion mode and specifies the plane of motion. This mode may be specified for motion on any set of two axes, including a combination of real and virtual axes for single-axis operation. The motion is specified by the instructions VP and CR, which specify linear and circular segments.

Up to 511 segments may be given before the Begin Sequence (BGS or BGT) command. The number of available segments is queriable via the _LMm operand.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m0 | A |  | H |  | A | Axis | First axis specified for vector motion |  |
| m1 | A |  | H |  | N/A | Axis | Second axis specified for vector motion |  |
|  | M |  | N |  | N/A | Axis | Virtual axis specified for vector mode | Used when performing vector mode for a single real axis |
| m2 | A |  | H |  | N/A | Axis | Tangent axis specified for vector mode. | m2 = null if tangent mode is not desired. |
|  | M |  | N |  | N/A | Axis | Virtual axis specified for vector mode. | Used to disable the tangent function if already enabled. Otherwise, use m2 = null. |

Remarks

Specifying one axis for vector mode is useful for obtaining sinusoidal motion on 1 axis using the CR command.

The Vector End (VE) command must be given after the last segment. This allows the controller to properly decelerate.

Additional segments may be given during the motion when the buffer frees additional spaces for new segments.

It is the responsibility of the user to keep enough motion segments in the buffer to ensure continuous motion.

The first vector in a coordinated motion sequence defines the origin for that sequence. All other vectors in the sequence are defined by their endpoints with respect to the start of the move sequence.

The VM command will apply to the selected coordinate system, S or T. To select the coordinate system, use the command CAS or CAT. _VMm contains instantaneous commanded vector velocity for the specified coordinate system, S or T.

###### Enabling Vector Mode

- Specify the desired coordinate system to use with the CA command. S is default.

- Specify the vector plane to be used with the VMm0m1 command. If using tangent axis include that as the m2 parameter

  - EG. for a AB vector plane with the D axis used as a tangent axis, issue VM ABD

  - If only the vector plane is desired for the above example, then issue VM AB

- Specify vector speed with VS, vector acceleration with VA, and vector deceleration with VD

- Specify vector segments with the VP command, or circular segments with the CR command

- When finished with the sequence of moves, issue VE

- Issue BGS to begin motion for the S coordinate system

- You can now wait for motion to complete, issue additional segments as buffer space is cleared, or start a new move on the T coordinate plane by specifying CAT and starting from step 2.

Examples

'Galil DMC Code Example

#A;'            Program Label

VM AB;'         Specify motion plane

VP 1000,2000;'  Specify vector position 1000,2000

VP 2000,4000;'  Specify vector position 2000,4000

CR 1000,0,360;' Specify arc

VE;'            Vector end

BG S;'          Begin motion sequence

AM S;'          Wait for vector motion to complete

EN;'            End Program

'Galil DMC Code Example

#A;'            Program Label

VM AN;'         Specify motion plane

VP 1000,2000;'  Specify vector position 1000,2000

VP 2000,4000;'  Specify vector position 2000,4000

CR 1000,0,360;' Specify arc

VE;'            Vector end

BG S;'          Begin motion sequence

AM S;'          Wait for vector motion to complete

EN;'            End Program

###### VM

VM applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VM

##### VP	Position

VP n

0

,n

1

< o > p

VP n

0

,n

1

< o > p

| Usage | VP n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _VPm | Operand has special meaning, see Remarks |

Description

The VP command defines a vector move segment for the VM mode of motion. The VP command defines the target coordinates of a straight line segment in a 2 axis motion sequence. The units are in quadrature counts, and are a function of the elliptical scale factor set using the command ES. For three or more axes in linear interpolation mode, use the LI command.

Arguments

| Argument | Min |  | Max | Default | Resolution | Description |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | - 2,147,483,648 |  | 2,147,483,647 | 0 | 1 | Specify the target position for the first vector axis |  |  | See Remarks |
| n1 | - 2,147,483,648 |  | 2,147,483,647 | 0 | 1 | Specify the target postion for the second vector axis |  |  | See Remarks |
| o | 2 |  | 15,000,000 | N/A | 2 | Specifies the vector speed to be commanded at the beginning of the linear segment. The controller will start accelerating or decelerating at the start of the sequence to this speed. |  |  | For MT 1,-1 |
|  | 2 |  | 3,000,000 | N/A | 2 | Specifies the vector speed to be commanded at the beginning of the linear segment. The controller will start accelerating or decelerating at the start of the sequence to this speed. |  |  | For MT 2,2,2.5, and 2.5. |
| p | 2 |  | 15,000,000 | N/A | 2 | Specifies the vector speed to be achieved at the end of the linear segment. The controller will decelerate or accelerate during the segment and will reach the specified speed at the end of the segment. |  |  | For MT 1,-1 |
|  | 2 |  | 3,000,000 | N/A | 2 | Specifies the vector speed to be achieved at the end of the linear segment. The controller will decelerate or accelerate during the segment and will reach the specified speed at the end of the segment. |  |  | For MT 2,2,2.5, and 2.5. |
| Argument | Value | Description |  |  |  |  | Notes |
| o | -1 | Specifies vector speed to be set by Vector Speed Variable (VV command) |  |  |  |  | See VV command |

Remarks

The first vector in a coordinated motion sequence defines the origin for that sequence. All other vectors in the sequence are defined by their endpoints with respect to the start of the move sequence.

Vector moves are defined as absolute positions from the origin of the sequence. The length of each vector segment must be limited to 8,388,607.

The VM command will apply to the selected coordinate system, S or T. To select the coordinate system, use the command CAS or CAT.

_VPm where m = axis designator A,B,C,D,E,F,G or H and contains the absolute coordinate of the axes at the last intersection along the sequence.

For example, during the first motion segment, this instruction returns the coordinate at the start of the sequence. The use of _VPm as an operand is valid in the linear mode, LM, and in the Vector mode, VM.

Examples

'Galil DMC Code Example

#A;'           Program Label

VM AB;'        Specify motion plane

VP 1000,2000;' Specify vector position 1000,2000

###### VP

VP 2000,4000;' Specify vector position 2000,4000

CR 1000,0,360;'Specify arc

VE;'           Vector end

BGS;'          Begin motion sequence

AMS;'          Wait for vector motion to complete

EN;'           End Program

'Galil DMC Code Example REM VP n,m p REM 'o' and 'p' are in counts/sample rather than counts/second as the VS command. REM This means that when TM <> 1000, commanded speed for VS will be different than REM values for 'o' and 'p' REM To get counts/second for 'o' and 'p', divide them by a ratio of 1000/_TM REM REM #vs and #vsop result in the same profile #vs TM 250 VMAN VS 100000 VA 2560000 VD 2560000 VP 20000,20000 VE BGS AMS EN ' #vsop TM 250 VMAN n=1000/_TM 'VS 100000 VA 2560000 VD 2560000 VP 20000,20000<(100000/n) VE BGS AMS EN

VP applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VP

##### VR	 Ratio

VR n

0

,n

1

VR n

0

,n

1

| Usage | VR n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _VRm | Operand holds the value last set by the command |

Description

The VR sets a ratio to be used as a multiplier of the current vector speed. The vector speed can be set by the command VS or the operators < and > used with CR, VP and LI commands. VR takes effect immediately and will ratio all the previous vector speed commands.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 0 |  | 10 | 1 | 1/65,536 | Vector ratio specified for the S coordinate plane |  |
| n1 | 0 |  | 10 | 1 | 1/65,536 | Vector ratio specified for the T coordinate plane |  |

Remarks

VR doesn't ratio acceleration or deceleration, but the change in speed is accomplished by accelerating or decelerating at the rate specified by VA and VD.

VR is useful for feedrate override, particularly when specifying the speed of individual segments using the operator '<' and '>'.

_VRm contains the vector speed ratio of the specified coordinate system - where m = S or T.

_VRS contains the vector speed ratio of the specified coordinate system

Examples

'Galil DMC Code Example #A;' Vector Program VM AB;' Vector Mode VP 1000,2000;' Vector Position CR 1000,0,360;' Specify Arc VE;' End Sequence VS 2000;' Vector Speed BG S;' Begin Sequence AM S;' After Motion JP#A;' Repeat Move #SPEED;' Speed Override VR(@AN[1]*.1);' Read analog input compute ratio vr=_VRS;' Store vector ratio in variable 'vr' JP#SPEED;' Loop XQ#A,0 XQ#SPEED,1;' Execute task 0 and 1 simultaneously

VR applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VR

## VS

VSm= n

VS n,n

VSm= n

VS n,n

| Usage | VSm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | VS n ... | Arguments specified with an implicit, comma-separated order |

Description

The VS command specifies the speed of the vector in a coordinated motion sequence in either the LM or VM modes. This speed is in place when individual segment speeds for VP, LI and CR are not specified.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | S |  | T | S | Axis | Coordinate plane to be specified |  |
| n | 2 |  | 15,000,000 | 25,000 | 2 | Vector speed applied to the coordinate system |  |

Remarks

Vector speed can be attached to individual vector segments using the operators '<' and '>'. For more information, see description of VP, CR, and LI commands. The VV command allows for variables to be specified during vector segments.

Vector Speed can be calculated by taking the square root of the sum of the squared values of speed for each axis specified for vector or linear interpolated motion.

_VSm contains the vector speed of the specified coordinate system

When issuing VS implicitly, the first argument refers to the "S" plane and the second refers to the "T" plane.

Examples

'Galil DMC Code Example :VS 2000;' Define vector speed of S coordinate system :VS ?;' Return vector speed of S coordinate system 2000 :

VS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VS

### VV	 Variable

VVm= n

VV n,n

VVm= n

VV n,n

| Usage | VVm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | VV n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _VVm | Operand has special meaning, see Remarks |

Description

The VV command sets the speed of the vector variable in a coordinated motion sequence in either the LM or VM modes. The VV command is used to set the "o" vector speed argument for segments that exist in the vector buffer for LI, CR and VP commands. By defining a vector segment begin speed as a negative 1 (i.e. "<-1"), the controller will utilize the current vector variable speed as the segment is profiled from the buffer.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | S |  | T | S | Axis | Coordinate plane to assign value |  |
| n | 0 |  | 15,000,000 | 0 | 2 | Variable vector speed | For MT 1,-1,1.5 and -1.5 |
|  | 0 |  | 3,000,000 | 0 | 2 | Variable vector speed | For MT 2,-2,2.5 and -2.5 |

Remarks

VV command is useful when vector segments exist in the buffer that use the "<" and ">" speed indicators for specific segment and corner speed control and the host needs to be able to dynamically change the nominal return operating speed. _VVm contains the vector speed variable of the specified coordinate system

Examples

'Galil DMC Code Example

:VVS= 20000;'          Define vector speed variable to 20000 for the S coordinate system :VP1000,2000<-1>100;'  Define vector speed variable for specific segment. :VVS=? 20000 :

VV applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

VV

INTERROGATION

### WH   Which Handle

WH

WH

| Usage | WH | Command takes no arguments |
| --- | --- | --- |
| Operands | _WH | Operand has special meaning, see Remarks |

Description

The WH command is used to identify the handle from which the command was received. This is useful for determining what interface or handle you are connected to.

Arguments

WH is an interrogation command with no parameters

Remarks

_WH contains the numeric representation of the handle from which the command was received.

The following table lists the possible string returned by WH, and the numerical value returned by _WH

| Communication Channel | WH | _WH |
| --- | --- | --- |
| Main USB Port | USB | -1 |
| Ethernet Handle A | IHA | 0 |
| Ethernet Handle B | IHB | 1 |
| Ethernet Handle C | IHC | 2 |
| Ethernet Handle D | IHD | 3 |
| Ethernet Handle E | IHE | 4 |
| Ethernet Handle F | IHF | 5 |
| Ethernet Handle G | IHG | 6 |
| Ethernet Handle H | IHH | 7 |

Related Commands

DH - DHCP Client Enable

IA - IP Address

IH - Open IP Handle

TH - Tell Ethernet Handle

Examples

'Galil DMC Code Example

:WH;' Request incoming handle identification

IHC

:MG_WH

2

'Galil DMC Code Example

:WH;'     Request incoming handle identification

USB Command received from USB port

:MG_WH

-1

WH applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

WH

PROGRAMMING, TRIPPOINT

### WT   Wait

WT n

0

,n

1

WT n

0

,n

1

| Usage | WT n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |

Description

The WT command is a trippoint used to time events.When this command is executed, the controller will wait for the amout of time specified before executing the next command.

The amount of time in the WT command is specified to be either samples or milliseconds, depending on the second argument of WT

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n0 | 2 |  | 2,147,483,646 | N/A | 2 | Specify amount of time to hold execution of code |  |
| n1 | 0 |  | 1 | 0 | 1 | Specify the type of WT | n = 0 or null specifies WT in msecs. n = 1 specifies WT in samples |

Remarks

If n1=1 for WTn0,n1 then the controller will wait for the number of samples specified before executing the next command. By default, WT is specified in milliseconds. If n1 is omitted, then n1 = 0 is used and WT is timed in milliseconds

Examples

'Galil DMC Code Example '10 seconds after a move is complete, turn on a relay for 2 seconds #A;' Program A PR 50000;' Position relative move BGA;' Begin the move AMA;' After the move is over WT 10000;' Wait 10 seconds SB 1;' Turn on relay (set output 1) WT 2000;' Wait 2 seconds CB1;' Turn off relay (clear output 1) EN;' End Program

WT applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

WT

PROGRAMMING

### XQ Execute Program

XQ #str,n

1

XQ n

0

,n

1

XQ #str,n

1

XQ n

0

,n

1

| Usage | XQ n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _XQ0 _XQ1 _XQ2 _XQ3 _XQ4 _XQ5 _XQ6 _XQ7 | Operand has special meaning, see Remarks |

Description

The XQ command begins execution of a program residing in the program memory of the controller. Execution will start at the label or line number specified.

Up to 8 programs may be executed simultaneously to perform multitasking.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| str | 1 char | 7 chars | See Notes | String | Label to begin code execution | If omitted, start from line 0 (n0=0) |
| n0 | 0 | 3,999 | 0 | 1 | Line number to begin code execution | Firmware Rev 1.2a and later |
| n0 | 0 | 1,999 | 0 | 1 | Line number to begin code execution |  |
| n1 | 0 | 7 | 0 | 1 | Thread number to execute code |  |

Remarks

_XQn contains the current line number of execution for thread n, and -1 if thread t is not running. If using ED to add code, you must exit ED mode before executing code.

Examples

'Galil DMC Code Example

XQ #apple,0;'  Start execution at label apple, thread zero

XQ #data,2;'   Start execution at label data, thread two

XQ ;'          Start execution at line 0

XQ applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

XQ

### YA Step Drive Resolution

YAm= n

YA n,n,n,n,n,n,n,n

YAm= n

YA n,n,n,n,n,n,n,n

| Usage | YAm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | YA n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _YAm | Operand holds the value last set by the command |

Description

Specifies the microstepping resolution of the step drive in microsteps per full motor step. Consult your drive documentation to determine its microstepping setting. See the table below for internal Galil stepper drives.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | 0 | 9,999 | 2 | 1 | Drive resolution in step counts/motor step for SPM mode | YA has special functionality for certain hardware configurations. See the rest of the notes in this table. |
|  | 1 | 16 | 2 | see Notes | Valid settings for SDM44040 (- D4040) | 1,2,4 and 16 set the step resolution of the SDM-44040 to full, half, 1/4th and 1/16th microstepping respectively. When full stepping (n=1) on the SDM-44040, the max gain will be 70% of value set with AG. Max current is available for any microstepping mode. |
|  | 64 | 64 | 2 | 0 | Valid setting for SDM44140 (- D4140) | The SDM-44140 is always configured for 64th microstepping, YA must be set to 64 for SPM mode |

Remarks

YA Settings for Galil Stepper Drives

| Stepper Drive Hardware | YA Setting | Notes |
| --- | --- | --- |
| AMP-43547 | 256 | Drive fixed at 1/256 step |
| SDM-44040 | 1 | Drive set to single step (70% current max) |
| SDM-44040 | 2 | Drive set to 1/2 step |
| SDM-44040 | 4 | Drive set to 1/4 step |
| SDM-44040 | 16 | Drive set to 1/16 step |
| SDM-44140 | 64 | Drive fixed at 1/64 step |

Examples

'Galil DMC Code Example 'Set the step drive resolution for a 1/64 Microstepping Drive: :YA 64,64,64,64 :'Query the D axis value :MG_YAD;'Response shows D axis step drive resolution 64.0000 ::

'Galil DMC Code Example 'Set the step drive resolution for a 1/256 Microstepping Drive: :YA 256 :'Query the A axis value :MG_YAA;' Response shows A axis step drive resolution 256.0000 ::

YA applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

YA

### YB StepMotor Resolution

YBm= n

YB n,n,n,n,n,n,n,n

YBm= n

YB n,n,n,n,n,n,n,n

| Usage | YBm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | YB n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _YBm | Operand holds the value last set by the command |

Description

The YB command specifies the resolution of the step motor, in full steps per full revolution, for Stepper Position Maintenance (SPM) mode.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 9,999 | 200 | 1 | Motor resolution in full steps/revolution |  |

Remarks

This command is only required if using SPM mode with stepper motors with an attached encoder. A 1.8 degree step motor is 200 steps/revolution.

Examples

'Galil DMC Code Example 'Set the step motor resolution of the A axis for a 1.8 degree step motor: :YBA=200 :'Query the A axis value :YBA=? 200 Response shows A axis step motor resolution

YB applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

YB

### YC Encoder Resolution

YCm= n

YC n,n,n,n,n,n,n,n

YCm= n

YC n,n,n,n,n,n,n,n

| Usage | YCm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | YC n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _YCm | Operand holds the value last set by the command |

Description

The YC command specifies the resolution of the encoder, in counts per revolution, for Stepper Position Maintenance (SPM) mode.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 32,766 | 4,000 | 1 | Encoder resolution in counts/revolution |  |

Remarks

This command is only required if using SPM mode with stepper motors with an attached encoder.

Examples

'Galil DMC Code Example 'Set the encoder resolution of the A axis :YCA=2000 :'Query the A axis value :YCA=? 2000 :'Response shows A axis encoder resolution

YC applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

YC

### YR ErrorCorrection

YRm= n

YR n,n,n,n,n,n,n,n

YRm= n

YR n,n,n,n,n,n,n,n

| Usage | YRm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | YR n ... | Arguments specified with an implicit, comma-separated order |

Description

The YR command allows the user to correct for position error in Stepper Position Maintenance mode. This correction acts like an IP command, moving the axis or axes the specified quantity of step counts. YR will typically be used in conjunction with QS.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -2,147,483,648 | 2,147,483,647 | 0 | 1 | Number of step pulses to increment position by |  |

Remarks

Users will typically use the value of QS to increment motor by the number of step pulses of error.

EG. YRm = _QSm increments the specified axis by the error magnitude.

The sign of YR depends on the polarity of the position encoder

If the encoder increments when the stepper moves forward (increasing TD), the correction is YRm=_QSm. This is typical.

If the encoder decrements when the stepper moves forward, the correction is YRm= -_QSm. See CE to invert the polarity of the position encoder, if desired.

Examples

'Galil DMC Code Example 'Query the error of the B axis: :QSB 253 :'This shows 253 step counts of error :'Correct for the error: :YRB=_QSB;' The motor moves _QS step counts to correct for the error :'and YS is set back to 1

'Galil DMC Code Example 'Query the error of the A axis: :QSA 253 :' This shows 253 step counts of error :'Correct for the error: :YRA=_QSA;' The motor moves _QS step counts to correct for the error 'and YS is set back to 1

YR applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

YR

### YS Stepper Position Maintenance Mode Enable, Status

YSm= n

YS n,n,n,n,n,n,n,n

YSm= n

YS n,n,n,n,n,n,n,n

| Usage | YSm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | YS n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _YSm | Operand has special meaning, see Remarks |

Description

The YS command enables and disables the Stepper Position Maintenance Mode function. YS also reacts to excessive position error condition as defined by the QS command.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H |  | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 1 |  | 0 | 1 | Setting of the SPM mode | n = 0 disables SPM mode, n = 1 Enables SPM mode. See Remarks |

Remarks

Both YSm = ? and _YSm contain the value of n. n is 1 when SPM mode is enabled and no error has occurred. If a position error has occured, n becomes 2.

If n = 2, this indicates a position error condition defined as more than 3 full motor steps of position error. Issuing an n = 1 will clear the error

Position Error Limit

| Microstep Setting (YA) | Error (QS) Limit |
| --- | --- |
| 1 | 3 |
| 2 | 6 |
| 16 | 48 |
| 64 | 192 |
| 256 | 768 |

Examples

'Galil DMC Code Example 'Enable the mode: :YSH=1 :'Query the value: :YS*=? 0,0,0,0,0,0,0,1 Response shows H axis is enabled

'Galil DMC Code Example 'Enable the mode: :YSA=1 :'Query the value: :YSA=? 1 Response shows A axis is enabled

YS applies to DMC50000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,DMC1806,DMC1802,EDD37010,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

YS

PROGRAMMING

### ZA UserData Record Variables

ZAm= n

ZA n,n,n,n,n,n,n,n

ZAm= n

ZA n,n,n,n,n,n,n,n

| Usage | ZAm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | ZA n ... | Arguments specified with an implicit, comma-separated order |
| Operands | _ZAm | Operand holds the value last set by the command |

Description

ZA sets the user variables in the data record. The user variables (one per axis) are automatically sent as part of the status record from the controller to the host computer. These variables provide a method for specific controller information to be passed to the host automatically.

Arguments

| Argument | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| m | A | H | N/A | Axis | Axis to assign value |  |
| n | -2,147,483,648 | 2,147,483,647 | 0 | 1 | Value of user variable for data record |  |

Remarks

n is an integer and can be a number, controller operand, variable, mathematical function, or string. Only 4 bytes are available for n. Fractional values are not stored or sent via the data record

Examples

'Galil DMC Code Example

#thread

ZAA= myVar;'   constantly update ZA with variable myVar

WT 10

JP#thread;'    run in an infinite loop

ZA applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC1806

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ZA

SYSTEM CONFIG

### ZN Negative Antifriction Bias

ZNm= n

ZN n,n,n,n,n,n,n,n

ZNm= n

ZN n,n,n,n,n,n,n,n

| Usage | ZNm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | ZN n ... | Arguments specified with an implicit, comma-separated order |

Description

ZN adds a negative open loop voltage to the controller?s command signal when the position error is negative.

Arguments

| Argument | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A | H |  | N/A | Axis | Axis to assign value |  |
| n | -9.9998 | 0 |  | 0 | 0.0003 | Open loop voltage (Volts) |  |

Remarks

Valid only for -NAN and -CER firmware

Examples

'Galil DMC Code Example

ZNA=-1;'set negative antifriction bias on A axis to -1 volt

ZPC=1;'set positive antifriction bias on C axis to 1 volt

ZN applies to CER,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ZN

SYSTEM CONFIG

### ZP Positive Antifriction Bias

ZPm= n

ZP n,n,n,n,n,n,n,n

ZPm= n

ZP n,n,n,n,n,n,n,n

| Usage | ZPm= n | Arguments specified with a single axis mask and an assignment (=) |
| --- | --- | --- |
|  | ZP n ... | Arguments specified with an implicit, comma-separated order |

Description

ZP adds a positive open loop voltage to the controller?s command signal when the position error is positive.

Arguments

| Argument |  | Min | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| m | A |  | H | N/A | Axis | Axis to assign value |  |
| n | 0 |  | 9.9998 | 0 | 0.0003 | Open loop voltage (Volts) |  |

Remarks

Valid only for -NAN and -CER firmware

Examples

'Galil DMC Code Example

ZNA=-1;'set negative antifriction bias on A axis to -1 volt

ZPC=1;'set positive antifriction bias on C axis to 1 volt

ZP applies to CER,NANO

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

ZP

PROGRAMMING

### ZS Zero Subroutine Stack

ZS n

ZS n

| Usage | ZS n ... | Arguments specified with an implicit, comma-separated order |
| --- | --- | --- |
| Operands | _ZS0 _ZS1 _ZS2 _ZS3 _ZS4 _ZS5 _ZS6 _ZS7 | Operand has special meaning, see Remarks |

Description

The ZS command is used to clear the stack when finishing or leaving a subroutine. This command is used to avoid returning from an interrupt (either input or error). This turns the jump to subroutine into a jump. The status of the stack can be interrogated with the operand _ZS, see Remarks.

Arguments

| Argument |  | Min |  | Max | Default | Resolution | Description | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n | 0 |  | 1 |  | 0 | 1 | Sets zero stack operation | n = 0 clears the entire stack. n = 1 clears one level of the stack. |

Remarks

Do not use RI (Return from Interrupt) when using ZS.

To re-enable interrupts, you must use II command again.

#### Operand Usage

_ZSn contains the stack level for the specified thread where n = 0 to 7.

The response, an integer between zero and sixteen, indicates zero for beginning condition and sixteen for the deepest value.

Examples

'Galil DMC Code Example

#A;'           Main Program

II1;'          Input Interrupt on 1

#B;JP #B;EN ;' Loop

#ININT;'       Input Interrupt

MG"INTERRUPT";'Print message

S=_ZS0;'        Interrogate stack

S=?;'          Print stack

ZS;'           Zero stack

S=_ZS0;'        Interrogate stack

S=?;'          Print stack

EN;'           End

ZS applies to DMC50000,DMC52000,DMC4000,DMC4200,DMC4103,DMC30010,DMC2103,RIO47000,DMC1806,DMC1802,DMC2105

©2022 Galil Motion Control. Revision: 1840 . Corrections, Feedback: support@galil.com

#### ZS